# 2     Installation

This chapter contains information about installing the RAIDWatch software (including its sub-module Event Monitor) for local and remote management.  The chapter includes the following topics:

- System requirements

- Platform-specific requirements

- RAID network charting

- Software setup

- In-band SCSI

## 2.1     System Requirements

The minimum hardware and software requirements for RAIDWatch are listed below.

Server Running RAIDWatch

- Pentium or above compatible (or equivalent PC) running Windows NT 4/Windows 2000; Solaris 7 & 8(SPARC, x86); AIX 4.3; or Red Hat Linux 6.1 (kernel v2.2.xx); Red Hat 7, SUSE 7

- At least one available RS-232 port (if remote  event notification over the telephone line is desired)

- Hayes-compatible modem (if pager/telephone event notification is desired) or fax/modem (if fax event notification is desired).  [Note: fax command class 2.0 support only.]

- SNMP service for Windows NT (if SNMP remote management is desired)

- Windows Messaging (MAPI) for Windows NT (if support for pager or fax notification is needed)

- Windows NetBEUI support for Windows NT (if network broadcast support notification is needed)

- Local client running RAIDWatch Manager

- Pentium or above compatible (or equivalent PC) running Windows NT 4/Windows 2000; Solaris 7 & 8 (SPARC, x86); AIX 4.3; or Red Hat Linux 6.1 (kernel v2.2.xx); Red Hat 7, SUSE 7, WIN95/98, Windows Me

- SNMP service for Windows NT (if SNMP agent is under the Windows NT environment)

- Windows Messaging (MAPI) for Windows NT/95/98/2000 (if support for pager or fax notification is needed)

- Windows NetBEUI support for Windows NT/95/98/2000 (if network broadcast support notification is needed)

## 2.2     RAID Chart

Before installing RAIDWatch and its various agents and modules, it is helpful for users to chart their RAID systems.  Users who operate a single RAID from a local or remote workstation may skip this section.  For users with multiple RAID systems, the following information provides guidelines for charting existing RAID systems.

Table 2-1  :  RAID Systems Chart

| ID/Name | Where? | OS? | IP Address? | Role | Internet Capable? |
| --- | --- | --- | --- | --- | --- |
| Example | HQ | Win NT | 205.163. 164.111 | Main RAID server | Yes |

- ID/Name – User designated; an ID or name should be a unique identifying label.

- Where – a specific geographic reference (e.g., Headquarters, building 3, equipment room 100).

- OS – the operating system running on the particular system.

- IP Address – if available

- Role – the purpose, relative to RAID operations, fulfilled by the particular system.

- Internet Capable – if a server is an internet server, the answer to this is, “Yes.”  If  a workstation will manage RAID systems through a browser, note the particular browser software and its version number.

## 2.3     Platform Requirements

RAIDWatch supports various operating systems both for servers (RAID management hosts or web servers) and for client management stations (RAIDWatch Manager workstations).  Support for Java, however, varies from OS to OS.  This section explains what steps need to be taken depending upon which OS will be used.

LIMITATION: 
The Java installation program, INSTALL.JAR, ONLY supports Netscape 4.5 (or above), Microsoft Internet Explorer 4.0 (or above) under Windows NT Server 4.0 (Windows 95/98/2000) and Netscape 4.5 (or above) under Solaris (x86, SPARC).

The RAIDWatch Manager program, GRM.JAR, ONLY supports Netscape 4.5 (or above) under Windows NT Server 4.0 (Windows 95/98/Me/2000), or Netscape 4.5 (or above) under Solaris (x86, SPARC).

### 2.3.1 Solaris Platforms

RAIDWatch supports both Solaris 7 and 8 x86 and SPARC servers and workstations.  Prior to running the installation procedure for RAIDWatch on a Solaris machine, complete the following steps:

(1) Mount installation CD

(2) Add agents: Please add necessary agents to the system shell script.  These agents will be loaded when the system initiates.  The following is an example of how to add agents to the system shell script:

- Append the following strings to /etc/profile:

/usr/hybrid/bin/secondary > /dev/null 2>&1

/usr/hybrid/bin/primary > /dev/null 2>&1

(3) If the Solaris machine is a web server, (Apache server software default subdirectories are used to illustrate), copy the “common” file folder from your RAIDWatch installation CD to the Apache html directory and type the following:

#cp –r /cdrom/common /usr/local/apache/htdocs/

(4) Modifying browsers for remote installation:

No configuration change is needed for systems using Internet Explorer.  If the browser used on a particular machine is Netscape Navigator or Hot Java and RAIDWatch will be installed remotely via the internet, you will have to modify your browser configuration as follows:

- A. For Netscape on a Solaris system, append the string:

user_pref(“signed.applets.codebase_principal_support”, true);

to

~/. netscape/preferences.js

- B. For HotJava on a Solaris system, append the string:

hotjava.default.security=low

to

~/.hotjava/properties

- Note:

HotJava will create a .hotjava directory under the user's home directory when it is run for the first time.   Netscape will create a .netscape directory under the user's home directory when it is run for the first time.

(5) Required Java Patch Files:

In order to use Java scripts under the Java Runtime Environment (JRE) on your Solaris machine, you will need to download certain Java patch files.

To download JRE patches for Solaris 7, and make the files accessible, complete the following steps: 

1. Use a web browser to open
http://www.sun.com/software/solaris/jre/download.html

2. Click on the HTTP hotpoint that corresponds with your Solaris version.  For example, if your platform is a SPARC workstation, and the Solaris is an English edition, click “Solaris SPARC Platform
Edition: English” to open the next HTML page.

3. The web server will then ask you to sign in. Please sign in.

4. When you see the license agreement, please click OK.

5. Click the appropriate message to download your  patches.  For example, if your system is a SPARC platform, please select the file below to download:

Download Patches for Solaris 7 REQUIRED for 1.2.2_05a, Solaris/Intel   8.93 MB)

The file name is 1[1].2.2_05a_patches_i386_5.7.tar

If your system is an x86 platform, please select the file below to download:

Download Patches for Solaris 7 REQUIRED for 1.2.2_05a, Solaris/SPARC   1.04 MB)

The file name is 1[1].2.2_05a_patches_sparc_5.7.tar

6. Put the patch files in the right directory.  The install shell script (intall.sh) needs for you to put the patch files
in your system.  For example in a Solaris SPARC workstation, if the patch file name is 2.2_05a_patches_sparc_5.7.tar
Enter the following:

#mkdir /usr/patches
#cp 2.2_05a_patches_sparc_5.7.tar /usr/patches
#cd /usr/patches
#tar xvf 2.2_05a_patches_sparc_5.7.tar

You have now tarred the patch files in the /usr/patches directory.

- Note:

### Patches displayed here are only examples.  Patches are continuously updated on Sun's web site.

- RAIDWatch also supports Red Hat Linux 6.1 servers and workstations.

- (1) Mount installation CD

(2) Add agents: Please add necessary agents to the system shell script.  These agents will be loaded when the system initiates.  The following is an example of how to add agents to the system shell script:

- Append the following strings to /etc/profile:

/usr/hybrid/bin/secondary > /dev/null 2>&1

- /usr/hybrid/bin/primary > /dev/null 2>&1

(3) If your Red Hat Linux 6.1 host is a web server:

Make sure that web server software is already installed on your Linux server.  The default web directory will be /usr/home/httpd.  Allow the RAIDWatch installation package to be accessed from /usr/home/httpd/html.

A. Mount the RAIDWatch installation CD to /cdrom:, type the following and press Enter to proceed (assuming that the CD-ROM drive is /dev/hdc):

- #mount /dev/hdc /cdrom

B.   Copy the “common” file folder from the installation package CD to the Apache html directory, type the following and press Enter to proceed:

#cp –r /cdrom/common /home/httpd/html/

(4) Modifying browsers for remote installation:

No configuration change is needed for systems using Internet Explorer.  If the browser used on a particular machine is Netscape Navigator or Hot Java and RAIDWatch will be installed remotely via the internet, you will have to modify your browser configuration as follows:

A. For Netscape on a Linux system, append the string:

user_pref(“signed.applets.codebase_principal_support”, true);

to

- ~/. netscape/preferences.js

B. For HotJava on a Linux system, append the string:

hotjava.default.security=low

to

~/.hotjava/properties

- Note:

HotJava will create a .hotjava directory under the user's home directory when it is run for the first time.   Netscape will create a .netscape directory under the user's home directory when it is run for the first time.

### 2.3.3 Windows Platforms

- RAIDWatch supports Windows NT 4.0 for servers and Windows 95/98/Me/NT/2000 for workstations.

(1) In order to use Netscape in Windows, append the string:

user_pref(“signed.applets.codebase_principal_support”, true);

to

c:\winnt\profiles\<username>

(for Windows NT or Windows 2000)

-or-

c:\Program_Files\Netscape\Users\<username>

(for Windows 95/98/Me)

- (2) SNMP Service

- SNMP service for Windows NT (if the SNMP agent is under a Windows NT environment) must be enabled.

- Locate “Services” under the Windows Control Panel.  Enable or install SNMP services.  Refer to your Windows documentation for more information.

- (3) MAPI for Windows

- Windows Messaging (MAPI) for Windows NT/95/98/Me/2000 (if support for pager, fax, or e-mail notification is needed) must be enabled.  Refer to your Windows documentation for more information.

- (4) NetBEUI support

- Windows NetBEUI support for Windows NT/95/98/Me/2000 (if network broadcast support notification is needed) must be enabled.  Refer to your Windows documentation for more information.

## 2.4     Software Setup

This section discusses how to install RAIDWatch in your system.  Before proceeding with the setup procedure, read through the Before You Start section below.

### 2.4.1	Before You Start

Before starting the installation, read through the notes listed below.

- TCP/IP must be installed and running with a valid IP address assigned to each primary and secondary agent station, even if RAIDWatch Manager is being used on the local host.

- Your system display must be running in 256 color mode or some configuration items will not be visible.

- Your RAID controller must either be defined as a peripheral device or logical drives mapped to host LUNs, otherwise RAIDWatch will be unable to locate the controller.

- Be certain that your system meets the minimum hardware and software requirements listed in the System Requirements section.

- Check to be certain that the RAID disk arrays and controllers are installed properly.  For the installation procedure, see the documentation that came with the controller.

- Follow the directions provided in the Platform Requirements section to prepare for installation and operation under different OS environments.

### 2.4.2	Installing RAIDWatch

Follow these steps to install RAIDWatch on your server(s) and RAID systems (the installation procedure for RAIDWatch Manager and Event Monitor is explained in section 2.4.3):

1. Insert the Infortrend RAIDWatch installation CD into your CD-ROM drive.

2. If installing on a Unix system, mount the RAIDWatch CD to /mnt.

3. If you are currently running other applications, close them before proceeding with the setup process.  This will minimize the possibility of encountering system errors during setup.

If you are installing remotely, skip step 4.

4. For local installs, run the install script file related to the OS you are using to start the installation process.  (Each OS has its own subdirectory.)  This method will open a command line window and ask first if you want to install Java Runtime Environment (JRE).  Enter “N” for no unless you are also installing RAIDWatch Manager (see 2.4.3 Installing RAIDWatch Manager).  The installation script will then ask if you want to install RAIDWatch.  Type “Y” for a first install, reinstall, or reconfigure of the program.

- If you have just completed step 4, skip step 5.

5. For remote installation, open …\common\install.htm through a web browser. (For example, use your web browser to open “/mnt/common/install.htm” from a Unix system or the directory  “d:\common\install.htm” under Windows NT.)  The installation procedure will auto-select the software modules that match your OS.

(The browser installation start explained in step 5 can also be used locally if you prefer.)

6. For both local and browser starts, accessing the install file should open the first stage of the installation process.  There are three options here for installing or configuring your package.  Please select Install Packages then Configure, and click  the OK button to continue.

7. Click the Next button on the Welcome screen to continue.

8. Read the contents of the License and click the I Agree button to continue.

9. If you are installing to a primary host, please select Primary Agent, NPC (Notification Processing Center), Event Monitor, and, if the primary host server is also connected to a RAID, please select Secondary Agent.  Then click on the Next button to continue.   Please note that although Event Monitor can run without starting RAIDWatch Manager, yet it can not be installed without  RAIDWatch Manager.

10. Click on the Install button under the SELECT PATHS Window to start installing the selected components to your system.  The installer program will automatically create a new directory for these components.  The default is “/usr/hybrid/bin.”  For Windows NT, the directory will be “c:\Program Files\hybrid\bin.”  RAIDWatch Manager and Event Monitor must be installed to the same directory.

11. Primary Agent Settings Assign the “TCP Port Number” that RAIDWatch Manager stations will use to communicate with the Primary Agent.  The “Max Client Number” and “Max Controller Number” are also configurable.  Users are recommended to keep the defaults for these two parameters.

- Redundant Modules:

It is recommended to install Primary and NPC Agents redundantly on different servers to avoid the blind time when the Primary Agent or NPC module is down.  Once the Primary Agent or NPC is down, the Primary Agent installed on another server will take over instantly allowing NPC to work.

Primary Agent and NPC should be installed manually on different servers.  The configuration utility does not automatically add these modules to the selected RAID server.

- Redundant enable: select to enable primary agent/NPC on another server for the precaution that a Primary Agent/NPC might fail and, as the consequence, fatal system events might then occur unnoticed.

- Mode: assign the preferred mode for the Primary Agent installed on current server as active or standby.  There exists an active-standby relationship between Primary Agents and the relationship can be automatically resolved between agents.

- Peer Host: enter the IP address.

- Peer's TCP Port Number: enter the same TCP port number as that on the current Primary Agent host.

- Negotiation Interval is the duration of time (in seconds) for Primary Agents to negotiate for the active-standby status.  A Primary Agent might wait for seconds for its peer agent to start up.

Important! at least one Primary Agent and one Secondary Agent must be installed in a network managed by RAIDWatch.  RAID controllers are managed by Secondary Agents which are in-turn managed by the Primary Agent.  Without Secondary Agents associated with each RAID, it will be impossible to manage RAID systems.

The Event Monitor also requires Primary and Secondary Agents to be running on RAID servers to record system events and report system status.

Even if the system only has one RAID and all RAIDWatch components are installed on a single server, both a Primary Agent and a Secondary Agent must be installed.

12. Managed Secondary Agent Settings (subsection of Primary Agent Settings): under this part of the Primary Agent configuration, list each Secondary Agent RAID server that will be managed through the Primary Agent where you are installing RAIDWatch.

- Host names are IP addresses for each RAID server.

- TCP Port settings should be the same for all Secondary Agents but should be a different Port setting from the Primary Agent setting and otherwise unique to the Secondary Agents.

Important: the Primary Agent default TCP port setting should be changed to a network-unique assignment.   The default TCP port setting is 58632.  Any TCP port number between 49152 and 65535 can be used.

Also, Managed Secondary Agents should all use a different, common-to-all Secondary Agents, TCP port setting from that of the Primary Agent.

13. Secondary Agent Settings: each Secondary Agent RAID server needs to be configured under this window.

Important: local Secondary Agents must be installed at all RAID hosts managed by RAIDWatch.

- Host Name/IP Address is the IP address for the current RAID server.

- TCP Port Number is the TCP port to be used by all secondary agents.

- Event Polling Period is the time interval (in seconds) for event queries from the Secondary Agent to the RAID controller.

- Port Name is the COM port which has a modem connected.

- Baud Rate is the maximum data transfer rate of the attached modem.

14. NPC Settings: Notifier has four possible means of informing RAID managers that an event has occurred.  They are: Fax, Pager, Broadcast, Email, and SNMP Traps.  In order to use fax or pager notification, Modem parameters must also be set.  For NT servers, Widows Messaging and Personal Fax must be installed and running for NPC to work.  If NPC will not be used, skip these steps.

- Modem Settings include Serial Port, which is a COM port setting and Baud Rate which is the maximum transfer speed of the modem.  A modem must be installed and setup in order to use NPC fax or pager notification.

- Important Note:

A pre-configured modem can only be removed from the list of Available Modems after all references to a modem have been removed.

- Fax Settings:

- Profile Name is a collection name required for logon to a Windows Messaging profile (FAX, E-mail, Exchange Mail).  It is usually the same with the account name.  A profile name validates the available messaging services and service providers during a particular MAPI session.

- Telephone is the phone number for an event message receiving fax machine.  (Note that any access dialing requirements, such as accessing an outside line, must be included in the phone number.  For example: 9,,,2241603 wherein “9” is access for an outside line and “,,,” indicates a pause.)

- Message is the text that will be sent as a fax. the message that will be printed on the fax message.  You may add, for instance, the contact information of technical personnel, etc.

- Available Modems provides a list of installed and configured modems that may be used for sending event notices via fax.

- Initializing String – standard AT command set.  The default is “&F1X0M0” (for Unix-based systems) and should be left as is for most fax modems.

- Pager Settings:

- Telephone is the phone number for an event message receiving pager.  (Note that any access dialing requirements, such as accessing an outside line or entering commands upon connecting with a pager service, must be included in the phone number.  For example: 9,,,2241603,,,1 wherein “9” is access for an outside line, “,,,” indicates a pause, and “1” is to leave a message to be forwarded to the pager.)

- Message is the text that will be sent to the pager.  (Note: any pager specific limitations regarding length or content must be considered.)

- Available Modems provides a list of installed and configured modems that may be used for sending event notices via this pager number.

- Initializing String – standard AT command set.  The default is “V1B1E0F0L1M1Q0TV1X1Y0&C1&D2&G0S7=20” and should be left as is for most pagers.

- Broadcast Settings include a Host Name (IP address) and the Message to be broadcast.  The host name should be the server for the LAN where the notification messages will be broadcast.

- Email Settings:

- Profile Name is a collection name required for logon to a Windows Messaging profile (FAX, E-mail, Exchange Mail).  It is usually the same with the account name.  A profile name validates the available messaging services and service providers during a particular MAPI session.

- SMTP Server is the mail server used to send event notifications via e-mail.

- Sender’s Email is the “From” part of e-mail notification messages.  It must be a valid internet e-mail address.

- Receiver’s Email allows users to enter multiple event notification e-mail recipients with messages for each.  Each entry must be a valid internet e-mail address.

- Subject allows users to add a subject to event notification emails.

- Message is the message sent to the particular e-mail address being added.   You may list, for example, the contact information of technical personnel.

- SNMP Settings:

- Severity is the parameter used to determine what levels of events to be sent via SNMP (1, all levels; 2, Warning and Alert; 3, only Alert).

- Host IP is the port number of the agents  listening for traps.  Click Add to avail agents to the Trap Receiver List.

- Trap Receiver List is a list of listening SNMP agents.

- Important Note:

Agent and NPC configuration parameters can be reconfigured later using the installation program.  Run the installation program and select Configure Only from the Start menu to reconfigure Agent and NPC settings.

If the Primary Agent, Secondary Agent, and NPC (if needed) have all been installed and configured, including Secondary Agents for each RAID host, then installation is complete.

After all elements have been installed in a particular machine, click the ExitSetup button to exit.

You may now install RAIDWatch Manager or Event Monitor at stations from which your RAIDs will be monitored and managed.

### Installing RAIDWatch Manager &

### Event Monitor

- If a web server is not available to run RAIDWatch, or for some other reason browser access to the program is not preferred, RAIDWatch Manager or Event Monitor can be installed on workstations on the LAN where the RAID server with the Primary Agent installed resides.

Follow these steps to install RAIDWatch Manager and Event Monitor on a workstation:

1. Insert the Infortrend RAIDWatch installation CD into your CD-ROM drive.

2. If installing on a Unix system, mount the RAIDWatch CD to /mnt.

3. If you are currently running other applications, close them before proceeding with the setup process.  This will minimize the possibility of encountering system errors during setup.

- If you are installing remotely, skip step 4.

4. For local installs, run the install script file related to the OS you are using to start the installation process.  (Each OS has its own subdirectory.)  This method will open a command line window and ask first if you want to install Java Runtime Environment (JRE) and second if you want to install RAIDWatch (including Event Monitor).  In both cases, type “Y” for a first install, reinstall, or reconfigure of the program.

- If you have just completed step 4, skip step 5.

5. For remote installation, open …\common\install.htm through a web browser. (For example, use your web browser to open “/mnt/common/install.htm” from a Unix system or the directory  “d:\common\install.htm” under Windows NT.)  The installation procedure will auto-select the software modules that match your OS.

(The browser method for starting the installation process explained in step 5 can also be used locally if you prefer.)

6. For both local and browser starts, accessing the install file should open the first stage of the installation process.  There are three options here for installing or configuring your package.  Please select Install Packages Only, and click the OK button to continue.

- Important Note:

Agent and NPC configuration parameters can be reconfigured later using the installation program.  Run the installation program and select Configure Only from the Start menu to reconfigure Agent and NPC settings.

7. Click the Next button on the Welcome screen to continue.

8. Read the contents of the License and click the I Agree button to continue.

9. Please select RAIDWatch and Event Monitor.  Click on the Next button to continue.  RAIDWatch Manager can be installed and run without Event Monitor. However, Event Monitor can not be installed without RAIDWatch Manager.

10. Click on the Install button under the SELECT PATHS Window to start installing the selected components to your system.

The installer program will automatically create a new directory for RAIDWatch components.  The default is “\usr\hybrid\bin.”  For Windows NT, the default directory will be “c:\Program Files\hybrid\bin.”

11. Before the installation procedure finishes, it will ask if you want to add shortcuts to your computer.  Choosing to add them will put a shortcut icons on your desktop for starting RAIDWatch Manager and Event Monitor.  (For Windows users only.)  Click the ExitSetup button on the final screen to finish.

The installation procedure is now complete.  You may start managing your RAID disk array systems.  For information on first time operating instructions, see Chapter 3, Basic Operations.

### 2.4.4	List of Filenames

Below is a list of key RAIDWatch files installed during the installation process.  All files should be found in the /usr/hybrid/bin/  (default) directory or the directory you chose during the installation.

RAIDWatch Manager access:

1.	GRM.JAR    executable .jar Java file

- GRM.HTM    browser accessible HTML file

Event Monitor access:

- GREM.JAR   executable .jar Java file

- GREM.HTM  browser accessible HTML file

Executable Agent files:

1.	PRIMARY.EXE    to start the Primary Agent

2.	SECONDARY.EXE    to start the Secondary Agent

3.	NPC.EXE    to start the Notification Processing Center

Configuration and Install files:

1.	INSTALL.JAR    executable .jar Java file

2.	INSTALL.HTM    browser accessible HTML file

3.	UNINSTALL.HTM    browser accessible HTML file

- Others (including OEM definitions):

| IFTBundle_0 IFTBundle_1 … | Multi-language support |
| --- | --- |
| em_oemname oemname oemlogo default.enc | OEM definitions .. .. OEM enclosure view definitions |
| /help /em_help | Java-help files |

### 2.4.5	Program Updates

As Infortrend’s valued customer, you are entitled to free program updates.  You can download the latest version of RAIDWatch from Infortrend’s FTP sites at ftp.infortrend.com in the U.S.A., or ftp.infortrend.com.tw in Taiwan.  For more information about this service, call Infortrend or an Infortrend distributor in your area.

## 2.5     In-band SCSI

### What is it and what is it used for?

- These days more and more external devices require communication with the host computer for device monitoring and administration. This is usually done through RS-232C ports.

An alternative means of communication is now available for SCSI-to-SCSI RAID controllers – in-band SCSI. The traditional way for SCSI controllers to communicate with the host computer has been via software  (such as RAIDWatch) using an RS-232C connection. With in-band SCSI, integrators have more flexibility.  They may use RS-232C or the existing SCSI cable instead.

In-band SCSI is particularly useful when creating a new RAID.  In order for a host to “see” the controller, and thus for Primary and Secondary agents, and RAIDWatch Manager to manage the controller, it must first be configured as a peripheral device.  In-band SCSI allows you to do this using Text RAID Manager.

### How is it able to use the SCSI cable?

- In-band SCSI technology translates the original commands into standard SCSI commands.  These SCSI commands are then sent to and received from the SCSI RAID controller.  RAIDWatch can manage the RAID controller just as it could before via RS-232C.  (Note: it is assumed that users of in-band SCSI possess the following – a third-party SCSI adapter and a channel on their Infortrend RAID controller that can be designated as a host channel.  Both of these are required for in-band SCSI communication between the host and the RAID controller.)

### Configuring a RAID Controller to Use In-band SCSI

#### RAID Controller Adjustments

- Some adjustments must be made to the RAID controller and to the host computer's SNMP settings before the two can communicate using SCSI commands.  The RAID controller settings can be changed using the Front Panel.  (Your front panel may be different in appearance from the one shown in this example.)

From the Main Menu, press  or  to select “View and Edit Configuration Parameters.”

View and Edit

Config Parms

Press <Enter>; and then use the  or  to select “Host-side SCSI Parameters.” Then press <Enter>.

Host-side SCSI

Parameters  ..

You will need to make adjustments in the following four submenus: Peripheral Device Type, Peripheral Device Qualifier, Device Support for Removable Media, and LUN Application. Different host operating systems require different adjustments. Look at the table below to find the proper settings for your host operating system.

Table 2-2 : Peripheral Device Type Parameters Reference

for Various Operating Systems:

| Operating System | Peripheral Device Type | Peripheral Device Qualifier | Device Support for Removable Media | LUN Applic-ability |
| --- | --- | --- | --- | --- |
| NT 4.0 | 1f | connected | disabled | All Undefined LUNs |
| NT 5.0 | 3 | connected | enabled | All Undefined LUNs |
| NetWare 4.x | 1f | connected | disabled | All Undefined LUNs |
| SCO Unix 5.0x | 7f | connected | either is okay | All Undefined LUNs |
| UnixWare 2.1x | 3 | connected | either is okay | All Undefined LUNs |
| Solaris 2.5.x/2.6 | 7f | connected | either is okay | All Undefined LUNs |

Table 2-3 : Peripheral Device Type Settings:

| Device Type | Setting |
| --- | --- |
| No Device Present | 7f |
| Direct-access Device | 0 |
| Sequential-access Device | 1 |
| CD-ROM Device | 5 |
| Scanner Device | 6 |
| MO Device | 7 |
| Unknown Device | 1f |
| Processor Type | 3 |

###### Example: Settings for Windows NT 4.0

- The settings for Windows NT 4.0 are provided here as an example. For the settings under other operating systems, please refer to table 2-2 above, Peripheral Device Type Parameters Reference for Various Operating Systems.

On the front panel, use  or  to select “Peripheral Device Type Parameters”; and then press <Enter>.

Periph      Dev

Type Parameters

(For this example, we assume that there are currently no peripheral devices.)

Device Type -

No Device (0x7f)

Press  or  to choose “Unknown Device - 1f”.

Set Device Type?

Unknown (0x1f)

Press <Enter> to confirm the selection. Now that we have changed the Peripheral Device Type, let us set the Peripheral Device Qualifier. Press <Esc> to return to the sub-menus mentioned above. Use the arrow keys to scroll down to Device Qualifier., press  or  to select “Device Qualifier Connected.”

The default setting is “Connected.” If your Front Panel reads “Disconnected,” press <ENT> and you will be prompted to change to “Connected.” If your Device Qualifier setting reads “Connected,” press <Esc> to return to the host-side SCSI submenus.

Device Qualifier

Connected

Use the  or  to select Support for Removable Media. The default setting is “Disabled.” If the LCD reads “Enabled,” press <Enter> and you will be prompted to accept a change. If the screen reads “Disabled,” press <Esc> to return to the host-side SCSI submenus.

SupportRemovable Media - Disabled

Press  or  to select “LUN Application”; and then press <Enter>. The default setting is “All Undefine LUN.”

LUN Application-

All Undefine LUN

Press <Enter> and use  or  to select “Undefine LUN-0’s.”

Applies to     ?

Undefine LUN-0’s

Press <Enter> to accept. The screen should display the following message.

LUN Application-

Undefine LUN-0’s

The RAID controller adjustments necessary to use in-band SCSI have been completed.

## 2.6     Out-of-band via Ethernet

Newer line of Infortrend controllers running firmware version 3.21 are equipped with an Ethernet port for connection with a management station.  RAIDWatch programs can now run on a RAID controller,  allowing users to manage the system from any management station in any place via TCP/IP protocol.

Listed below are steps necessary for preparing RAIDWatch programs on a RAID controller.

- Follow the instructions in documentation that came with your controller for details on how to connect  Ethernet port.  Requirements for a management station are the same as those for the clients running RAIDWatch manager.  Support for Java should be enabled on a management station accessing the manager program through web browser.

- Create a logical drive by the use of LCD front panel or firmware-embedded manager.  Make sure there is no pre-configured storage sector on the selected member drives.

- Assign an IP address to the controller and provide Net Mask and gateway values.  Reset the controller for the configuration to take effect.

- The controller will take a while formatting a small storage sector for the manager programs.  When the controller is finished with formatting drives, ftp RAIDWatch programs to the designated controller  IP address.  Program files transferred to a controller for out-of-band management should include: grm.htm, grm.jar, and IFT-Bundle_0.  Files named as IFT-Bundle are definitions of multi-language support.  For an English interface, ftp IFT-Bundle_0 to the controller.

- Primary and Secondary Agents are not necessary here.

- Other modules, NPC and Event Monitor, should be installed on host computer.  The requirements and installation procedures are the same as described in the previous sections.

- Start your web browser and enter the IP address  assigned to the controller followed by grm.htm as your URL (e.g.,  http://222.212.121.123/grm.htm).

- A RAID system thus connected can also be accessed by a Primary Agent host in a greater topological scheme involving multiple RAID systems.  The RAID system will then be considered as one of the Secondary Agent hosts and managed from where another RAIDWatch manager resides through the Primary Agent.  The RAID system can also be defined as one of the monitoring targets where multiple systems are managed by an NPC or Event Monitor.