#### Interpreting the FSMs

The high level FSMs contained in 9.2 through 9.6 and the low level FSMs contained in annexes L, M, N, Q and R use the following set of rules.

These FSMs illustrate only the Port and Station Operation Table Join, Transmit and Monitor functions even though all Port and Station Operation Tables defined are required for correct operation.

The Join, Transmit, and Monitor FSMs can operate simultaneously.  Each FSM can have only one State active at a time.

Figure 9-3 illustrates states and transitions as follows.

- STATES as vertical lines with the State name at the top of the line.

- TRANSITIONS as horizontal lines, always contain an arrow head to show a connection to a STATE and may contain a number to indicate state/transition number (e.g., J12).

  - When state/transition numbers are present within a FSM, the first character represent the FSM 
(J for Join, T for Transmit and M for Monitor), the first digit is the State being exited and the second digit is the State being entered.

  - Occasionally multiple transitions with different actions occur between two States (e.g., J13A and J13B) and the state/transition number ends with a character.

- Entry into a STATE occurs only when the transition ends at a STATE with an arrow head.

- Some transitions are awkward to illustrate because of location.  TRANSITION J31 exits STATE J3 and enters STATE J1 (e.g., J31 labeled (A)).

- Some transitions cross a STATE without an arrow head which indicates the STATE is not entered.  TRANSITION J31 exits STATE J3 and enters STATE J1 (e.g., J31 labeled (B)).

- OPTIONAL conditions are enclosed in brackets (e.g., [condition 6]).

Figure 9-3 - State/transitions (S/T) illustration

Figure 9-3 illustrates the following state transitions.

- STATE J1 to STATE J2 (transition J12) due to event of Condition_1, action of Action_1.

- STATE J1 to STATE J3 (transition J13A) due to event of Condition_2 OR Condition_3, action of Action_2.

- STATE J1 to STATE J3 (transition J13B) due to event of Condition_4 AND Condition_5 OR optional Condition_6, action of Action_3.

- STATE J3 to State J1 - two different ways of illustrating the same transition.

  - STATE J3 to STATE J1 (transition J31) due to event of Condition_9, action of Action_4.

  - STATE J3 to STATE J1 (transition J31) due to event of Condition_9, action of Action_4.

The Clause 9 Port and Station Operation Tables EVENT/CONDITION column contains the current state, but this state is not shown in the Low Level FSMs in annexes L, M, N, Q or R since the current state is illustrated by the vertical line.

Some of the Clause 9 Port and Station Operation Tables for the Join, Transmit and Monitor functions contain transitions without a state/transition entry (S/T) and, although these transitions are not shown in the FSMs in annexes L, M, N, Q or R, they are required for proper operations.