# ZyXEL ES-2024A V3.60(TX.2)C0

# Release Note/Manual Supplement

Date:  May 24, 2006

This document describes the features in the ES-2024A product for its 3.60(TX.2)C0 release.

## Support Platforms:

ZyXEL ES-2024A V3.60(TX.2)C0 supports models: ZyXEL ES-2024A

## Version:

OS Version: V3.60(TX.2) | 04/26/2006

BootBase Version: V1.07 | 04/20/2005

## Default Bootbase Setting:

| ZyNOS Version | V3.60(TX.2) ¦ 04/26/2006 17:35:00 |
| --- | --- |
| Bootbase Version | V1.07 ¦ 04/20/2005 13:38:02 |
| Vendor Name | ZyXEL |
| Product Model | ES-2024A |
| ZyNOS Code Model | ES2024A |
| HTP Code Model | Unknown |
| ZyNOS ROM address | bfc10000 |
| System Type | 8 |
| MAC Address | 001349000001 |
| Default Country Code | FF |
| Boot Module Debug Flag | 01 |
| RomFile Version | E2 |
| RomFile Checksum | 2ecd |
| ZyNOS Checksum | a986 |
| SNMP MIB level & OID | 060102030405060708091011121314151617181920 |
| Main Feature Bits | C0 |
| Other Feature Bits |  |
| DD 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00-00 00 13 00 00 00 |  |

## Features:

- MAC address learning

- Support IEEE 802.1D transparent bridge

- Support IEEE 802.1Q tagged VLAN

- Support Port-based VLAN

- Support IEEE 802.1X

- Support IEEE 802.1W

- Support IEEE 802.3AD

- Support GVRP

- IGMP snooping

- Support IEEE 802.1p

- Automatic age out

- Specific MAC addresses forwarding per port

- MAC Filtering

- Port Trunking

- Port Mirroring

- Bandwidth Control

- Broadcast Storm Control

- Support Strict Priority and WRR queuing method

- Static IP management or dynamic IP(DHCP client)

- Multiple IP address management

- Firmware upgrade and configuration backup/restore.

- Remote manageable.

- Cluster Management

- WEB manageable

- Support RFC-1213 MIB II

- Support RFC-1493 Bridge MIB

- Support RFC-1643 Ethernet MIB

- Support RFC-1757 Four group of RMON

- Support RFC-2674 VLAN MIB

- NEW CLI supported

- System configuration backup / restore in text-mode

- Multiple login supported

- Radius management authentication

- SSH/SSL support

- Support EMS

## Known Issue:

- For the same ports, LACP and RSTP cannot be activated at the same time

- The counter of packets whose length is between 1519 and 1522 (without VLAN tag but including FCS) will be counted both in the range of 1024 to 1518 and Giant.

- The device will be too busy to responding if it is still learning VLAN from GVRP and configuring the VLAN table especially for 4094 VLANs. After the process of configuring VLAN table is done, the device will still work well.

## Bug Fix:

## Limitation of Settings:

- VLAN 1Q static entry 256

- Static MAC forwarding entry 256

- MAC filtering entry 256

- Cluster member 24

- Management IP address 64

## Change History:

V3.60(TX.2)C0 (04/26/2006)

Support new hardware chipset

V3.60(TX.1)C0 (06/28/2005)

Support EMS and fix bugs

V3.60(TX.0)C0 (06/28/2005)

First Public Version

## Firmware Upgrade:

The ES-2024A uses FTP to upgrade firmware in run-time through its built-in FTP server.  You can use any FTP client (for example, ftp.exe in Windows) to upgrade ES-2024A. The upgrade procedure is as follows:

Upgrade ES-2024A FW:

C:\> ftp <ES-2024A IP address>

User : <Enter>

Password: 1234

230 Logged in

ftp> put 360TX2C0.bin ras

ftp> bye

Where

- User name : just press <Enter>

- Password : the management password, 1234 by default

- 360TX2C0.bin : the name of firmware file you want to upgrade

- ras : the internal firmware name in ES-2024A

## Configuration Upgrade:

The ES-2024A uses FTP to upgrade firmware in run-time through its built-in FTP server.  You can use any FTP client (for example, ftp.exe in Windows) to upgrade ES-2024A. The upgrade procedure is as follows:

Upgrade ES-2024A configuration:

C:\> ftp <ES-2024A IP address>

User : <Enter>

Password: 1234

230 Logged in

ftp> put 360TX2C0.rom rom-0

ftp> bye

Where

- User name : just press <Enter>

- Password : the management password, 1234 by default

- 360TX2C0.rom : the name of configuration file you want to upgrade

- rom-0 : the internal configuration name in ES-2024A