ZyXEL

Firmware Release Note

ZyWALL 5

Release 4.01(XD.3)

| Date: | December 04, 2006 |
| --- | --- |
| Author: | Jeffrey Zhou |
| Project Leader: | Joe Zhao |

ZyXEL ZyWALL 5 Standard Version

release 4.01(XD.3)

Release Note

# Date: December 04, 2006

Supported Platforms:

## ZyXEL ZyWALL 5

Versions:

ZyNOS Version: V4.01(XD.3) | 12/04/2006

Bootbase Version: V1.08 | 01/28/2005

Vantage Agent Version: 1.0.0

Notes:

- Restore to Factory Defaults Setting Requirement: No.

- The setting of ignore triangle route is on in default ROM FILE. Triangle route network topology has potential security crisis. If you are not clear about it, please refer to Appendix for the triangle route issue.

- When firewall turns from "off" to "on", the firewall initialization procedure will disconnect all connections running through the ZyWALL.

- SUA/NAT address loopback feature was enabled on ZyWALL by default, however, if users do not need it, a C/I command "ip nat loopback off" could turn it off.

- In WLAN configuration, a switch for enable / disable WLAN is added. The default value is “disable” since WLAN without any security setting is vulnerable. Please configure MAC filter, WEP and 802.1X when you enable WLAN feature.

- When UPnP is on, and then reboot the device, Windows XP will not detect UPnP and refresh “My Network PlacesLocal Network”. Plug in network wire again can solve this problem.

- For ZW5/35, the default port roles are LAN. For ZW70, the default port roles are DMZ.

- In bridge mode, If LAN side DHCP clients want to get DHCP address from WAN side DHCP server, you need to turn on the firewall rule for BOOT_CLIENT service type in WANLAN direction.

- Under Bridge Mode, all LAN ports will behave as a hub, and all DMZ ports will also behave as another hub.

- For users using the default ROMFILE in former release, please remove “ip nat session 1300” from autoexec.net by CI command “sys edit autoexec.net”. (Upgrade from 3.62)

- In previous 3.64 firmware, the VID value of DPD is not correct. VID change will cause current version doesn’t work with the wrong value. Please be sure to connect with devices which has updated VID, or the DPD may not work correctly.

- In SMT menu 24.1, "WCRD" only represents the WLAN card status when you insert WLAN card into the ZyWALL. If you insert TRUBO card, you will see " WCRD" is always down.

- If you do not want a mail to be scanned by Anti-Spam feature, you can add the mail into whitelist in eWC->Anti-Spam->Lists

- If you want traffic redirect feature to work, you should turn on WAN ping check by "sys rn pingcheck 1". (For ZW5 only)

- The first (first two) entry for static route is reserved for creating ZW5 (ZW35/70) WAN default route and is READ-ONLY.

- If you had activated content filtering service but the registration service state is "Inactive"after upgrading to 4.00, please click "Service License Refresh" in "eWC->REGISTRATION->Registration" or wait until device synchronize with the myzyxel.com.

- WAN1 and WAN2 must be different subnet.

- In Firewall/IDP/AV/AS/BM security rule, Dial backup traffic belongs to WAN interface. (In ZW35/70, the Dial backup traffic belongs to the higher priority WAN interface, for example, if WAN1 priority is higher than WAN2, Dial backup traffic will manage by WAN1 in Firewall/IDP/AV/AS/BM security rule.)

Known Issues:

System Limitation

[Bandwidth Management]

- Bandwidth Management doesn’t work on wireless LAN.

[Content Filter]

- Can’t block ActiveX in some case. (Sometime the ActiveX block fails. This is because the ActiveX is cached in C:\WINNT\Downloaded Program Files\ If you want to test the ActiveX block functionality. Please clear the cache in windows.)

[Wireless]

- The fragmentation threshold size support between 800~2432.

[MISC]

- At SMT24.1, the collisions for WAN, LAN and DMZ port are not really counted.

- Symptom: LAN host can ping Internet while LAN host change cable from LAN port to DMZ port.
Condition:

(1) Host connects to LAN port and gets DHCP address from router.
(2) Unplug LAN host cable and plug it into DMZ port.
(3) The host can still ping Internet using LAN DHCP address.
(4) The scenario will continue about 30secs.

- When device is writing flash, all the interrupt/service will be stopped. (Firmware upload and signature update for full version will take tens of seconds)

- Because of the memory shortage (ZW5/P1), device have to restart when customer need to upgrade firmware sometimes.

Issues

[ALG]

- H323 does not support the server in LAN topology.

- Currently, we do not support NAT loopback on SIP registration or proxy server, which means if your SIP client is located on LAN, your registration server address cannot use ZyWALL WAN IP to do loopback to SIP server which located on LAN.

(1) NAT loopback for SIP server on LAN.

(2) NAT one to one address mapping for SIP server on LAN.

(Indeed, this should be described as the NATed WAN IP of SIP client and SIP server should be the same)

(3) Use default server of NAT port forwarding table for SIP server on LAN.

(4) Client A(WAN) call Client B(LAN) using LAN’s IP, and the reversed way. That is you should call phone number directly to each other without the IP address.

There issue will be improved for future plan.

[Anti-Spam]

- Mail cannot pass through 2 devices with Anti-Spam enabled.

- Customer need to turn off the redundant check for AS and AV for gathering more CPU resource or CPU will always reach 100%. When CPU reaches 100%, the AS/CF query will be timeout sometimes because there is no resource for it.

- The maximum length of the mail subject is 2037 right now. The mail subject you

input is 2037, but you need to add the length of (A)"subject: " and (B)"\r\n", (A)+(B)

is 11 bytes. They equal to 2048.

[Bandwidth Management]

- Bandwidth management H.323 service does not support Netmeeting H.323 application.

- If H323/SIP ALG doesn’t work, the Bandwidth management cannot manage the traffic too.

[Bridge Mode]

- When device boots in Bridge Mode, some CI command error messages will be displayed on console. This is because some predefined CI commands in autoexec.net is forbidden to execute in Bridge Mode.

- In the following topology, Firewall VPN to LAN ping can’t be permitted.

PC1--------------DUT1--------NAT Router------PQA lab-----DUT2------PC2

IP: 192.168.1.33 IP: 192.168.1.2   LAN: 192.168.1.1		WAN: 172.25.21.24   IP: 192.168.2.33

GW: 192.168.1.2			 WAN: 172.25.21.200        LAN: 192.168.2.1    GW: 192.168.2.1

DUT1 is on bridge mode, DUT2 is on router mode, build VPN tunnel between them.

On DUT1 enable Firewall, and set Drop for VPN to LAN, then add a firewall rule of VPN to LAN:

Source address = 192.168.2.33

Destination Address = 192.168.1.33

Selected Service = Any (ICMP)

Action for matched Packets = Permit.

Can’t ping 192.168.1.33 from 192.168.2.33 and you can find “Unsupported/out-of-order ICMP: ICMP (Echo Reply)” log on log page.

Note:

(1) Here, PC1’s GW is DUT1’s LAN IP. With the ICMP reply packet, the destination IP is 192.168.2.33. In PC1, the packet will match the default GW (192.168.1.2) and change the destination MAC as DUT's LAN MAC. DUT receive the packet and the destination MAC is DUT's LAN, DUT thinks this packet is send to itself and the ICMP out of order happens. This is because there is no ICMP request packet for the device itself but an ICMP reply packet for DUT.

(ICMP out of order scenario, not ICMP request but with ICMP reply)

(2) If set the default GW in PC1 as 192.168.1.1, the packet's destination MAC is NAT-Device's LAN (192.168.1.1), not DUT's IP. DUT knows the packet is not for itself and ready pass through it. But the packet match the VPN rule and it will encrypted by DUT.

[Content Filter]

- CF Denied Access Message can run script.

- Due to add OutpostPro firewall support, some block logs can’t be shown on log page when you use customization block function such as “Forbidden Web Site List” and “Keyword Blocking”.

- And the categories function can also has some issue because of the OutpostPro firewall bug fix. When user want to block some categories, such as“Search Engines/Portals”, external DB search work normally the first time. But after refreshing the page or open the website again in another Browser window, only “Please contact your network administrator!!” can be showed, without the link to bluecoat.

[Firewall]

- Some limitations on Firewall CLI configuration, (1) User can not delete specific address or custom port entry from a rule. (2) CLI doesn’t support Modify and Move for rules implemented in eWC. (3) eWC can not display firewall rule field correctly if rule is added by CI command and its type is port/address range.

[UPnP]

- Sometimes on screen the “Local Area Connection” icon for UPnP disappears. The icon shows again when restarting PC.

- When you use MSN messenger, sometimes you fail to open special applications, such as whiteboard, file transfer and video etc. You have to wait more than 3 minutes and retry these applications.

[VPN]

- SNMP tools get ZYWALL VPN MIB data, the index of received data are wrong if rules are larger than 1.

- VPN rule swap does not support NAT Traversal.

[MISC]

- The DMZ TxPkts counter increment at about 1 pkt/min even without any Ethernet cables ever connected.

- In eWC->Statistics, Tx data for Dial Backup is not correct.

- ZyWALL does not support WAN 1/WAN 2 on the same sub-net. (For Multiple WAN products)

Features:

Modifications in V4.01(XD.3) | 12/04/2006

Modify for formal release.

Modifications in V 4.01(XD.3)b1 | 11/24/2006

1. [ENHANCEMENT] SPR ID: 061109533

Enlarge mail header size from 1024 to 2048.

2. [BUG FIX] SPR ID: 060711576

Symptom: Content filter is fail when user installs Outpost Firewall.

Condition:

(1) Install OutpostPro Firewall software.

(2) Set "disable all web traffic except for trusted web sites" and enable content filter.

(3) Enable Outpost Firewall, user can surf the website as usual.

(4) If we disable Outpost Firewall, web surfing will be blocked besides trusted web sites.

3. [BUG FIX] SPR ID: 060810690

Symptom: Redirect URL have not limit special character, it will caused DUT crash.

Condition:

(1) In eWC>CF Denied Access Message or Redirect URL, input %s%s%s%s and apply, DUT will be crash.

4. [BUG FIX] SPR ID: 060927777

Symptom: The "Up Time" shown on the Port Statistics and Home page is quite

different when the ZyWALL uptime is more than 100 hours.

Condition:

(1) Let ZyWALL WAN1 uptime be more than 300 hours.

(2) Go to eWC>HOME page, the "Up Time" is "4:00:00".

(3) Click "Port Statistics" button, the WAN1 "Up time" of pop-up window is

"300.00.00".

5. [BUG FIX] SPR ID: 060420608

Symptom: Two SIP clients cannot talk to each other when both of them are in LAN.

Condition:

Topology:

SIP Client_A -------(LAN) ZyWALL (WAN)----------SIP Server

SIP Clinet_B ------|

(1) Two SIP clients register on SIP server which is in the WAN.

(2) Create a call between client A and client B, they cannot hear each other.

6. [BUG FIX] SPR ID: 060419442, 060512720, 060601086

Symptom: The VoIP client cannot hear the voice when SIP server is set behind the LAN.

Condition:

Topology:

P2002A------------+-(LAN)ZW70(WAN)---------P2002B

SIP Server--------|

(1) Create a port forwarding rule on ZW70 to let SIP traffic of P2002B can be

forwarded to SIP server.

(2) Dial a phone call from P2002A to P2002B, P2002B can hear the voice of P2002A, but P2002A cannot hear the P2002B.

7. [ENHANCEMENT]

Symptom: SIP alg enhancement. Additional SIP ALG codes to supports SIP server on LAN. or WAN

Condition: SIP function has some issues to work correctly.

Topology:

(1) Client_A------(L)ZW_1(W)-----SIP Server-----(W)ZW_2(L)-----Client_B

(2) Client_A----SIP Server----(L)ZW_1(W)-------(W)ZW_2(L)-----Client_B

(3) SIP Server----(L)ZW_1(W)----Client_A----(W)ZW_2(L)-----Client_B

(4) SIP Server----(L)ZW_1(W)-------(W)ZW_2(L)+-----Client_A

+------Client_B

8. [ENHANCEMENT] SPR ID: 061102140

Add PPTP CHAP v2 support.

9. [BUG FIX]

AV side effect fix.

10. [BUG FIX] SPR ID: 060928848, 060928863

Symptom: Mail gets stuck when using VPN + PPPoE

Condition:

Topology:

DeviceA(PPPoE) --- DeviceB --- PC(192.168.2.33)

|

Mail Server(192.168.70.103)

(1) Device A behaves as head-quater, device B behaves as branch-office.

(2) DeviceB makes VPN tunnel to DeviceA, all traffic from PC goes through VPN tunnel to Device A then go out from DeviceA's WAN(PPPoE).

(3) DeviceA enables AS for WAN->VPN direction.

(4) PC receives mail from mail server, mail gets stuck.

11. [ENHANCEMENT] SPR ID: 060331694

Add quick timeout mechanism for UDP sessions. This mechanism can for you to search more games in internet by some game platform. If no this mechanism the number of the game you can search is about NAT session number limited.

12. [BUG FIX] SPR ID: 061101036

Symptom: ZyWALL does not get new rating server list after all rating server has been removed.

Condition:

Topology: PC---------(L)ZW5(W)----------Server

(1) PC sends mails by Mail-Group, and receives mail by outlook.

(2) Configure Mail-Group to max connection 64, and add attachments to 1M~2M.

(3) Keep sending and receiving mail until there are all rating servers in the server list has been removed.

(4) ZyWALL will never update a new rating server list after the all rating server has been removed. The ZyWALL will always query fail.

13. [BUG FIX] SPR ID: 061024840

Symptom: SMTP authentication fails on elias.hp-interex.ch (MX V5.4 AnGc).

Condition:

(1) Go to eWC>LOGS>Log Settings.

(2) Set Mail Server of E-mail Log Settings to elias.hp-interex.ch, enable

SMTP Authentication and set related SMTP settings.

(3) The device sends mail will fail on SMTP authentication.

14. [BUG FIX] SPR ID: 060822272

Symptom: ZyWALL will not mail its LOG if the IP specified on the One-To-One Public IP.

Condition:

Topology:

Mail Server-----------(DMZ)ZyWALL(WAN)

192.168.2.33  	192.168.2.1	 10.0.0.1

10.0.0.2

(1) Restore to default romfile.

(2) Set NAT type to full feature.

(3) Build a one-to-one rule for mail server in DMZ.

Local IP         Global IP

192.168.2.33 <-> 10.0.0.2

(4) In the LOG setting, set mail server IP to 10.0.0.2.

(5) Then, disable the firewall and press the "Email Log Now" button to send mail.

(6) You will see the log "SMTP fail (Cannot connect to SMTP server 10.0.0.2)".

15. [BUG FIX]

Symptom: ZyWALL cannot trigger dial backup.

Condition:

Topology:

PC--(LAN)ZyWALL(dial backup)--Internet

(1) Restore default romfile.

(2) Set up dial backup.

(3) PC sets ZyWALL to be DNS proxy server.

(4) PC starts to ping a domain name, but ZyWALL do not trigger dial backup.

16. [BUG FIX] SPR ID: 061005220

Symptom: Device crashes because of mbuf double free in Anti-Spam.

Condition:

(1) System crashes sometimes on customer site.

17. [ENHANCEMENT]

Vulnerability bug: It depends on an error in verifying the PKCS-1 padding

of the signed hash and we update the patch file from safeNet.

18. [ENHANCEMENT]

Vulnerability bug: It depends on an error in verifying the PKCS-1 padding

of the signed hash and we update the patch file from safeNet.

Modifications in V4.01(XD.2) | 10/25/2006

Modify for formal release.

Modifications in V 4.01(XD.2)b1 | 10/18/2006

- [ENHANCEMENT] SPR ID : 060815905,050414612

We change the ZyWALL break mechanism for the infected file. The ZyWALL just breaks the first infected file packet and stop track the file session in the previous mechanism. The old one has better performance, but there is a risk that it couldn't break the file with more than one virus. Now ZyWALL breaks the first infected file packet and the following file packet as well. It is safer but downs performance for handling infected files. We also fix the line-assembly bug for FTP and HTTP in this enhancement.

- [ENHANCEMENT] SPR ID: 060809590, 060809591, 060809592.

The Anti-Spam will modify the server response string ""250[ -]PIPELINING" to

"250[ -]PIPE******". Because ZyWALL does not the SMTP PIPELINING function.

- [ENHANCEMENT] SPR ID: 060830643

Add an option to enable or disable the "Dynamic ACL" log in ZyWALL.

The check box is in:

(1) "eWC->LOGS->Log Settings->Dynamic ACL".

(2) SMT 24.8.

  - "sys logs load".

- "sys logs switch dynacllog".

- "sys logs save".

- "sys logs switch display".

Note: "2006-08-09 00:42:30 Firewall matches a dynamic ACL rule of an ALG session: TCP 192.168.111.2:50999 66.59.243.66:26397 ACCESS PERMITTED"

Engineer Note: The value in default ROM file is "on" in 4.01.

- [ENHANCEMENT]

Wording changed. Out of memory when F/W upload.

(1) FTP

Was:  file size too large.

Is: file size too large. Please reboot device, and try again.

(2) HTTP/HTTPS

Was:  disk full!

Is: disk full! Please reboot device, and try again.

- [ENHANCEMENT] SPR ID: 060522258

If users let "Redirect URL" in Content Filter be blank, the blocking page will be displayed on the forbidden object only.

- [ENHANCEMENT] SPR ID: 060925662

In eWC>MAINTENANCE>Time and Date, add "Madrid" capital in "GMT+1" time zone.

- [FEATURE CHANGE] SPR ID: 060705182, 060705183

WAS: Set "My IP" as WAN2 IP Address in VPN IKE rule, the IKE and IPSec traffic still go through WAN1 because WAN1 has higher metric than WAN2.

IS:  The IKE and IPSec packets will be sent out according to "My IP" field in VPN IKE rule.

Engineer note: The bug fix only applies to multiple WAN products.

- [BUG FIX] SPR ID: 060809598

Symptom: PC can not access the web server (www.fapa.com.pl) via our ZyWALL.

Condition:

PC---(LAN)ZyWALL(WAN)---internet

(1) Get a ZyWALLwith default romfile.

(2) Let PC try to access www.fapa.com.pl.

(3) PC can not access the web server.

(4) It is OK without ZyWALL.

Special case packet flow:

Client(PC)                                 Server(www.fapa.com.pl)

SYN ->

<- ACK = 0

<- SYN, ACK = 1

ACK = 1  ->

HTTP Get ->

- [BUG FIX] SPR ID: 060711547

Symptom: "Don’t block Java/ActiveX/Cookies/Web proxy to trust Web site" function in content filter cannot work.

Condition:

(1) In eWC->SECURITY->CONTENT FILTER->General page, enable "Content filter" and

block "Java Applet/ActiveX/Cookies/Web Proxy".

(2) In eWC->SECURITY->CONTENT FILTER->Customization page, enable "Web site customization" and

"Don't block Java/ActiveX/Cookies/Web proxy to trusted Web sites".

Add "web.haccpsoft.it" to "Trusted Web Sites".

(3) A PC in ZYWALL's LAN side browses "http://web.haccpsoft.it:8080" website.

(4) Login in and click the date, the popup window should show a calendar instead of another login page.

(5) It is blocked by content filter.

- [BUG FIX] SPR ID: 060607461

Symptom: After run 5 hours BT, no traffic can be forwarded by ZyWALL.

Condition:

(1) Restore to default romfile.

(2) In NAT port forwarding page, add a rule with port range from 20000 to 40000.

(3) After running about 5 hours BT, no traffic can pass through ZyWALL.

- [BUG FIX] SPR ID: 060925635, 060727788.

Symptom: System crashes.

Condition:

(1) Enable Firewall, Content Filter, IDP, Anti-Virus and Anti-Spam

Functions, and turn on all related logs.

(2) System crashes sometimes.

- [BUG FIX] SPR ID: 060831744

Symptom: PC cannot ping WLAN interface IP.

Condition:

Topology:

PC1(10.0.0.1)----(10.0.0.2)(WAN)ZyWALL(WLAN)(192.168.7.1)

(1) Restore default ROM file.

(2) Disable firewall feature.

(3) In SMT 24.8, type "ip nat routing 2 1".

(4) Set WLAN interface IP as "192.168.7.1".

(5) Set NAT to "Full Feature" mode.

(6) PC1 generates a PING packet to "192.168.7.1".

(7) There is no response from "192.168.7.1" and the centralized log will show

"Packet without a NAT table entry blocked: ICMP(Echo)"

- [BUG FIX] SPR ID: 060703050

Symptom: Local PC cannot find Remote Host by NetBIOS via VPN tunnel.

Condition:

PC1----(WLAN)DUT-----(VPN)-----ZYWALL(LAN)----PC2

(1) The configured romfile please refer to SPR.

(2) PC1 cannot see PC2 by NetBIOS via VPN tunnel.

Note: This problem only happens when policy index is not equal to IKE index.

Engineer Note: This problem happens in 4.00 and 4.01.

- [BUG FIX] SPR:060925632

The firmware of 4.01’s self-assigned-certificate can’t be used in Mozilla-firefox

- [BUG FIX] SPR ID: 060908449

Symptom: The ZyWALL assigns a used IP to a DHCP client.

Condition:

Topology ZyWALL(LAN)------PC1,PC2

(1) Let the PC1 get a DHCP IP(192.168.1.33).

(2) Pull out the cable of PC1 and set PC2 to use a static IP 192.168.1.33.

(3) Plug back the PC1, PC1 starts to get a DHCP IP, but it still gets 192.168.1.33,

although the ZyWALL has asked if someone is using this IP and gets a response.

IDP module.

Modifications in V4.01(XD.1) | 09/04/2006

Modify for formal release.

Modifications in V4.01(XD.1)b1 | 08/29/2006

- [ENHANCEMENT]

Support 60 categories in content filtering.

New categories: ""Hacking", Phishing", "Spyware/Malware Sources", "Spyware Effects/Privacy Concerns", "Open Image/Media Search", "Social Networking", "Online Storage", "Remote Access Tools", "Peer-to-Peer", "Streaming Media/MP3s" and "Proxy Avoidance".

- [ENHANCEMENT]

Add second time schedule setting in content filtering

- [ENHANCEMENT]

Enhance the CI command "ip ifconfig".

(1) Add a new argument "mss" to configure the MSS value.

(2) After finishing the configuration, the interface information will be displayed.

Usage: ip ifconfig [iface] [ipaddr</mask bits>] <broadcast [addr]> <mtu [value]> <mss [value]> <dynamic> <showoff>

Ex: ip ifconfig enif1 192.168.70.222/24 broadcast 192.168.70.250 mtu 1500 mss 1460

- [ENHANCEMENT]

Add CI command "av zipUnsupport". Processing ZIP file will destroy encrypted

file if flag is on, otherwise pass it.

- [ENHANCEMENT]

Add a CI command to turn on or off the LDAP packet parsing in NAT module.

Usage: "ip nat service ldap [on|off]"

- [BUG FIX]

Symptom: zywall 5 WAN fixed 100/full negotiation fail against cisco 3550/2900.

Condition:

(1) Configure cisco 3550/2900 port to fixed 100/full.

(2) Configure zywall 5 WAN to fixed 100/full.

(3) Zywall 5 WAN can not sync up; remain down.

- [BUG FIX]

Symptom: The DHCP table shows incorrect information.

Condition:

(1) Set the ZyWALL's DHCP IP Pool Starting Address is 192.168.102.146.

(2) Add a DHCP static IP 192.168.102.22 for a PC on the LAN.

(3) Add another PC on the LAN but this PC doesn't have a corresponding DHCP static

IP rule, and then it gets 192.168.102.146 from the ZyWALL.

(4)Go to eWC>Home>DHCP Table, the ZyWALL doesn't show 192.168.102.146, but show 192.168.103.157.

- [BUG FIX]

Symptom: The packet will be dropped if the device does not have the ARP entry of the receiver of this packet.

Condition:

(1) Clear ARP table by "CI>ip arp flush".

(2) Send a ping to 168.95.1.1, but the PC will not get a response in the first ICMP Echo Request.

(3) After the first ping, the rest of pings can get responses.

- [BUG FIX]

Symptom: ZyWALL serial cannot connect one CDMA terminal RWT FCT CDMA.24.

Condition:

Russia raised this issue that our ZyWALL cannot connect one kind of CDMA terminal RWT FCT CDMA.24, but it is okay when this Terminal connect to P662 and D-Link route. After check, they found when short-circuit the CTR and DTS can make it work (ZyWALL connect to the CDMA)

- [BUG FIX]

Symptom: Device crashes because of memory double free in Content Filter.

Condition:

(1) Enable Content Filter and Web site customization.

(2) After a while, the device will crash sometimes.

- [BUG FIX]

Symptom: Device crashes when enable CNM agent.

Condition:

(1)Enable AV/IDP/CNM.

(2)Disable AS.

(3)Block LAN to LAN packet from Firewall.

(4)Make LAN to LAN heavy traffic.

- [BUG FIX]

Symptom: Trace route fails to get response from our device.

Condition:

Topology:

PC-----(LAN)ZW70(WAN)

(1) On PC, try trace route a host(www.yahoo.com).

(2) Trace route cannot get response from our device.

- [BUG FIX]

Symptom: Device crashes (software watchdog wakes up by NAT).

Condition:

(1) Restore default romfile.

(2) After a while, the device will crash sometimes.

- [BUG FIX]

Symptom: Backuping the configuration of AntiVirus is too slow.

Condition:

(1) In eWC->SECURITY->ANTI-VIRUS->Backup & Restore, click "Backup" button to backup the AntiVirus configuration.

(2) Sometimes we need to wait for the popup window for a prolonged period of time.

Modifications in V4.01(XD.0) | 08/08/2006

Modify for formal release

Modifications in V4.01(XD.0)b5 | 07/31/2006

- [BUG FIX]

Symptom: Device crashes when upload F/W.

Condition:

Topology : PC_A == ZyWALL == P1 == PC_B

(1) Build tunnel between PC_A and PC_B and sent TFGEN traffic(1M) between PC_A and PC_B.

(2) Use eWC to upload F/W from ZyWALL’s WAN and device crashes.

Modifications in V4.01(XD.0)b4 | 07/11/2006

- [BUG FIX]

Symptom: Anti-Spam cannot work in NAT loopback situation.

Condition:

(1) Put PC1 and PC2 on LAN side of ZyWALL.

(2) ZyWALL enables Anti-Spam and disables External Database.

(3) PC2 installs the Merak Mail Server.

(4) PC1 uses the outlook express to send mail to itself by the mail server of PC2.

(5) When the PC1 is sending mails will cause mail stuck until timeout.

- [BUG FIX]

Symptom: Upload firmware by eWC will cause CPU load 100%.

Condition:

  - Use GUI to upload firmware will cause CPU 100%.

  - It will be successful, but need more than 1 minute.

- [BUG FIX]

Symptom: There should be a progress page when upload F/W by eWC.

Condition:

(1) Goto eWC>Maintenance to upload F/W.

(2) ZyWALL should show a progress page, but it is not.

(3) ZyWALL should display login page after reboot, but it is not.

Modifications in V4.01(XD.0)b3 | 06/25/2006

- [FEATURE CHANGE]

Change log format of Spam mail.

Was: Mail score is higher than threshold - Spam Score:<Score><Title>!<Direction>

Is: Mail score is higher or equal than threshold - Spam Score:<Score><Title>!<Direction>

- [FEATURE CHANGE]

Change some wordings which contain "fail back" in GUI and log.

Was: "Fail back ****".

Is: "Fall back ****".

- [FEATURE CHANGE]

WAS: In eWC>HOME page, the memory bar will become red when the percentage of memory usage is over 90%.

IS:  In eWC>HOME page, the memory bar will become red when the percentage of memory usage is over 95%.

- [FEATURE CHANGE]

In eWC>BW MGMT>Class Setup page, change wording:

WAS: "filter, to filter, (filter number)", "Filter class Search Order"

IS: "class, to class, (class number)", "Enabled classes Search Order"

- [ENHANCEMENT]

Enlarge Anti-Spam session number from 5 to 20

- [ENHANCEMENT]

Microsoft cryptographic library supports only odd-sized keys for generating the RSA-modulus. Let the key number of creator primes be odd-size.

Note: Without this enhancement, importing self-signed certificate with PKCS#12 format into MS IE sometimes will fail.

- [ENHANCEMENT]

  - In eWC>HOME page, show MAC address in Network Status Table.

  - Change ZyWALL eWC refresh pages to consistent with HOME page.

- [BUG FIX]

Symptom: Device will crash in bridge mode AV testing.

Condition: PC(mail client)----(LAN)DUT(WAN)----Mail Server

  - In bridge mode, enable AV and activate SMTP from LAN to WAN direction.

  - Disable Outlook SMTP authentication in PC.

  - PC on LAN and sent out Microsoft Outlook testing mail.

  - Device will crash immediately.

- [BUG FIX]

Symptom: ZyWALL WLAN & DMZ ports cannot work in dynamic VLAN ports.

Condition:

(1) Restore default romfile.

(2) Set Port Roles as 1>LAN, 2>LAN, 3>DMZ, 4>WLAN.

(3) Set DMZ IP as 10.10.2.1/24, DHCP as None.

(4) Set Wireless Card bridge to WLAN.

(5) Unplug wireless card and reboot device.

(6) PC connects to DMZ port, IP is 10.10.2.100/24 and gateway is 10.10.2.1, and the PC ping 10.10.2.1 will fail.

- [BUG FIX]

Symptom: The eWC>Firewall>Default Rule page will popup JavaScript error in router mode.

Condition:

(1) Go to eWC>FIREWALL>Default Rule page.

(2) Click Reset button, ZyWALL pop-ups a JavaScript error.

- [BUG FIX]

Symptom: Unknown crash.

Condition:

(1) Restore default romfile.

(2) Switch device to Active/Active mode, and confirm WAN1 and WAN2 can work fine.

(3) Set WAN2 ping check point to User-defined.

(4) After a while, the device sometimes will crash.

- [BUG FIX]

Symptom: IDP Total Sessions Scanned is wrong.

Condition:

(1) Enable AV, SMTP service and enable all directions.

(2) Enable IDP, but disable all traffic direction.

(3) Attacker sends the mail containing virus to victim via ZyWALL to check if Anti-Virus can detect viruses.

(4) In eWC>REPORTS>THREAT REPORTS, Total Sessions Scanned of IDP will count number. But it should not.

- [BUG FIX]

Symptom: ZyWALL crashes if you try to backup Configuration AV or IDP.

Condition:

(1) Go to eWC>Security>ANTI-VIRUS(or IDP)>Backup & Restore page.

(2) Click Backup or Restore button.

(3) System will crash sometimes.

- [BUG FIX]

Symptom: The ZyWALL should use user configured time server to do daily time adjustment.

Condition:

(1) Reboot the ZyWALL, set 'abc.abc.edu' as user defined 'Time Server Address'.

(2) The time synchronization will fail at start-up and use the default built-in time server list.

(3) The ZyWALL will always use one of built-in time servers to adjust time daily, but the ZyWALL should use user configured time server to do daily time adjustment.

- [BUG FIX]

Symptom: The IDP should work when the traffic is "from VPN to LAN".

Condition:

Topology

PCB-------ZYWALL----tunnel-----ZYWALL--------PCA

(1) Build a tunnel between PCA and PCB.

(2) Enable IDP and check the direction of "From VPN to LAN" and download a file "eicar.com" by HTTP.

(3) The IDP doesn’t detect the virus.

(4) But IDP works when you choose ’From LAN to VPN’.

- [BUG FIX]

Symptom: The device will crash when using VPN manual mode.

Condition:  PC1--ZWA--ZWB--PC2

(1) Add a VPN manual mode rule in both ZWA and ZWB and make sure PC1 can ping PC2 through the VPN tunnel.

(2) PC1 ping PC2 continuously.

(3) Unplug the physical link in WAN, the VPN traffic will pass through (ZWA).

(4) ZWA will crash.

- [BUG FIX]

Symptom: The incorrect data shows on the eWC>THREAT REPORTS>AV.

Condition:

(1) Enable AV and use Edonkey behind the ZyWALL.

(2) The incorrect data shows on the eWC>THREAT REPORTS>AV.

The detect virus name shows ’Unknown Signature’ and the Occurrence is very big, even is a negative number.

- [BUG FIX]

Symptom: Sometimes we cannot login ZyWALL by HTTP or HTTPS after enabling the password hash function.

Condition:

(1) Enable password hash function in SMT 24.8, "sys pwdHash on".

(2) After the convert of password, we can never login by HTTP or HTTPS.

- [BUG FIX]

Symptom: In ZyWALL 5 bridge mode, the Port statistics of eWC>HOME page shows "Dial Backup" port information.

Condition:

(1) ZW5 switches to bridge mode.

(2) Go to eWC＞HOME>Port statistics.

(3) The redirected page will show "Dial Backup" port information.

Modifications in V4.01(XD.0)b2 | 05/22/2006

- [FEATURE CHANGE]
The multicast AH or ESP packet will not pass to the VPN module in ZyWALL.

- [FEATURE CHANGE]
Change wording of one category name in external content filtering.
Was: Streaming Media/MP3
Is: Streaming Media/MP3/P2P

- [FEATURE CHANGE]
WAS: In SMT 24.8, "ipsec adjTcpMss auto" will let the "IPSec adjust TCP MSS" switch to auto mode.
IS: "ipsec adjTcpMss 0" will change to auto mode.

- [ENHANCEMENT]
(1) System Resources:
  1. Some memory, which is used by running features and system process, has gone in system resource bar. Add back this part of memory in the bar.
  2. Give a space between number and MB.
      WAS: 19/64MB;   IS: 19/64 MB
(2) Time representation: Modify eWC>home page>Up Time as a running clock.
(3) Firmware Version: Give eWC>Homepage>Firmware Version a hyperlink to eWC>Maintenance> F/W Upload.
(4) Security Services:
  1. Give eWC>Homepage>IDP/Anti-Virus Definitions a hyperlink to eWC>IDP> Update.
  2. Add eWC>Homepage>IDP/Anti-Virus Expiration Date a hyperlink to eWC>Registration> Service.
  3. Give eWC>Homepage>Anti-Spam Expiration Date a hyperlink to eWC>Registration> Service.
  4. Give eWC>Homepage>Content Filter Expiration Date a hyperlink to WC>Registration> Service.
(5) Interfaces
  1. Give each eWC>interface a hyperlink to link to the corresponding configuration page.

WAN1/WAN2 link to eWC>Network>WAN page

Dial Backup link to eWC>Network>WAN>Dial Backup page

LAN link to eWC>Network>LAN>LAN page

IP alias1/2 link to eWC>LAN>IP alias 1/2 page

WLAN link to eWC>Network>WLAN>WLAN page

IP alias1/2 link to eWC>WLAN>IP alias 1/2 page

DMZ link to eWC>Network>DMZ>DMZ page

IP alias1/2 link to eWC>DMZ>IP alias 1/2 page
(6) Remove underlines from the links in eWC>Homepage.
(7) Put eWC>Homepage a warning message for Turbo card is not installed.
(8) If there is no Turbo Card installed, the Security Services should be presented accordingly:

WAS: Intrusion Detected 0

Virus Detected 0

IS: Intrusion Detected N/A

Virus Detected N/A

- [ENHANCEMENT]

Support dual multiple WAN devices for IPSec HA scenario.

- [ENHANCEMENT]

Change the Anti-Spam wording in log.

WAS: "Mail Parser buffer is overflow!"

IS: "AS checking bypassed as a mail header line exceeds 1024 characters!"

- [ENHANCEMENT]

(1) Remove the eWC check box: Enable Firewall for VPN traffic.

(2) Remove CI command "ipsec swFwScan on|off".

- [BUG FIX] 
Symptom: Device crashes when sends large number of mails.
Condition:

(1) Enable Anti-SPAM and external database.

(2) Enable Bandwidth management in WAN and DMZ.

(3) Send and receive large number of mails between DMZ and WAN interface.

(4) Device will crash.

- [BUG FIX] 
Symptom: Traffic can’t pass VPN tunnel after a long while.
Condition: 
Topology:
PC1 (192.168.1.33) --- ZW_A (192.168.70.100) ==== VPN tunnel ==== (192.168.70.200)ZW_B --- (192.168.2.33)PC2
(1) VPN configuration on ZW_A:
IKE 1:	Secure gateway: 192.168.70.200

Enable XAUTH client

SA lifetime = 180 seconds
Policy 1:	Local network: 1.1.1.1/24

Remote network: 2.2.2.2/24

Enable Nail up

SA lifetime = 28800 seconds
Policy 2:	Local network: 192.168.1.33/24

Remote network: 192.168.2.33/24

SA lifetime = 180 seconds
(2) VPN configuration on ZW_B:
IKE 1:    	Secure gateway: 192.168.70.100

Enable XAUTH server

SA lifetime = 180 seconds
Policy 1: 	Local network: 2.2.2.2/24

Remote network: 1.1.1.1/24

SA lifetime = 28800 seconds
Policy 2:	Local network: 192.168.2.33/24

Remote network: 192.168.1.33/24

SA lifetime = 180 seconds
(3) PC1 ping PC2
(4) After a while the Policy 2 can’t be established anymore.

- [BUG FIX] 
Symptom: Some wordings in "eWC-＞ANTI-VURUS" are not correct.
Condition:

(1) Go to "eWC-＞ANTI-VIRUS-＞General".

(2) The wording "POP3 (TCP/UDP 110)” should be ”POP3 (TCP 110)"

(3) The wording "SMTP (TCP/UDP 25)” should be ”POP3 (TCP 25)"

- [BUG FIX] 
Symptom: The device can’t enable multiple proposal in IKE rule.
Condition:
(1) Add an IKE rule using "Preshare key" as authentication type.
(2) Add another IKE rule using "Certificate" as authentication type, different preshare key and enable the multiple proposals.
(3) This IKE rule cannot save.

- [BUG FIX] 
Symptom: In eWC>HOME>Network Status>more page, wireless cannot get correct port status. 
Condition:

(1) Insert G-110 wireless card.

(2) Switch device to bridge mode.

(3) Go to eWC>HOME>Network Status>more page.

(4) The "Port Status" of Wireless Card is 100M/Full, but SMT is 54M.

(5) The "Port Status" of WLAN Interface has no any information.

- [BUG FIX] 
Symptom: In PPTP encapsulation, enable VPN, AV and AS, PC can not receive the mail via VPN tunnel.
Condition:
PC1(mail-server:argosoft1.8)--(DMZ)ZW70(WAN:PPPoE)---(WAN:PPTP)ZW5(LAN) ------PC2(Outlook Express)
(1) Establish a VPN tunnel between ZW70 and ZW5.
(2) In ZW70, enable AV, disable AS.
(3) In ZW5, enable AS. 
(4) PC2 can’t receive the mail from PC1.

- [BUG FIX] 
Symptom: Bridge mode Network Status Bridge Port loss DMZ port.
Condition: 
Bridge mode in GUI Home> Network Status>More> Bridge Port loss DMZ port.

- [BUG FIX] 
Symptom: VPN rule swap fails on phase one ID check.
Condition:

Topology:

(LAN) Bridge_A (WAN)=======(WAN) Bridge_B(LAN)

(1) On Bridge_A, add a VPN rule:

IKE: Static rule, enable XAUTH and set as client mode.

Local ID: Type=DNS Content = d.c.b.a

Peer ID: Type=DNS Content = a.b.c.d

IPSEC Policy: Local=Single 1.1.1.1, Peer=Single 2.2.2.2

(2) On Bridge_B, add two VPN rules:

1. Rule one:

IKE: Static rule, XAUTH is disabled.

Local ID: Type=DNS Content = a.a.a.a

Peer ID: Type=DNS Content = b.b.b.b

IPSEC: Local=Single 3.3.3.3, Remote=Single 4.4.4.4

2. Rule two:

IKE: Dynamic rule, enable XATUH and set as server mode.

Local ID: Type=DNS Content = d.c.b.a

Peer ID: Type=DNS Content = a.b.c.d

IPSEC Policy: Local=Single 1.1.1.1, Remote=Single 2.2.2.2
(3) Dial VPN tunnel from Bridge_A to Bridge_B, the VPN tunnel will fail to build up
by phase one ID mismatch.

- [BUG FIX] 
Symptom: User can’t receive mail through VPN tunnel when WAN is in PPTP encapsulation.
Condition: 
Topology:
PC1 (mail client) --- ZW5 (PPTP) === VPN tunnel === ZW70 ---- PC2 (mail server)
(1) Establish VPN tunnel between ZW5 and ZW70.
(2) ZW5's WAN is PPTP, enable AS.
(3) ZW70's WAN can be any encapsulation type, disable AS.
(4) PC1 receives mail from PC2 but it fails.

- [BUG FIX] 
Symptom: Asymmetrical route cannot work.  
Condition:

Topology as follows:

PC (A) ---- [L]DUT(B)[W] ------- Internet --- HTTP server(D)(66.102.7.104)

|                      |

-- [L]Router(C)[W] --- Internet

(1) DUT configures a static route that forwarding packets of destination IP 66.102.7.104 through internal link to Router(C).

PC (A)’s default route entry is DUT (B).

Router (c) is NAT enabled.
(2) PC (A) establishes HTTP connection to HTTP server (D).

a. SYN Packet: A -> B (LAN) -> C (LAN) -> C (WAN) -> D.

b. SYN ACK Packet: D -> C (WAN) -> C (LAN) -> A.

c. ACK Packet: A -> B (LAN), and DUT drop it.

- [BUG FIX] 
Symptom: Trigger port can’t be reconnected.
Condition:
Topology:
PC1(192.168.1.33)------(LAN)ZyWALL(WAN:192.168.70.175)-----PC2(192.168.70.176)   
(1) Reset to default romfile.
(2) Go to eWC>WAN>WAN1, set WAN IP Address=192.168.70.175.
(3) Go to eWC>NAT>Port Triggering>WAN1 Interface>Index 1, set Name=ftp, Incoming Start Port=21, incoming End Port=110, Trigger Start Port=21,Trigger End Port=21.
(4) Disable Firewall.
(5) PC1 ftp to PC2, and then PC2 ftp to PC1.
(6) PC2 disconnects ftp session and then reconnects to PC1 will be fail, while PC1 ftp session still connected.

- [BUG FIX] 
Symptom: GUI popup java script error in eWC>NAT>NAT Overview
Condition:

(1) Go to eWC>NAT>NAT, change Max concurrent session per host to 500 and press key "Enter".
(2) ZyWALL popup java script error.
(3) The status bar shows "spSave () fail with Error -6103".

- [BUG FIX] 
Symptom: Redundant gateway sometimes can’t be saved if it's in domain name format.
Condition:
(1) Create an IKE rule with IPSEC HA is enabled.
(2) Configure a non-exist domain name as redundant gateway.
(3) Let Domain Name Update Timer query this non-exist domain name. It will fail.
(4) Try to modify the domain name with a correct one and save it.
(5) Several minutes later, users will find the domain name has not been changed; it's still the old one.

- [BUG FIX] 
Symptom: In eWC>VPN, VPN Rules page shows incorrect domain name.  
Condition:
(1) Go to eWC>DNS>DDNS, set a WAN domain name as "123456789.123456789.123456789.123456789.123456789.123456789.123".
(2) Go to eWC>VPN, create a VPN rule using My domain as 123456789.123456789.123456789.123456789.123456789.123456789.123".
(3) While applying the setting, VPN Rules page shows incorrect domain name.

- [BUG FIX] 
Symptom: Wireless client still can scan wireless network after disabled wireless card.
Condition:
(1) Plug in G100/G110 wireless card.
(2) Go to eWC/Network/Wireless Card/Wireless Card, enable wireless card and set ESSID as "testWlan".
(3) Wireless Client can scan the "testWlan" network by Odyssey tool.
(4) Disable wireless card.
(5) Wireless Client still can scan the "testWlan" network by Odyssey tool.

- [BUG FIX] 
Symptom: ZyWALL crashes when setting NAT address mapping rules. 
Condition:

(1) Go to eWC＞NAT＞Address Mapping page.

(2) Add a new rule, configure

Type= Many-to-Many-Overload,

Local Start IP= 1.1.1.1

Local End IP= 3.3.3.3

Global Start IP= 4.4.4.4

Global End IP= 5.5.5.5

(3) Click "Apply" button, then ZyWALL crashes.

- [BUG FIX] 
Symptom: Change WAN IP in GUI, the "Private" option in SMT11.1->Edit IP will be set as "NO".
Condition:
(1) Go to SMT11.1, configure Encapsulation as "PPPoE" or "PPTP".
(2) Go to SMT11.1->Edit IP, change "Private" to "Yes".
(3) Go to eWC->WAN->WAN1, set IP as static IP address.
(4) Go to SMT11.1->Edit IP, the value of "Private" will become "No".

- [BUG FIX] 
Symptom: NAT Many-to-Many Overload rule cannot be set in eWC. 
Condition:

(1) Go to eWC>NAT>Address Mapping page, click "Insert" button.

(2) In NAT - ADDRESS MAPPING page, select Type= Many-to-Many Overload.

(3) Click the "Apply" button, and the status shows "Extra characters were detected in the item".

- [BUG FIX] 
Symptom: NAT historical high NAT session per host will over one session than Max          concurrent session per host.
Condition:
(1) Go to eWC>NAT>NAT overview, change Max concurrent sessions per host to 500.
(2) Use BluePortScan to do port scan.
(3) Historical high session per host is 501.

- [BUG FIX] 
Symptom: Anti-Spam cannot work in NAT loop back situation.
Condition:

(1) Put PC1 and PC2 on LAN side of ZyWALL.

(2) ZyWALL enables Anti-Spam and disables External Database.

(3) PC2 installs the Merak Mail Server.

(4) PC1 uses the outlook express to send mail to itself by the mail server of PC2.

(5) When the PC1 is sending mails will cause mail stuck until timeout.

- [BUG FIX] 
Symptom: Device responds an invalid sysObjectID value while SNMP browsing.
Condition:
(1) Restore default romfile.
(2) MIB browser connects to device and will get invalid value enterprises.890.1.2 (prestige).

- [BUG FIX] 
Symptom: VPN can be successfully built up with wrong IPSec rule.
Condition:

Topology:

(LAN) ZyWALL_A (WAN)=======(WAN) Bridge_B (LAN)

(1) On ZyWALL A, add a VPN rule:

IKE: Static rule, enable XAUTH and set as client mode.

IPSEC Policy: Local=Single 1.1.1.1, Remote=Single 2.2.2.2

(2) On Bridge_B, add two VPN rules:

1. Rule one:

IKE: Static rule, enable XAUTH and set as server mode.

IPSEC: Local=Single 3.3.3.3, Remote=Single 4.4.4.4

2. Rule two:

IKE: Dynamic rule. XATUTH is disabled.

IPSEC Policy: Local=Single 1.1.1.1, Remote=Single 2.2.2.2

(3)Dial VPN tunnel from ZyWALL_A to Bridge_B, the VPN tunnel will be successfully built up with Bridge_B’s rule two.

- [BUG FIX] 
Symptom: The eWC>Firewall>Default Rule page will pup up JavaScript error in bridge mode. 
Condition:

(1) Go to eWC>FIREWALL>Default Rule page.

(2) Click Reset button, ZyWALL pup up JavaScript error.

- [BUG FIX] 
Symptom: Device crash (Soft watchdog starts up.)
Condition:

(1) Firewall+NAT+AV+IDP+AS+AS black list+LB

(2) LAN has a mail client、mail server；DMZ has a mail client、2 mail server；

WLAN has a mail client. All of them are on IxLoad

(3) Run IxLoad 10 minutes，device crash

- [BUG FIX] 
Symptom: Traffic can’t go out after use the tfgen tool.
Condition: 
(1) Restore default rom file.
(2) In LAN, use the TfGen with following setting.
Utilization: 40000; Destination: 168.95.1.1; Port: 777;
After using the tfgen, all the traffic from LAN can’t go outside.

Modifications in V4.01(XD.0)b1 | 04/24/2006

- [ENHANCEMENT]
(1) Add UTM reports for IDP/AV/AS.
(2) Change linkage from GUI>Logs>Reports to GUI>UTM Reports>System Reports.
(3) Re-layout UTM Home GUI for ZyWALL 4.01.

- [ENHANCEMENT]
Add redundant IPSec gateway (IPSec HA).

- [ENHANCEMENT]
IPSec traffic can be managed by security rule (IDP/AV/AS/FW/CF/BM)

- [FEATURE CHANGE]
Was: IPSec auto-build tunnel command can only build tunnels with same secure gateway IP.
Is: Users can automatically build VPN tunnels with incremental secure gateway IP addesses.
Usage of CLI command: ipsec build<secure gateway> <local IP address> 
<remote IP address> <Nailed-Up> <num> <Control ping> in which

- [ENHANCEMENT]
Add direction matrix setting in Firewall/AV/AS/IDP.

- [ENHANCEMENT]
Chage weighting of Anti SPAM servers based on average time and fail rate.

- [ENHANCEMENT]
(1) Add CI command to see the runtime data for AntiSpam. 
"as display runtimedata <all|black|white> [all|ip|mime|email|subject]"
(2) Wildcard support for subject and email fields in black list and white list.
1. Support "*" to indicate match any character 0 or more times.
2. It is case-insensitive.
3. The maximum length of the email and subject fields is 63 characters.

- [ENHANCEMENT]
Add PKCS12 for ZyNOS.

- [ENHANCEMENT]
WLAN Zone enhancement. 
(1) ZyWALL has an independent WLAN Zone interface, no matter WLAN card. 
(2) WLAN card is not the independent WLAN interface.
(3) WLAN card can be bridged to LAN, DMZ and WLAN Zone interface.

- [ENHANCEMENT]
support WLAN in "ip nat routing" CI command. Turn on this option for LAN/DMZ/WLAN, packets will be routed when it cannot match any NAT rule.

- [ENHANCEMENT].
Add a CI command "ip alg ftpPortNum [port number]" to support a different port number on FTP ALG. This port is an additional FTP ALG port, the original FTP port(21) still works. Note: This CI command will not save to SPT, so user will need to put into autoexec.net if they want to keep the setting.

- [ENHANCEMENT]
Consolidate "Router reply ICMP packet" log.
(1) Router reply ICMP packet: ICMP(Port Unreachable).
(2) Router reply ICMP packet: ICMP(Host Unreachable).

- [ENHANCEMENT]
Add a CI command "sys arp ackGratuitous", let ZyWALL to support gratuitous ARP request and update MAC mapping on ARP table for the sender of this ARP request. There are two subcommands under "ackGratuitous":
(1) "active [yes|no]": Let ZyWALL accept gratuitous ARP request. 
(2) "forceUpdate [on|off]" If zywall ARP table already had target IP address ARP entry, forceUpdate option will update the exist MAC mapping to new one.

- [FEATURE CHANGE]
WAS: The ZyWALL uses a fixed NTP server list with 10 NTP servers to adjust the system time.
IS: Use 0.pool.ntp.org, 1.pool.ntp.org, 2.pool.ntp.org instead of specific NTP servers to adjust the system time. 
The pool.ntp.org is a virtual cluster of timeservers, it uses a round robin way to provide different NTP server to clients.

- [ENHANCEMENT]
Device will detect if Turbo Card is instered or not to determine the NAT and TOS session number. Without Turbo Card inserted, device will recover NAT and TOS session number to 6000.

Modifications in V4.00(XD.8) | 03/16/2006

Modify for formal release.

Modifications in V 4.00(XD.8)b1 | 03/10/2006

- [ENHANCEMENT]
Support Green Product Turbo Card

- [FEATURE CHANGE]
Change the command usage for saving password with hashed by MD5 in the ROM file.
WAS: sys pwdEncryption <on | off> [newPassword] [oldPassword]
IS: sys pwdHash <on | off> [newPassword] [oldPassword]

Modifications in V4.00(XD.7) | 02/24/2006

Modify for formal release.

Modifications in V4.00(XD.7)b1  | 02/17/2006

- [BUG FIX]
Symptom: Device crashes after about 5 minutes with default romfile.
Condition:
(1) Restore default romfile.
(2) Only connect WAN port to internet.
(3) Do NOT access eWC.
(4) After about 5 minutes, device crashes with Prefetch Abort message.

- [FEATURE CHANGE]
WAS: Device accepts any ARP reply information if has its ARP entry.
IS: Device does not accept ARP reply information which does not have corresponding ARP request by device itself.

- Symptom: GUI and SMT behavior is not consistent.
Condition: 
(1) In eWC>VPN>Global Setting page, "Adjust TCP MSS" can be configured as uint16  but as an integer in SMT.

- [BUG FIX]
Symptom: ZyWALL crashs when receiving unsupported IKE packet.
Condition: 
	ZW5A-------(tunnel)------ZW5B
        |------PC(send IKE unsupported packetl)
(1) Edit an IKE rule and an IPSEC rule.
(2) PC sends unsupported IKE exchange type packet.
(3) ZyWALL will crash.

- [ENHANCEMENT]
Add a CI command "ip arp ackGratuitous", let ZyWALL to support gratuitous ARP request and update MAC mapping on ARP table for the sender of this ARP request.
There are two subcommands under "ackGratuitous":
(1) "active [yes|no]":
      Let ZyWALL accept gratuitous ARP request.
(2) "forceUpdate [on|off]"
      If zywall ARP table already had target IP address ARP entry, forceUpdate option will update the exist MAC mapping to new one.

- [ENHANCEMENT]
Add a CI command, "ipsec initContactMode gateway|tunnel", to support multiple VPN clients which located behind the same NAT router can build VPN tunnel to ZyWALL.

- [BUG FIX]
Symptom: IKE last packet send out after ESP packet.
Condition: 
(1) A PC continuously generates traffic from LAN and it matches a VPN tunnel.
(2) ZyWALL should intiate the VPN tunnel.
(3) After receiving the second packet of quick mode from the peer, ZyWALL send some ESP packets before sending the last quick mode packet.
(4)  It should send out the last quick mode packet first, and then send those ESP packet.

- [ENHANCEMENT].
Add a CI command "ip alg ftpPortNum [port number]" to support a different port number on FTP ALG. 
Note: This port is an additional FTP ALG port, the original FTP port (21) still works.

Modifications in V4.00(XD.6) | 02/06/2006

Modify for formal release.

Modifications in V4.00(XD.6)b1  | 01/23/2006

- [ENHANCEMENT]
Add CLI command "as scoreTimeout" to change AS rating server query timeout value.

- [ENAHNCEMENT]
CLI command "as display serverlist" shows information about rating server query round trip time.

- [ENHANCEMENT]
The password saved in ROM file can be encrypted by MD5.
(1) "sys pwdEncryption <on | off> [newPassword] [oldPassword]"
(a)Use this CI command to turn on or off this feature. Once the feature is on in a ROM file, the F/W without this feature support can not deal the ROM file well. Ex. login problem.
(b) To turn off the feature, you must provide two password, "newPassword" is the new password that will be saved in the ROM file in plaintext. "oldPassword" is the original administration password that is for security issue.
(2) "sys md5 <string>" Input a string, it will output the md5 code.

- [ENHANCEMENT]
Add CI command, "ipsc swSkipPPTP [on/off]", to let all traffic pass through VPN tunnel setting not to apply on PPTP traffics.

- [ENHANCEMENT]
ZyNOS adds device local port conflict protection. ZyWALL will avoid port 1029 as local port.

- [FEATURE CHANGE]
WAS:  The DDNS of ZyWALL will not update IP when the ZyWALL's WAN IP is static.
IS:      The DDNS of ZyWALL will update IP when WAN IP changes, no matter the ZyWALL's WAN IP is static or dynamic.

- [FEATURE CHANGE]
WAS: If the zip session is over the maximum zip sessions, this session is bypassed.
IS: Add CI command to decide this session is bypassed or blocked.

- [FEATURE CHANGE]
Expend dial bacokup initial string length from 31 characters to 63 characters.

- [BUG FIX]
Symptom: The registered username is wrong in eWC->REGISTRATION->Registration page.
Condition: 
(1) A registered device with username abcdefgh.
(2) In 24.8, type "sys myZyxelCom load".
(3) Type "sys myZyxelCom config username 1234567890".
(4) Type "sys myZyxelCom save".
(5) Type "sys myZyxelCom serviceRefresh".
After the refresh is finished.
(6) Type "sys myZyxelCom load".
(7) Type "sys myZyxelCom display".
(8) You can see the username field is wrong "abcdefgh90", it should be "abcdefgh".

- [BUG FIX] 
Symptom: Can not change gateway IP address to "0.0.0.0".
Condition: 
(1) In eWC->NETWORK->WAN->WAN1(WAN2), set WAN interface as static IP address and gateway = "10.0.0.1".
(2) Change gateway IP address to 0.0.0.0 and click "Apply".
(3) Goto eWC->NETWORK->WAN->WAN1(WAN2), the gateway IP address is still "10.0.0.1".

- [BUG FIX] 
After we rebooted ZyWALL, ZyWALL fails to transmit data through VPN.
Condition: 
Topology: PC1------(LAN)ZW A(WAN)=====+======(WAN)ZW B(LAN)
                                              192.168.167.1         |           192.168.1.1
                                                                   DHCP Server
(1) On ZW A, WAN uses dynamic IP address and set a static VPN rule with policy as below:
IKE: Nail-up is ON  
Local: Subnet Type   192.168.167.0/24
Peer:  Subnet Type    192.168.1.0/24
(2) On ZW70, set a Dynamic VPN rule with policy as below:
Local: Subnet Type   192.168.1.0/24
Peer:  Any
(3) PC1 ping ZW B's LAN IP. Ping result is OK.
(4) Reboot ZW A.
(5) Check ZW A eWC SA monitor page, we can see a new VPN tunnel was successfully built up.
(6) PC1 ping ZW B's LAN IP again. Ping result is fail.

- [BUG FIX] 
Symptom: Anti Spam cannot work in NAT loopback situation.
Condition:
(1) Put PC1 and PC2 on LAN side of ZW70W.
(2) ZW70W enables Anti Spam and disables External Database.
(3) PC2 installs the ArgoSoft Mail Server.
(4) PC1 uses the outlook express to send mail to itself by the mail server of  PC2.
(5) When the PC1 is receiving mails will cause mail stuck until timeout.

- [BUG FIX] 
Symptom: VPN tunnel up time of ZyWALL private MIB has some problems.
Condition: 
(1) Successfully build a VPN tunnel.
(2) Use MIB browser to get the up time value from ZyWALL. The returned result is correct.
(3) Add a new ipsec policy.
(4) Get the up time value again. The returned result of the built VPN tunnel is "0(days) 00:00:00".

- [BUG FIX] 
Symptom: AS log is not correct.
Condition: The source/destination IP address in log "Exceed maximum mail session" are the same.

Modifications in V4.00(XD.5) | 01/06/2006

Modify for formal release.

Modifications in V4.00(XD.5)b1  | 01/05/2006

- [BUG FIX]
Symptom: The incremental rules may not work after updating signature. 
Condition: 
(1). Register myzyxel.com and update the signature.
(2). Enable the IDP feature.
(3). For some time, update the signature.
(4). The incremental rules may not work.

Modifications in V4.00(XD.4) | 12/15/2005

Modify for formal release.

Modifications in V4.00(XD.4)b1| 12/13/2005

- [BUG FIX] 051202307
Symptom: DUT can not block infected zip file. 
Condition: 
(1) Use I.E. browser to get  http://www.vx.netlux.org.
(2). DUT can not block the infected zip file, which extended file name is not “zip”.

- [BUG FIX] 051208573 
Symptom: User updated some version signature, IDP/AV configuration may be lost.
Condition:
(1) If user updated 1.092 version signature, IDP/AV configuration will be lost.

Modifications in V4.00(XD.3) | 12/09/2005

Modify for formal release.

Modifications in V4.00(XD.3)b1| 12/06/2005

- [ENHANCEMENT]
In eWC->LAN (DMZ, WLAN)->LAN (DMZ, WLAN) page, the DHCP WINS servers now can be configurable via GUI.

- [ENHANCEMENT]
In eWC->VPN-> Global Setting page, add two fields "Adjust TCP Maximum         Segment Size" and  "VPN rules skip applying to the overlap range of local and remote IP addresses."

- [ENHANCEMENT]
VPN configuration by CI commands "ipsec adjTcpMss" and "ipsec swSkipOverlapIp" will be reflected in the two fields of GUI -- "Adjust TCP Maximum Segment Size" and "VPN rules skip applying to the overlap range of local and remote IP addresses.", and vice versa.

- [FEATURE CHANGE]
Login-Name for PPPOE, PPTP need support 63 characters, Password need support 31 characters in GUI.

- [BUG FIX]
Symptom: VPN Relay does not work.
Topology:Branch A[ZyWALL30W] === HQ[ZyWALL5] ==== Branch B[ZyWALL30W]
Condition: 
		Device Settings:
			Branch_A
				WAN:10.0.0.2
				LAN:192.168.167.0/24
			HQ
				WAN:10.0.0.1
				LAN:192.168.168.0/24
			Branch_B
				WAN:10.0.0.3
				LAN:192.168.169.0/24
		VPN settings:
			Branch_A
				Local IP address 192.168.167.0/24
				Remote IP address 192.168.168.0~192.168.169.255
			Headquarter
			(1) 
				Local IP address 192.168.168.0~192.168.169.255 
				Remote IP address 192.168.167.0/24
			(2)
				Local IP address 192.168.167.0~192.168.168.255 
				Remote IP address 192.168.169.0/24
			Branch_B
				Local IP address 192.168.169.0/24
				Remote IP address 192.168.167.0~192.168.168.255.
Action:	Dial up VPN in from both Branch A and Branch B to HQ. Then you can pingBranch B from Branch A, but you can not login any FTP server in Branch A from Branch B. In packets trace, it seems that ZYWALL5 with 4.00(WZ.2)C0 can not relay TCP packets in VPN from wan-to-wan.

- [BUG FIX]
Symptom: Under bad network environment, transmit a lot of packets by a VPN tunel, there are a lot of "Replay Packet" log entries.
Condition:
(1) Network environment is bad (ex: heavy traffic).
(2) Build up a VPN tunnel.
(3) Transmit heavy traffic through the tunnel, after few days it shows a lot of "Replay Packet" log entries.

- [BUG FIX] 
Symptom: Change Log Mail server, mail will send to old mail server.
Condition: 
(1). Fill Log mail server ”mail.zyxel.com.tw” and fill other fields by correct data (Enable SMTP Authentication) 
(2). Click ”E-mail Log Now” and receive this log will successful 
(3). Modify Mail server to ”mail.aaa.com.tw” click ”E-mail Log Now", user also can receive this log mail.

- [BUG FIX] 
Symptom: VPN rule with subnet mask 0.0.0.0 should allow all traffic to pass through VPN, but it doesn’t. 
Condition: 
1. Restore default ROM file.
2. Set up a VPN policy with remote address type = subnet, remote starting IP address = 0.0.0.0, remote subnet mask = 0.0.0.0.
3. Trigger tunnel by local PC, and it wil never trigger tunnel.

- [BUG FIX] 051014149
Symptom: PC can't send the mail (L to W: SMTP) when the device's bridge and IP is different subnet with the mail client.
Condition: 
Topology: Mail Server(192.168.12.123/24)---Internet----Device(192.168.11.9/23)----PC(192.168.12.163/24)
(1) Change the device to Bridge Mode, IP = 192.168.11.9, Mask = 255.255.254.0, Gateway = 192.168.10.11, DNS = 168.95.1.1.
(2) Edit web eWC/Anti Spam, Enable Anti Spam = Enable.
(3) Edit web eWC/Anti Spam/External DB Enable, threshold = 0.
(4) PC can't send the mail to MailServer.
(5) if we disable Anti-Spam or change the device's IP subnet to 192.168.12.x/24, it  works.

- [BUG FIX]
Symptom: Mail stuck when enable Anti-Spam, because of checksum error.
Condition: 
          Topology:  Client ------(W) ZYWALL (L) ------ Mail Server
(1) Enable AS.
(2) Set port forwarder default server to Mail server.
(3) Client receive mails, sometimes mail stuck.

- [BUG FIX]
Symptom: Mail get stuck.
Condition:
    Mail receive/send stuck when AS is on and mail is going through VPN tunnel.

- [BUG FIX]
Symptom:In eWC->VPN>Global Setting page, warning messages is not correct.
Condition:
WAS: (Warning: When this checkbox is checked, you may not access device because of triggering VPN tunnels) 
Warning messages should be :
(Warning: When this checkbox is not checked, you may not access device because of triggering VPN tunnels).

- [BUG FIX] 
Symptom: Using Outlook Express to receive mails with ZyWALL Anti-Spam enabled, it will stuck until timeout.
Condition:
(1) PC1 -- [LAN]ZW35_A[WAN] -- [WAN]ZW35_B[LAN] -- PC2.
(2) ZW35_A enables NAT + Firewall + Anti-Spam, and Anti-Spam enables 	  external database, Spam Tag = "[**SPAM**]", Tag for No Spam Score = "".
(3) ZW35_B enables NAT + Firewall.
(4) PC2 installs the ArgoSoft Mail Server.
(5) PC1 uses the outlook express to send mail to itself by the mail server of  PC2.
(6) When the PC1 is receiving mails will cause mail stuck until timeout.

- [BUG FIX]
Symptom: All traffic goes through VPN does not work.
Condition: 
Topology: PC1----------(LAN)ZW35A(WAN)===Internet===(WAN)ZW35B(LAN)--------PC2
    192.168.1.1/24             |                                           192.168.2.1/24 
 (1) On ZW35A, set a Static VPN rule with policy as below:
    Local: Subnet Type
		       192.168.1.0/24
	Peer:  Single Type
		       0.0.0.0
(2) On ZW35B, set a Dynamic VPN rule with policy as below:
	        Local: Single Type
	               0.0.0.0
	        Peer:  Any
(3) Under the setting, we expect all PC1's traffic to PC2 will go through VPN tunnel to ZW35B first then to PC2.
(4) But it doesn't work.

Modifications in V4.00(XD.2) | 10/26/2005

Modify for formal release.

Modifications in V4.00(XD.2)b2| 10/19/2005

1. [BUG FIX] 051013130

Symptom: Convert rom file from 3.64 to 4.00, Max. Concurrent session Per Host has some problem.

Condition:

(1) Upgrade firmware from 3.64 to 4.00.

(2) In eWC->ADVANCE->NAT, Max. Concurrent Sessions Per Host is 6000, it should be 4000.

2. [BUG FIX] 051014221, 051014222, 051014223

Symptom: Spelling error in eWC->Registration page.

Condition:

(1) In eWC->REGISTRATION-> Registration page, set two different passwords.

(2) Press "Apply" button, the status shows "Password and Confirm password are differencet".

(3) A word "differencet" spells error. It should be "different".

3. [BUG FIX]

AS fail count will not be increased even the real timeout occurs

4. [BUG FIX] 050928542, 051012075, 051012076, 051012077

Symptom: The added source IPs of Firewall rule will be lost.

Condition:

(1) Go to GUI->FIREWALL->RULE EDIT page.

(2) Edit a firewall rule.

(3) Add a source IP(or destination IP) that exceeds its maximum size(20 for ZW5).

(4) The added item will be lost.

5. [FEATURE CHANGE] 051018364 , 051018365, 051018366

In eWC->Registration page, change Username field behavior.

WAS: "-" character is not allowed to key in.

IS: "-" character is allowed to key in.

6. [BUG FIX] 051018403

Symptom: PPTP (GRE) cannot pass through NAT.

Condition:

PPTP Server(192.168.1.33)--(LAN:192.168.1.1)DUT(WAN:192.168.11.100)--PC(192.168.11.200)

(1) Add PPTP Server(192.168.1.33) as Default Server in Port Forwarding

(2) Firewall is disabled.

(3) PC(192.168.11.200) can not dial in PPTP on 192.168.11.100

7. [BUG FIX] 051014198, 051014199, 051014200

Symptom: Use registration wizard to enable service, and last page wording error.

Condition:

(1) In eWC->HOME->Internet Access button, go to the last page.

(2) Registration status wording was wrong.

Modifications in V4.00(XD.2)b1| 10/08/2005

1.  [BUG FIX] 050906259

Symptom: Disable bridge mode Firewall "Log Broadcast Frame". Broadcast logs always appear.

Condition:

(1) In bridge mode, disable all Firewall -> Default Rule -> "Log Broadcast Frame".

(2) Broadcast logs always appear.

2.  [BUG FIX] 050825052

Symptom: Tfgen tool causes router crash.

Condition:

(1) Use tfgen to send 40000 to 172.21.0.254 and turn it off.

(2) Use "dev chan disp enet3" to make sure the sending bit is 1.

(3) Unplug and plug wan2 and router will crash.

3.  [BUG FIX]050912438

Symptom: Device will hang and reboot after "Email Log Now" in bridge mode.

Condition:

(1) Topology(Public IP):  PC(211.72.158.115) --- [LAN]ZW70_BridgeMode(211.72.158.116)[WAN] --- Internet/MailServer/MailRecipient.

(2) Set the device as Bridge mode.

(3) Configure eWC->LOGS: "E-mail Log Settings".

(4) Click eWC->"Email Log Now" to send log mail.

(5) System will hang and then reboot by software watchdog.

4.  [BUG FIX]050905192

Symptom: Anti-Spam causes memory leak in bridge mode.

Condition:

(1) Topology: Mail Client --- ZyWALL --- Mail Server

(2) Turn on Anti-Spam at ZyWALL (Bridge Mode).

(3) Mail Client sents mail to Mail Server. (You can try 500 mails with 2 attachments, total size is about 30k).

(4) ZyWALL memory leaks.

5.  [BUG FIX] 050922955

Symptom: After updating signature, sometimes the server IP address is incorrect in centralized log.

Condition:

(1) In SMT 24.8, type "sys update signatureUpdate".

(2) After updating signature, type "sys log dis".

(3) Sometimes you can see a signature update log with incorrect server IP "127.0.0.1".

6.  [ENHANCEMENT]

In eWC->FIREWALL->EDIT RULE page, we added the limitation on the number of source ip address and destination ip address. The limitation is 20.

7.  [ENHANCEMENT]

The device will not retry to update the signature if the update is triggered by user. Ex. CI command "sys update signatureUpdate", "idp update start", "av update start" or "Update Now" button in eWC.

8.  [ENHANCEMENT]

In eWC>Anti-Spam>General>Action taken when mail sessions threshold is reached, the wording of "Discard" will mislead user to think the system will "drop the mail" when mail session reach the system's limit.  In fact, the system doesn't drop the mail, it just drop the mail connection until system have an available mail session to process incoming connection. We replaced "Discard" with "Block" and the wording of "Block" will be explained in web help and User's Guide by "System will Block this mail until a mail session is available".

9.  [BUG FIX]

Symptom: Sometimes device will crash when receiving special mails.

Condition:

Topology: Mail_Client --- ZyWALL --- Mail_Server

(1) ZyWALL turn on Anti-Spam, turn on external DB, threshold = 0.

(2) Mail_Client receive mail from Mail Server

(3) Sometimes ZyWALL will crash due to ”Data Abort”, ”not mbuf cookie”, ”mbuf double free”, or mail did not tagged with spam string.

10. [BUG FIX] SPR ID: 050926383,050926384,050926385

Symptom: AS+AV Enable, it can’t send or receive mail if attached virus files.

Condition:

(1) AS and AV enable.

(2) AV General Setup select all.

(3) Send or receive a mail with attached virus files.

(4) It will can’t send or receive mail.

11. [BUG FIX] 051003282

Symptom: PC cannot transfer file from server (172.20.0.38)

Condition:

Topology: PC ---- ZyWALL(WAN:172.x.x.x)(Bridge/Router) --- trunk (172.20.0.38)

(1)   Restore default romfile.

(2)   PC get file from trunk, but it always fails after several seconds.

12. [BUG FIX] SPR ID: 050930643

Symptom: Edit NAT port forwarding default server = 192.168.1.33, then ping from DUT2 to DUT1, it should show W to L logs, but it show W to W logs.

Condition: PC1-------LAN DUT1 WAN-------PQA LAB-----------WAN DUT2 LAN

(1) Set with CI commend ”sys romr|y”

(2) Edit web eWC/WAN/WAN1,My WAN IP Address =172.202.77.121,My WAN IP Subnet Mask=255.255.0.0 ,Gateway IP Address=172.202.77.1

(3) Edit NAT port forwarding default server = 192.168.1.33, then ping from DUT2 to DUT1, it should show W to L logs, but it show W to W logs.

-> If we telnet from DUT2 to DUT1, it shows W to L logs, and this right.

-> If we ping from DUT2 to DUT1, it shows W to W logs, but it should show W to L logs.

13. [BUG FIX] 051003323

Symptom: NAT many one to one cannot work.

Condition:

(1) Edit web eWC/NAT/Address Mapping,WAN Interface =WAN2,Insert a Many One-to-One rule (Local Start IP=192.168.1.41,Local End IP=192.168.1.42,Global Start IP=192.168.12.100,Global End IP=192.168.12.101) on eWC/NAT/Address Mapping page

(2) Set with CI command ”ip nat reset enif1”

(3) 192.168.12.110 do port scan 192.168.12.100(port 1-100) and 192.168.12.101(port 1-100)

(4) 192.168.1.41 and 192.168.1.42 cannot capture all port scan packets.

14. [BUG FIX] 050930647

Symptom: Some mails should have SPAM tag or NoScore tag but they didn't have any tag

Condition:

(1) Enable AS

(2) eWC->AS->ExternalDB-: Enable external DB, set the threshold=0, fill the tag for no spam score

(3) MS Outlook Express received a lot of mails from the mail server

(4) Some mails did not have any Spam/No Score tag.

15. [FEATURE CHANGE]

WAS: Allow timeouted ConeNAT session to recreate NAT session from WAN to LAN.

IS: Do not allow timeouted ConeNAT session traffic to recreate NAT session from WAN to LAN.

Modifications in V4.00(XD.1) | 09/26/2005

Modify for formal release.

Modifications in V4.00(XD.1)b2| 09/21/2005

1.  [BUG FIX] 
Symptom: Content filter was registered in router mode and changed to bridge mode without configure DNS server. One PC open a web site can make DUT crash. 
Condition:
(1) In router mode, register content filter and enable it. Edit eWC/Content Filter/Categories/Select Categories, and enable some items (Pornography, Business, Gambling, etc.)
(2) Change DUT to bridge mode without configure DNS server.
(3) PC1 on LAN open a website, and IE would show “block (DNS resolving failed)”
(4) DUT crashed.

Modifications in V4.00(XD.1)b1| 09/12/2005

1.  [ENHANCEMENT]
Add CI command "ip urlfiler bypass [LAN/DMZ/WAN] [ON/OFF]" to let traffic matches LAN->LAN, DMZ->DMZ or WAN->WAN directions can be bypassed content filtering.
NOTE: (1) This is a runtime CI command, user can add it into autoexec.net.

(2) This command only support in router mode.

2.  [ENHANCEMENT] 
Periodically sending the keep-alive zero window TCP ACK when the AS engine handles the mail. The default value is 5 seconds.

3.  [BUG FIX] 050830189
Symptom: Enable AS ”Discard SMTP mail” and send a mail with attached file will cause the device hangs up
Condition: 
(1) Enable AS ”Discard SMTP mail”
(2) Send a over 20k sized mail
(3) The device hangs up

4. [BUG FIX] 050831205
Symptom: Device will crash if users turn on myZyxelCom debug message then process device registration and trial service activation.
Condition:
(1) Turn on myzyxel.com debug message by "sys myZyxelCom debug type 3" 
(2) Go to eWC>REGISTRATION, register device and activate trial service for Content Filter.
(3) Device will crash.

5. [BUG FIX] 050701018
Symptom: DHCP client gets IP failed
Condition:
(1) Topology: PC---(192.168.1.1) Router switch to: PC---(192.168.70.250) DUT
(2) PC connects to the router LAN port with DHCP, and get an IP.
(3) DUT set a static DHCP rule for the PC.
(4) PC switch to DUT, and gets an IP failed. The user must release IP manually, then PC will get IP successfully.

6.   [BUG FIX]
Symptom: ZyWALL sends [HASH][DELETE] to delete VPN tunnel after output timed-out even they keeps traffic via the tunnel. 
Condition: PC1 -----ZyWALL-------------------------PC2(Zywall VPN client)

(L)   (W)                  |------------PC3(Zywall VPN client) 
(1) Configure a dynamic VPN-rule in the ZyWALL. 
(2) Establish first VPN tunnel by PC2 using ZyWALL VPN client.
(3) Establish Second VPN tunnel by PC3 using ZyWALL VPN client.
(4) Both PC2 and PC3s’ PCs keep ping to PC1. 
(5) ZyWALL sends [HASH][DEL] to 2nd VPN peer only every 2 minutes which is output Idle time-out timer.

7.   [BUG FIX] 050907311
Symptom: Bridge mode VPN can’t work if configure by Wizard.
Condition: 
(1) Configure bridge mode VPN with wizard.
(2) Dial VPN rule and it always fail.

8.  [BUG FIX] 050907308
Symptom: Device will hang forever when editing firewall custom service 
Condition:
(1) Enable firewll and add custom service, service name=test1, IP protocol=TCP/UDP , port range=2222-2223. 
(2) Edit eWC/firewall/rule summary, packet direction=WAN to WAN/ZyWALL, insert service "test1", Action for matched packet=permit. 
(3) Edit eWC/firewall/service and add another custom service, service name=test2, IP protocol=TCP , port range=100-200. 
(4) Edit eWC/firewall/rule summary, packet direction=LAN to WAN, insert service "test2", Action for matched packet=Drop. 
(5) Edit eWC/firewall/service and modify custom service "test2", change IP protocol to UDP then click apply.
(6) Device will hang.

Modifications in V4.00(XD.0) | 09/02/2005

Modify for formal release.

Modifications in V4.00(XD.0)b5| 09/02/2005

1. [BUG FIX]

Device crashed sometimes when doing FTP stress test.

Modifications in V4.00(XD.0)b4| 08/27/2005

1. [BUG FIX] 050819823

Symptom: Device will crash.

Condition:

(1) Enable Anti Spam.

(2) Enable "Discard SMTP mail. Forward POP3 mail with tag in mail subject".

(3) Send a spam mail.

(4) Device will crash.

2. [BUG FIX] 050822932

Symptom: CPU loading will be very heavy.

Condition:

(1) Set two IKE rules which secure gateways are both domain name.

(2) Go to CI command "sys cpu display", CPU loading is 100%.

3. [BUG FIX] 050824993, 050824994, 050824995

Symptom: Sometimes system DNS cannot resolve domain name to IP address.

Condition:

(1) In CLI, enter "ip dns query name myupdate.zywall.zyxel.com"

(2) Try (1) more times and sometimes cannot be resolved.

4. [BUG FIX] 050819842

Symptom: ZyWALL 5 will crash when upload firmware via GUI.

Condition

(1) Upload a very large file via GUI.

(2) Device will crash.

5. [BUG FIX] 050823954

Symptom: The IPSec rule swap without configuring ID Content will fail (XAUTH     
    case).

Condition:

(1) Add one static IPSec rule with XAuth (Rule one).

(2) Add one dynamic IPSec rule with XAuth. Keep the "Peer ID Content" and "Local ID     
Content" unchanged "0.0.0.0" (Rule two).

(3) Dial the VPN tunnel from peer gateway, the device won't swap to rule two, and the 
    connection can not be built up.

6. [BUG FIX] 050822915

Symptom: VPN can not be established if reponder has multiple rules and the correct 
    rule's phase 2 ID type is subnet.

Condition:

Topology: ZyWALL_A(WAN)----(Internet)----(WAN) ZyWALL_B

(1) IPSec policy in ZyWALL_A:

Policy 1:

Local: 192.168.3.10/255.255.255.0

Remote: 192.168.2.7/255.255.255.0

Policy 2:

Local: 192.168.1.10/255.255.255.0

Remote: 192.168.2.6/255.255.255.0

(2) IPSec policy in ZyWALL_B:

Policy 1:

Local: 192.168.2.0/255.255.255.0

Remote: 192.168.1.0/255.255.255.0

(3) The other phase 1 and phase 2 parameters for ZyWALL_A and ZyWALL_B are the 
          same.

(4) Establish policy 1 tunnel from ZyWALL_B.

(5) ZyWALL_A should establish VPN tunnel by using policy 2, but it fails.

7. [ENHANCEMENT]

Add CI command "aux usrmdn [1/0]" to switch USR modem flag. If this flag is on, user 
    can dial USR modem successfully.

Note:

(1) For USR modem, user should disable hardware flow control(initial string is "at&f1"); 
          or the modem speed should be 38400 BPS.

(2) This is a runtime CI command, and this flag is not saved into flash. User can add this 
  command into autoexec.net.

8. [BUG FIX] 050517977

Symptom: IPSec check rule conflict on IP 0.0.0.0 is incorrect.

Condition:

(1) Restore default romfile.

(2) Configure the two IPSec rules shown as follow:

Rule A: local: 0.0.0.0        remote: 192.168.3.33

Rule B: local: 192.168.70.94   remote: 192.168.3.33

These two IPSec rules conflict and we should add check for it.

9. [BUG FIX] 050823946, 050819858, 050820885

Symptom: The UPnP discovery mechanism cannot work normally.

Condition:

(1) Disable the UPnP function.

(2) Reboot device.

(3) Enable the UPnP function.

(4) The XP network place cannot show the UPnP icon.

10. [BUG FIX] 050822912

Symptom: Device crashes when doing VPN stress test.

Condition:

- Create several VPN tunnls and do stress test.

- Device will crash and output the following message on console.
- Prefetch abort exception
  Fault Status = 0xXXXXXXXX
  Fault Addr = 0xXXXXXXXX

Modifications in V4.00(XD.0)b3| 08/17/2005

1.  [BUG FIX] 050727190

Symptom: Spelling invalid in IDP eWC.

Condition:

(1)  In eWC>IDP>Signature, click the "Switch to query view".

(2) The wording of the type selection item "Trojan Hourse" is not right. The word "Hourse" should be "Horse".

2.  [BUG FIX] 050721992

Symptom: Inactivate Wireless without wireless card will cause device hang.

Condition:

(1) Insert wireless card, and enable wireless function.

(2) After taking out B-100 card, upgrade firmware and disable wireless function.

(3) Reboot the device, the device will hang and cannot finish the system booting.

3.  [BUG FIX] 050715808

Symptom: The wireless clients with 802.1x + dynamic WEP cannot ping each other.

Condition:

(1) Setup 802.1x+dynamic WEP environment.

(2) We find that these wireless clients cannot ping each other after rebooting the device.

4.  [ENHANCEMENT]

Make AntiVirus LOG be consistent with IDP LOG in signature Release Date format.

5.  [ENHANCEMENT]

Change the strategy of the search by name to be case-insensitive in eWC->IDP->Signature->Query page.

6.  [FEATURE CHANGE]

Change the wording "WLAN ZONE" to be "WLAN" in the SMT menu 7.1.

7. [BUG FIX] 050727161

Symptom: Output idle timer should not be disabled.

Condition: In eWC->VPN->Global Setting page and SMT 24.8, we should not allow users to set output idle timer = 0.

8. [FEATURE CHANGE]

In SMT 24.1, Wording change: CARD -> WCRD.

9. [BUG FIX] 050728301, 050728302, 050728303

Symptom: Execute SMT 24.1->Press Command->”9-Reset Counters”, device will crash.

Condition:

(1) Insert turbo card.

(2) Execute SMT 24.1->Press Command->”9-Reset Counters” many times, device will crash.

10. [ENHANCEMENT] 050708441, 050708442 and 050712620

(1) In eWC>AV/IDP>Update, avoid a blank web page be displayed.

(2) In eWC>WIRELESS CARD>Wireless Card, remove "Your device must have a wireless card installed…" if the wireless card is installed.

(3) In eWC>AV>General/IDP>General, remove "Your device must have a turbo card installed…" if the turbo card is installed.

11. [BUG FIX] 050616759, 050708438, 050712618

Symptom: System crashes sometimes while signature update or service license refresh.

Condition:

(1) Disconnect WAN interface when you update signature. Hence, the update will fail.

(2) Re-connect the device WAN interface to Internet.

(3) After the update fail, the device will crash sometimes.

12. [ENHANCEMENT] 050808225

Include "WLAN to WLAN" for FireWall hint message.

WAS :In eWC>FireWall>Default Rule page, update message is "Warning:When this box is checked, all LAN to LAN, WAN to WAN, DMZ to DMZ and packets will bypass the Firewall check."

IS   :  In eWC>FireWall>Default Rule page, change message to "Warning: When this box is checked, all LAN to LAN, WAN to WAN, DMZ to DMZ and WLAN to WLAN packets will bypass the Firewall check."

13. [ENHANCEMENT] 050808256

Message in signatue update needs to be update.

WAS :In eWC>IDP>signatue update>waiting page, update message is "This may take up a few seconds. Please wait..."

IS      :In eWC>IDP>signatue update>waiting page, change message to "This may take up to minutes. Please wait..."

14. [ENHANCEMENT]

WAS: In eWC>REGISTRATION>Service page, when service is expired, the Expiration Day field and Registration Type is empty.

IS      : In eWC>REGISTRATION>Service page, when service is expired, the Expiration Day field shows expired date, and Registration Type shows type of expired service.

15. [BUG FIX] 050803125

Symptom: Create two VPN rules which Remote Gateway IP are domain name,the second security gateway can't update automatically.

Condition: PC1 ---- ZW5_1 (wan)----Internet ---- (wan) ZW5_2 ---- PC2

(1) ZW5_1 configuration:

- Set WAN Encapsulation = PPPoE mode.

- Set DDNS & active it.

- Create 2 IKE & 2 ipsec, both security gateway are IP address.

(2) ZW5_2 configuration:

- Set WAN Encapsulation = Ethernet/ Static IP.

- Set DNS server= 168.95.1.1.

- Create 2 IKE & 2 ipsec, both security gateway are domain.

- eWC/ VPN/ Global Setting, Set " Gateway Domain Name

(3) Dial up 2 VPN tunnels.

(4) Drop ZW5_1's PPPoE line then dial up again.

(5) After 2 minutes into ZW5_2's menu 24.8, issue " ipsec ikeL" to check the security gateway IP --> The Second security gateway not update new IP .

16. [ENHANCEMENT]

Add a centralized LOG "Error: download signature file failed." for signature update fail due to not receive complete signature file. This situation most happens when the network connection is not stable so that device can not receive complete signature.

17. [BUG FIX]

Symptom: Mail can not be sent or received when device turn on Anti-Spam.

Condition:

(1) Device turn on Anti-Spam

(2) Generate a lot of mail sessions with a lot of mails from LAN side hosts at the same time.

(3) Mail can not be sent or received in the following conditions:

(3.1) If queued 20k mail can not be sent successfully after query succeed, then that mail will send fail.

(3.2) ACK packets generated by ZyWALL will cause the TCP connection between client and server abnormal.

(3.3) Re-trasmit packets from mail client or server may be dropped by ZyWALL.

18. [ENHANCEMENT]

Imporve Anti-Spam external database query timeout rate by adjusting internal system parameters.

19. [BUG FIX] 050814513

Symptom: System timer will be exhausted when using TfGen to send heavy traffic to LAN interface.

Condition:

(1) Enable AV/IDP feature.

(2) In LAN side PC, Use TfGen to generate heavy traffic to LAN interface. (Heavy traffic : 40000 kbps/sec up.)

(3) In SMT 24.8, type "sys updateServer signatureUpdate", the router will crash.

20. [BUG FIX]

Symptom: Device crashes in Bridge Mode when enable IDP and Content filter

Condition:

(1) Insert Turbo card and restart device to Bridge Mode.

(2) Download signature to device and restart.

(3) In ”eWC->IDP->General”, enable IDP and activate all interface.

(4) In CI command, type

(4.1) idp tune load

(4.2) idp tune con l7Httpasm on

(4.3) idp tune save

(5) In ”eWC->Content Filter->General”, enable content filter.

(6) In ”eWC->Content Filter->Customization”, enable customization and add a forbidden web site  ”www.zyxel.com”.

(7) Access http://www.zyxel.com from a LAN PC.

(8) Device crashes.

21. [FEATURE CHANGE]

Change log behavior when mail session threshold is reached.

WAS: Only generate log when action is DISCARD.

IS: Generate log when action is FORWARD and DISCARD.

Modifications in V4.00(XD.0)b2 | 07/25/2005

1.  [FEATURE CHANGE]

WAS: After deleting the white/black rule via CLI, user needs to type the save command.

IS: After deleting the white/black rule via CLI, user needn't to type the save command.

2.  [BUG FIX] 050614631

Symptom: IP overlapping check function in eWC->ADVANCED->NAT->Address Mapping sometimes will malfunction in some case in NAT address mapping.

Condition:

(1) In eWC->ADVANCED->NAT->Address Mapping->Edit a rule.

(2) Select Type ”Many-To-Many Overload”, set ”Local Start IP” as ”0.0.0.0”, ”Local End IP” as ”1.0.0.5” ”Global Start IP” as ”1.0.0.2”, ”Global End IP” as ”6.0.0.8”.

(3) Click ”Apply”, this rule will be saved, it should not.

3.  [ENHANCEMENT] AS GUI wordings change

In eWC>IDP>Signature>Signature Groups Table, refine "select all", "select partial" and "select none" icons in Active / Log / Alert fields.

4.  [BUG FIX] 050624163

Symptom: Host traffic can't pass through VPN tunnel with dial backup

Condition:   PC1-------ZW5 A------Internet-----ZW5 B-------------PC2 Dial backup

(1) ZW5A add one IKE and one Ipsec rules ,Enable Dial backup

(2) ZW5B add one IKE and one Ipsec rules

(3) Dail from ZW5 A, and make sure VPN tunnel build up

(4) PC1 ping PC2 and PC2 ping PC1 is successful

(5) Pull out ZW5A WAN line ,Dial backup will dial up ,Dail from ZW5 A, and make sure VPN tunnel is rebuild

(6) PC1 ping PC2 is successful, but PC2 ping PC1 is fail

5.  [BUG FIX] 050628469

Symptom: In bridge mode of the multiple-WAN devices, the LAN web site hits of eWC->LOGS->Reports on WAN2 have not any data.

Condition:

(1) In Bridge mode, the WAN 1 is disconnected and WAN 2 is connected.

(2) Enable LOGS->Reports "Collect Statistics" and "Send Raw Traffic Statistics to Syslog Server for Analysis".

(3) A LAN PC uses IE to connect to "www.google.com".

(4) Set "Statistics Report"->"Report type" is Web Site hits, and we cannot find any data.

6.  [BUG FIX] 050701007

Symptom: After displaying the log by CI, you will see the logs related to Anti-spam are broken.

Condition:

(1) Enable Anti-Spam and send a Email(not spam mail) through the ZyWALL.

(2) Use CI->sys logs display to display the logs.

(3) You will see the logs related to Anti-spam are broken like "!               er1@192.168.70.20 Subject:EmailBomb".

7.  [BUG FIX] 050705232

Symptom: In VPN rule name, when users key-in " ' ", GUI will corrupt.

Condition:

(1) In eWC>VPN>VPN Rules(IKE) Summary Table, click "+" to add a gateway policy.

(2) Fill in "Name" field with " ' ".

(3) Key in "Pre-Shared Key" with 12345678 and click "Apply".

(4) The GUI will refresh to VPN Rule(IKE) Summary Table page, but is abnormal.

8.  [BUG FIX] 050628421

Symptom: Device will crash after testing dial backup a period time.

Condition:

(1) Set WAN 1 as PPTP and enable Dial backup and Set Allocated Budget=1 minute, period=1 hour.

(2) Ping 168.95.1.1 with DOS command from LAN site host successfully.

(3) Dial backup will hang up after 1 minute.

(4) Device will crash after pull out WAN and LAN and Dial Backup line for 10 mins.

9.  [BUG FIX] 050707366

Symptom: Device cannot get DHCP IP after WAN IP is released.

Condition:

(1) Device WAN port connects to DHCP server (WAN get DHCP IP).

(2) Use SMT 24.4.2, "WAN DHCP Release" but not use "WAN DHCP Renewal".

(3) LAN side PC ping outside, device cannot renew DHCP automatically.

10. [BUG FIX]

Symptom: Content filter cannot add keyword.

Condition:

(1) Goto GUI->Content Filter->Customization page.

(2) Add Trusted website to its maximum number.

(3) Add Forbidden website to its maximum number.

(4) Keyword cannot be added any more.

11. [BUG FIX] 050707368, 050708419

Symptom: In the eWC->Firewall Rule Summary page, insert a new rule and click "Back" button of IE. Then insert rule again, Firewall will have a null record rule.

Condition:

(1) In eWC>Firewall>Rule Summary page, click "Insert" button, then click IE "Back" button.

(2) Click "Insert" button again, and set one rule then "Apply".

(3) Rule Summary page have an additional null record rule.

12. [BUG FIX] 050708444, 050708443

Symptom: When IDP/AV service expired, the expiration day displayed incorrect format in eWC/AV/Update page.

Condition:

(1) Device IDP/AV service expired.

(2) The expiration day displayed incorrect format in eWC>IDP and AV>Update.

13. [ENHANCEMENT]

Change AV>Update error message.

WAS : In eWC>AV>Update, update message is "The signature search engine is not ready".

IS : In eWC>AV>Update, change message to "Can not find the signature , please update the signature!"

14. [BUG FIX] 050706310

Symptom: Hardware watchdog wake up and sometimes device hand up.

Condition:

(1) In SMT24.8, input ”ip ping 168.95.1.1”

(2) Use Ctrl+C to break it.

(3) Repeat steps 1, 2 fast and you can see the watch dog wake up or device hang.

15. [ENHANCEMENT]

Add firewall predefined services: POP3S/IMAP/IMAPS

16. [BUG FIX] 050627328

Symptom: ZyWALL will log ”SMTP successfully” when SMTP authentication fail. Condition:

(1) In ”eWC->LOGS->Log Settings”, set ”E-mail Log Settings”.

(2) Enable ”SMTP Authentication” and set wrong ”Mail Sender”.

(3) In ”eWC->View Log”, click ”Email Log Now”.

(4) There will have a log ”SMTP successfully”.

(5) Actually, the mail was not sent because SMTP server return a error code (454).

17. [BUG FIX] 050725067

Symptom: Fail in receiving the specific mail when the AV works Condition:

(1) Enable POP3 AV , Enable POP3 Assembly mode

(2) Run the POP3 Based-64 AV test with a lot of mail samples

(3) Some mails couldn't be received

18. [FEATURE CHANGE]

WAS: When Turbo card is not inserted, and accessing IDP at the moment, it shows ”The turbo card is not ready , please insert the card and reboot! ”.

IS:  When Turbo card is not inserted, and accessing IDP at the moment, it shows ”The turbo card is not ready. Please power down the appliance, insert the card and reboot!”.

19. [FEATURE CHANGE]

WAS: Wording "WLAN" in the network status field in SMT menu 24.1 indicates the wireless card status. Wording "ZONE" indicates the WLANZONE channel status.

IS:  "WLAN" -> "CARD, "ZONE" -> "WLAN". So that Wording "CARD" in the network status field in SMT menu 24.1 indicates the wireless card status. Wording "WLAN" indicates the WLANZONE channel status.

20. [FEATURE CHANGE]

When the device sends registration information to MyZyXEL.com server, the router should send 3 digit country number.

21. [BUG FIX] 050713682

Symptom: The router should filter the country code when it is "0".

Condition:

(1) In SMT 24.8, type "sys myZyxelCom register 123456 123456 1234@1.2.3.4 0" (the country code is 0 which is invalid).

(2) It should not be accepted by the router.

22. [BUG FIX] 050712614

Symptom: In eWC>WIRELESS CARD>Wireless card page, the max length of "ESSID" field is too short.

Condition:

In eWC>WIRELESS CARD>Wireless card page, the max length of "ESSID" field is 30 characters, but user can key in 32 characters via SMT.

23. [BUG FIX] 050715784, 050715785, 050715786

Symptom: In eWC->UPnP page, after saving the related items by Firefox will cause device crash sometimes.

Condition:

(1) Open the Firefox browser and goto the eWC->UPnP page.

(2) Disable the UPnP function, and enable some items.

(3) Click the "Apply" button, the device will crash sometimes.

24. [ENHANCEMENT]

Add help pages.

25. [FEATURE CHANGE]

(1) Modify "Update Server" and "myZyXEL.com" logs.

(2) Pop-up new browser in IDP security policy links.

26. [ENHANCEMENT]

Add hyper link to pop up a new window to display certificate error reasons for certificate log message.

27. [ENHANCEMENT]

Unify eWC>Logs datetime format to ISO 8601 (YYYY-MM-DD hh:mm:ss)

28. [ENHANCEMENT]

Update G100/G110 AP F/W version from 1.0.4.3 to 1.2.8.0.

29. [ENHANCEMENT]

Add the Anti-Virus decompress option in eWC>Anti-Virus->General.

30. [BUG FIX] 050715809

Symptom: The device will reboot in bridge mode when setting wireless authentication as 802.1x.

31. [ENHANCEMENT]

In eWC>REGISTRATION>Registration page and eWC>HOME>wizard page, add username field format check for the myzyxel.com registration.

32. [ENHANCEMENT]

(1) Add the available free memory to the eWC->Home->memory

(2) GUI Memory bar will become red when the memory usage percentage is larger than 90%

33. [ENHANCEMENT]

(1) Change signature version format from 001.001 to 1.001 in the eWC->IDP/AV->Update page

(2) After signature updated, GUI shows "Get signature success". It should be "Get signature successfully."

(3) We should provide users hidden CI commands for clearing signature files.

These CI commands are "idp/av clearAllSig".

(4) When the Turbo card is not inserted, in the console: "Current IDP Signatures: N/A" may confuse users. Change to  phrase "Turbo card is not installed" when Turbo card is not installed.

(5) The severity sorting function should perform according to the severity ,not the string case in the eWC->IDP->Signature/Query page

(6) There should be one space after the SID in the IDP log. Was: IDP:10578,Windows Ping  Is:  IDP:10578, Windows Ping

(7) In LOG "Update the signature file successfully", it should be modified as "Signature updat OK - New pattern version: V1.001 Release Date: 2005-06-24".

(8) The "idp/av update display" should be consistent to the eWC->IDP->Update page

34. [BUG FIX] 050714719 ,050714720, 050714735

Symptom: If VPN policy enable NAT Traversal, VPN  tunnel can’t be built up.

Condition: PC1(192.168.33.33)------VPN1(192.168.1.33)--(L)DUT(W)(192.168.12.100)----(192.168.12.101)VPN2--(192.168.2.33)PC2

(1) Edit DUT web eWC/NAT/Port Forwarding, index1/Incoming Port(s)=500-500, index1/Server IP Address=192.168.1.33

(2) Edit VPN1 web eWC/VPN:

- IKE: NAT-T=Enable, Name=IKE1, Remote Gateway Address=192.168.12.101, Pre-Shared Key=12345678, Local ID Content=192.168.1.33, Peer ID Content=192.168.12.101

- IPSec: Active=Yes, Name=IPSec1, Gateway Policy=IKE1, Local Network Starting IP Address=192.168.33.33 Remote Network Starting IP Address=192.168.2.33

(3) Edit VPN2 web eWC/VPN:

- IKE: NAT-T=Enable, Name=IKE1, Remote Gateway Address=192.168.12.100, Pre-Shared Key=12345678, Local ID Content=192.168.12.101, Peer ID Content=192.168.1.33

- IPSec: Active=Yes, Name=IPSec1, Gateway Policy=IKE1, Local Network Starting IP Address=192.168.2.33, Remote Network Starting IP Address=192.168.33.33

(4) To dial up VPN policy, and it will fail.

35. [ENHANCEMENT]

(1) In eWC>AV/IDP>General, add some warning messages if turbo card is not inserted but AV/IDP is activated. The behavior is similar with WLAN.

(2) When Turbo card is not inserted, in eWC>IDP/AV>Update>Current IDP Signatures will display "Turbo card is not installed".

(3) eWC> MAINTENANCE> Backup&Restore changes to eWC> MAINTENANCE>  Backup & Restore.

36. [ENHANCEMENT]

Add centralized logs for signature updating events and errors.

37. [ENHANCEMENT]

Add a centralized log when WAN ping check fails.

38. [FEATURE CHANGE]

Change signature numbers displayed in "eWC->IDP->Signature" page.

39. [ENHANCEMENT]

Display IDP action in centralized log.

40. [BUG FIX] 050715787, 050715788, 050715789.

Symptom: In eWC "HOME" page , "System Time" display error.

Condition:

(1) Go to eWC>HOME Page.

(2) "System Time" display error, the field length is too short.

41. [BUG FIX] 050719921

Symptom: Mail can't be received via POP3.

Condition: Topology:

PC ----- ZyWALL ----- Mail Server

1. Enable Anti-Spam.

2. PC receives mails from Mail Server.

3. PC sometimes can't receive mail and mail client will timeout.

42. [ENHANCEMENT] 050708486, 050712606, 050719906, 050707395, 050712605

Add protection to avoid setting unsupported security in "eWC->Wireless Card" when inserted wireless card is B100. Note: B100 does not support WPA, WPA-PSK, 802.1x + Dynamic WEP.

43. [ENHANCEMENT]  Wording

WAS: The device will now reboot. As there will be no indication of when the process is complete, please wait for one minute before attempting to access the router again

IS: The system will now reboot. As there will be no indication of when the process is complete, please wait for one minute before attempting to access the system again.

44. [FEATURE CHANGE]  Update registration message

WAS:

(1) When user upgrade IDP/AV/AS services, the LOGS shows "service upgrade successfully" but users can not know which service is upgraded"

(2) When user activate trial service(s), the LOGS shows "trial service activation successfully" but users can not know which service is activated. IS:

(1) When user upgrade services, the LOGS will show

"Content Filter service upgrade successfully" or

"IDP/Anti-Virus service upgrade successfully" or

"Anti-Spam service upgrade successfully" depends on which service license key is used.

(2) When user activate trial service(s), the LOGS shows which trial service is activated. Ex. "Content Filter, IDP/Anti-Virus trial service(s) activation successfully"

Modifications in V4.00(XD.0)b1 | 07/01/2005

1.  [ENHANCEMENT]

Change the input format of trap destination in eWC->Remote Management->SNMP  rom text to IP format.

2.  [ENHANCEMENT]

Support small font size on ZyWALL GUI.

3.  [ENHANCEMENT]

Replace the Cerberian logo by Blue Coat in Content Filter blocked page.

4.  [ENHANCEMENT]

Support Turbo Card (external IDP/AV signature search accelerator)

5.  [ENHANCEMENT]

Add ARP probe for DHCP server.

(1) Change probe type by CI command "sys probeType [icmp | arp]".

(2) Default type is "ICMP".

(3) ARP probe only works when you use arp probe type and dhcp mode should be "Server".

(4) This value will be saved in ROM.

6.  [FEATURE CHANGE]

Add ALG configuration in navigation panel.

7.  [ENHANCEMENT]

Re-layout ZyWALL navigation panel on GUI.

8.  [ENHANCEMENT]

Add "Service Status" and "Expiration Date" in Content Filter GUI. The modified GUIs are:

eWC>CONTENT FILTER>Categories

9.  [ENHANCEMENT]

Add a sender email field in "E-mail Log Settings".

10. [ENHANCEMENT]

When the daylight saving is activated, there should be a "DST" string trailed behind the time in eWC.

11. [ENHANCEMENT]

WAS: DNS domain name is not case insensitive.

IS: DNS domain name is case insensitive.

12. [ENHANCEMENT]

Firewall ”Available Services” add some common services which are

(1) Microsoft RDP (remote desktop protocol) - tcp:3389

(2) VNC (virtual network computer) - tcp:5900

(3) NTP - tcp/udp:123

13. [ENHANCEMENT]

Consolidate log “Under SYN flood attack, sent TCP RST”

14. [ENHANCEMENT]

(1) Users cannot enter characters into eWC>VPN>GATEWAY POLICY >EDIT>SA Life Time (Seconds)

(2) User cannot enter characters in eWC>VPN>NETWORK POLICY >EDIT>Protocol

(3) Users cannot enter characters into eWC>VPN>NETWORK POLICY >EDIT>SA Life Time (Seconds)

15. [ENCHANCEMENT]

Add IDP,Anti-Virus and Anti-Spam features.

16. [ENHANCEMENT]

Add the SMTP server to the log entry.

17. [ENHANCEMENT]

Add sequence number and SPI in log for ESP / AH packets.

18. [ENHANCEMENT]

DHCP log shows the hostname.

19. [ENHANCEMENT]

Add VPN over Bridge feature.

20. [ENCHANCEMENT]

Add MyZyxel.Com and Registration features.

21. [ENHANCEMENT]

Add Firewall Custom Service enhancements.

Modifications are listed below:

(1) Allow user to configure ICMP type and code in Firewall ACL.

(2) Allow user to configure IP protocol in Firewall ACL.

(3) Add "Any IP Protocol" in default service.(GUI only)

(4) Replace "PING" with "Any ICMP" in default service. (GUI only)

(5) Allow user to configure Firewall rule name.

(6) Firewall (defeault/rule) action supports "permit", "drop" and "reject".

(7) Centralized LOGS shows descriptions for matched ICMP packet instead of displaying type/code value only.

22. [ENHANCEMENT]

Enhance Firewall Custom Service

(1) In eWC>Firewall>add new page "Service",  it displays the summary of custom services and predefined services.

(2) In eWC>Firewall>Service>Firewall Service Edit page, add two new options: IP protocol and ICMP.

23. [ENHANCEMENT]

On eWC>FIREWALL>Threshold, add a GUI option to enable/disable DoS Attack protection.

24. [ENHANCEMENT]

Each static route entry should have its own "Modify" and "Delete" icons.

25. [ENHANCEMENT]

Add dial backup support for CI command.

The following is the original SPR description.

Enhance SMT “sys rn accessblock 0 ” debug message.

(1) CI “sys rn load 3”

(2) CI “sys rn accessblock 0”

(3) CI “sys rn save”

(4) And SMT will occur message “[-6103] Bad entry number”

26. [ENHANCEMENT]

Enhance Firewall Edit GUI to make it more user-friendly.

Before: When users click Add/Modify/Delete button to configure an address or select a service from Available Service to Selected Service, the page will be submitted to the ZyWALL immediately to have a rule check and then refresh. It consumed too much time on editing a firewall rule for a user.

Now: When users click Add/Modify/Delete or select a service, the page will not be submitted to the ZyWALL immediately. The page will be submitted to the ZyWALL to have a rule check after users click "Apply" button. It reduces the refresh time and it is more convenient for the users.

27. [ENCHANCEMENT]

(1) Enhance WLAN to be an independent interface so that traffic passes through WLAN can be handled by firewall.

(2) WLAN can be bound to LAN or DMZ for user’s chosen.

(3) DHCP sever can be applied on LAN, DMZ and WLAN.

28. [ENHANCEMENT]

In order to solve ZW5 available memory is not enough for 4.00, allocate a share memory for signature download and firmware upload.

29. [ENHANCEMENT]

Add DDNS as My Address in VPN IKE rule. (GUI)

30. [ENHANCEMENT]

Add ping check switch for single WAN products.

CI command: sys rn pingcheck [0:disable|1:enable]

Note: This will not be saved in romfile.

Modifications in V3.64(XD.3) | 06/21/2005

Modify for formal release.

Modifications in V3.64(XD.3)b1 | 06/16/2005

1. [BUG FIX]

Symptom: Router crash.

Condition:

(1) Use router for a long time.

(2) Sometimes Router will crash and the console shows

"Common TOS: Free queue session number > max session number..

\tos.c:960 sysreset()".

2. [ENHANCEMENT] 050418857

DNS domain name should be case insensitive.

3. [BUG FIX]

Symptom: IPSec check rule conflict on IP 0.0.0.0 is incorrect.

Condition:

1. Restore default romfile.

2. Configure the two IPSec rules shown as follow:

Rule A: local:0.0.0.0       	     remote:192.168.3.33

Rule B: local:192.168.70.94     remote:192.168.3.33

these two IPSec rules conflict and we should add check for it.

4.  [BUG FIX] 050526694

Symptom: IPSec input idle timer does not work correctly.

Condition:

Topology:

PC1-ZWA--Intranet--ZWB-PC2

Add normal VPN rule in both side.

(1) In ZWB, set "Input Idle Timeout" as "30" seconds.

(2) Dial the tunnel up, there is no traffic in the tunnel.

(3) In ZWB, SMT 24.8, type "ipsec sho sa", the "input idle count" in "INBOUND"

will be decreasing, it works correctly.

(4) Now, In PC1, ping PC2 from PC1 with one packet then stop the traffic in the

tunnel.

(5) In ZWB, SMT 24.8, type "ipsec sho sa", the "input idle count" in "INBOUND"

stay unchanged.

(6) The input idle timeout mechanism will not work anymore.

5. [BUG FIX]

Symptom: Output idle timer doesn’t work correctly.

Condition:

PC1--(L)ZW5(W)--Intranet--(W)Router(L)--PC2

(1) ZW5 and Router had established VPN tunnel.

(2) Output idle timer=120 secs, input idle timer=30 secs.

(3) Unplug the WAN link of Router, make a ICMP echo request to PC2 from PC1.

(4) ZW5 doesn’t send out "are u there" packets to peer gateway after 120 seconds.

6. [BUG FIX] 050613568

Symptom: There is no conflict check between VPN dynamic rule and static rule on local

ip address.

Condition:

1. Goto CUI VPN page, add one dynmaic IKE rule and static IKE rule.

2. Add one policy with local ip set as 192.168.1.0/24 into dynamic rule.

3. Add one policy with local ip set as 192.168.1.1/32 into static rule.

4. The static rule’s policy can be saved without conflict error.

7. [BUG FIX] 050615688

Symptom: The Log Consolidation Period can not configure properly.

Condition:

1. Goto eWC->LOGS->Log Settings page, input the vaule, 300, into

”Log Consolidation Period” field, then apply the setting.

2. Refresh the Log Settings page, the value in ”Log Consolidation Period” field show

as 44.

Modifications in V3.64(XD.2) | 06/10/2005

Modify for formal release.

Modifications in V 3.64(XD.2)b3 | 05/31/2005

1. [BUG FIX] 050527748

Symptom: DNS of Dail backup had some problem if WAN's Encapsulation

= PPTP mode.

Condition:

1. Restore default Rom.

2. WAN is configured as PPTP, and WAN is connected.

3. Configure Dial backup.

4. Unplug the WAN, and WAN is disconnected, and Dial backup is

connected.

5. eWC/DNS/System, DNS server keep old DNS IP ( Assigned from

PPTP server) .

Modifications in V 3.64(XD.2)b2 | 05/25/2005

1.  [BUG FIX]  050414592

Symptom: Dynamic rule with more than two initiators has problem.

Condition:

1. ZyWALL 5 as responder has one dynamic rule and use XAUTH.

2. Two initiators (two devices or two vpn clients..).

3. Dial one of them, the packets can be transmitted through the tunnel correctly.

4. Dial the second, only one of them can work correctly.

2.  [BUG FIX]

Symptom: Trigger dial fail in dial backup.

Condition:

1. Restore default rom file.

2. Setup dial backup account and phone number, make sure it can work.

3. Put a PC in router's LAN and ping 168.95.1.1 continually.

4. Unplug modem's phone line and wait for 5 mins.

5. Plug it and router will not dial from modem automatically.

3.  [BUG FIX]

Symptom: Dial back-up does not support FULL-FEATURE NAT.

Condition:

1. Enter SMT menu 11.3 for ”dial backup” remote node

2. Go to ”Edit IP” and change NAT selection to FULL Feature. (will see the NAT

Lookup Set= 3)

3. Go to SMT menu 15.1 and found there is no NAT_SET 3.

4.  [BUG FIX]  050502014

Symptom: VPN tunnel can't be up with dynamic rule.

Condition:

Initiator: One IKE with one policy. And in policy, local ID type = Subnet. Dest ID

type = Subnet.

Responder: One dynamic IKE with two policies:

(1) Policy 1: Encryption is wrong. Local ID type = Subnet. Local starting IP Address

is wrong.

(2) Policy 2: All settigs are correct.

5.  [BUG FIX]  050502014

Symptom: Modification to exisiting WANtoWAN rule (with IKE and BOOTP) can not

work

Condition:

In the example, use SSH

1) Change SSH port to 2222 in Remote MGMT.

2) Go to WAN to WAN / ZyWALL and create a custom service, TCP/UP 2222.

3) Add the rule in the default rule that has IKE and Bootp.

4) Try to connect with Putty or other prefered SSH client. It doesn’t work

5) Now add the standard SSH (or any other predefined TCP rule) service to the same

firewall rule. It works.

6.  [BUG FIX]  050311653

Symptom: DNS cannot work after switching WAN and Dial backup.

Condition:

1. Restore default romfile.

2. WAN is configured as PPTP, and nail-up, and WAN is connected.

3. Configure Dial backup, and is always-on.

4. Unplug the WAN, and WAN is disconnected, and Dial backup is connected.

5. Plug in the WAN line again, and PPTP is connected, get an IP.

6. Go to eWC->DNS->DHCP page, DNS from ISP is none; if PC DNS is

ZyWALL, it cannot browse to the internet.

7.  [BUG FIX] 050502038

Symptom: Daylight Saving problem: Current Time is faster 2 hours than Taiwan

during daylight saving.

Condition:

1. Restore default romfile.

2. Go to eWC->Maintenance->TimeAndDate.

and the problem happened only when

3. Apply the "Time Zone" = "(GMT+08:00)", activate "Enable Daylight Saving"

and set the date range include the  current time.

4. Click the "Apply" button and the page will be refreshed.

5. The current time is faster 2 hours than Taiwan, it should be faster 1

hour only.

8. [BUG FIX]

Symptom: Router crash.

Condition:

(1) Turn on firewall.

(2) Sometimes router will crash when suffer attack.

9. [BUG FIX]

Symptom: Dial backup will be triggered abnormally.

Condition:

(1) Configure a Dial Backup.

(2) Close ping check flag by “sys rn pingcheck 0”.

(3) WAN is ethernet, gets an IP, and cannot access Gateway.

(4) Dial backup will be triggered, it is not right.

10. [BUG FIX]

Symptom: Dial backup will be triggered abnormally.

Condition:

(1) Configure a Dial Backup.

(2) WAN is ethernet.

(3) Reset the router, Wan gets an IP but dial back-up still will be triggered.

11. [FEATURE CHANGE]

When edit a firewall rule, the source IP and destination IP rule numbers are limited to

20.

12. [FEATURE CHANGE]

At the beginning of router restart, the pingcheck is disabled.

Modifications in V 3.64(XD.2)b1 | 05/18/2005

1.  [BUG FIX]  050414592

Symptom: Dynamic rule with more than two initiators has problem.

Condition:

1. ZyWALL 5 as responder has one dynamic rule and use XAUTH.

2. Two initiators (two devices or two vpn clients..).

3. Dial one of them, the packets can be transmitted through the tunnel correctly.

4. Dial the second, only one of them can work correctly.

2.  [BUG FIX]

Symptom: Trigger dial fail in dial backup.

Condition:

1. Restore default rom file.

2. Setup dial backup account and phone number, make sure it can work.

3. Put a PC in router's LAN and ping 168.95.1.1 continually.

4. Unplug modem's phone line and wait for 5 mins.

5. Plug it and router will not dial from modem automatically.

3.  [BUG FIX]

Symptom: Dial back-up does not support FULL-FEATURE NAT.

Condition:

1. Enter SMT menu 11.3 for ”dial backup” remote node

2. Go to ”Edit IP” and change NAT selection to FULL Feature. (will see the NAT

Lookup Set= 3)

3. Go to SMT menu 15.1 and found there is no NAT_SET 3.

4.  [BUG FIX]  050502014

Symptom: VPN tunnel can't be up with dynamic rule.

Condition:

Initiator: One IKE with one policy. And in policy, local ID type = Subnet. Dest ID

type = Subnet.

Responder: One dynamic IKE with two policies:

(1) Policy 1: Encryption is wrong. Local ID type = Subnet. Local starting IP Address

is wrong.

(2) Policy 2: All settigs are correct.

5.  [BUG FIX]  050502014

Symptom: Modification to exisiting WANtoWAN rule (with IKE and BOOTP) can not

work

Condition:

In the example, use SSH

1) Change SSH port to 2222 in Remote MGMT.

2) Go to WAN to WAN / ZyWALL and create a custom service, TCP/UP 2222.

3) Add the rule in the default rule that has IKE and Bootp.

4) Try to connect with Putty or other prefered SSH client. It doesn’t work

5) Now add the standard SSH (or any other predefined TCP rule) service to the same

firewall rule. It works.

6.  [BUG FIX]  050311653

Symptom: DNS cannot work after switching WAN and Dial backup.

Condition:

1. Restore default romfile.

2. WAN is configured as PPTP, and nail-up, and WAN is connected.

3. Configure Dial backup, and is always-on.

4. Unplug the WAN, and WAN is disconnected, and Dial backup is connected.

5. Plug in the WAN line again, and PPTP is connected, get an IP.

6. Go to eWC->DNS->DHCP page, DNS from ISP is none; if PC DNS is

ZyWALL, it cannot browse to the internet.

7.  [BUG FIX] 050502038

Symptom: Daylight Saving problem: Current Time is faster 2 hours than Taiwan

during daylight saving.

Condition:

1. Restore default romfile.

2. Go to eWC->Maintenance->TimeAndDate.

and the problem happened only when

3. Apply the "Time Zone" = "(GMT+08:00)", activate "Enable Daylight Saving"

and set the date range include the  current time.

4. Click the "Apply" button and the page will be refreshed.

5. The current time is faster 2 hours than Taiwan, it should be faster 1

hour only.

8. [BUG FIX]

Symptom: Router crash.

Condition:

(1) Turn on firewall.

(2) Sometimes router will crash when suffer attack.

9. [FEATURE CHANGE]

When edit a firewall rule, the source IP and destination IP rule numbers are limited to

20.

10. [FEATURE CHANGE]

At the beginning of router restart, the pingcheck is disabled.

Modifications in V3.64(XD.1) | 05/03/2005

Modify for formal release.

Modifications in V 3.64(XD.1)b2 | 04/27/2005

1.    [BUG FIX] 050201039
Symptom: "Gateway Domain Name Update Timer" in eWC --> VPN --> Global

Setting didn't work.
Condition:
    (1) Set one IKE rule which secured gateway address is domain name. 
    (2) Set "Gateway Domain Name Update Timer" to 15 minutes and apply.
    (3) System will not update secured gateway domain name according to the setting

unless system reboot.

2.    [BUG FIX]

# Symptom: LAN & WAN deathed when recieve UDP packets which comes from

# TfGen.
Condition:
    (1) Restore default rom file.
    (2) In WAN side, place a PC and open TfGen tool to send packets to router's WAN.
    (3) The TfGen's setting in my PC is: Ultilization: 4kbps, Destion: “DUT’s WAN

# IP”, Port: 500.

(4) After a period time, DUT’s LAN & WAN both deathed that all traffic can’t

go out.

3.    [BUG FIX] 050203206

Symptom: In bridge mode, after device synchronized the defined NTP server, the

result displayed failed.
Condition:
    (1) PC(192.168.1.33) --- DUT(192.168.1.254) --- NAT(192.168.12.106) ---

Internet.
(2) In eWC/Maintenance/Time and Date, get from Time Server: Time

Protocol=NTP (RFC 1305), Time Server Address= a.ntp.alphazed.net, then

clicked "Synchronize Now" button.
(3) The result displayed failed. ("System Time and Date Synchronization Fail")
(4) However, a successful log showed in eWC/LOGS.
(5) Actually, the device was successful to synchronize the defined NTP server.

Modifications in V 3.64(XD.1)b1 | 04/22/2005

- [ENHANCEMENT]
Enlarge content filter web site, forbidden key word and trusted website size to 100.

- [ENHANCEMENT]
Add sequence number and SPI in log for ESP / AH packets

- [ENHANCEMENT] 
Change DNS Address Record size from 8 to 30

- [EHNANCEMENT] 050419889
Add IP information for my IP address and Secure Gateway address. In CI command, "ipsec ikeDisp #" will show IKE rule configuration. When my IP address or secure gateway address is domain name, the resolved IP will show after domain name.

- [BUG FIX] 050128770
Symptom: When users remotely manage the ZyWALL via a PPTP connection, a strange firewall session (between PPTP server and PPTP client) timeout log may be observed.
Condition: 
(1) Configure the ZyWALL's WAN port to use PPTP encapsulation.
(2) Remotely login eWC (http/https) via the PPTP connection.
(3) After a few minutes, check the centralized logs or syslogs, you will observe a sequence of firewall logs of http/https session timeout.

- [BUG FIX] 040507153
Symptom: Telnet function takes too much time.
Condition:
(1) Type the CI command "ip telnet host_A".
(2) When telnet from router to non-exist server host_A, it always takes about 40 
seconds or more to connect. And users cannot interrupt the router and can do nothing.

- [BUG FIX] 050420986
Symptom: P2000W and P2000W can not talk to each other in P2P mode.
Condition:
(1) Topology:
P2000W----DUT---Internat---DUT---P2000W
(2) P2000W and P2000W can not talk to each other in P2P mode.

- [BUG FIX] 050217478
Symptom: Netbios packet cannot pass through VPN tunnel .
Condition:
(1) Configure a VPN tunnel as follows:
1.1 local subnet mask is 192.168.1.1/255.255.0.0.
1.2 remote subnet mask is 192.169.1.1/255.255.0.0.
1.3 Enable ”Netbois pass through” in local and remote gateway.
1.4 PC A(Local)------ZyWALLA------ZyWALLB---PC  B(Remote)192.168.1.1/24 192.169.1.1/24
(2) Establish the VPN tunnel.
(3) In PC A, Search PC B’s computer name.
(4) PC A will send a broadcast packet to search PC B.
(5) ZyWALL A will change destination IP address from 192.168.255.255 to 192.169.255.255 and send to ZyWALL B after encryption. Horever, ZyWALL A should adjust the UDP checksum but it didn’t.
(6) PCB will drop the received broadcast UDP pcket from PC A due to error UDP checksum.

- [BUG FIX] 050214274
Symptom: VPN My IP Addr will resolving fail 
Condition: 
(1) Add a VPN rule and My IP Address and Remote Gateway Address are domain type.
(2) Click Dial button, it will fail to build tunnel first time (second time is ok)
(3) Check log will display “Cannot resolve My IP Addr for rule xxx”

- [BUG FIX] 050304284
Symptom: There is no log for replay packets 
Condition:
(1) Enable ”Anti-Replay” function.
(2) Sniffer an ESP packet and replay it.
(3) This ESP packet will be dropped by there is no log.
(4) There should be log to show this action.

- [BUG FIX] 050316859
Symptom: ZyWALL (3.64) crashes while remote VPN software (ZyWALL VPN Client) make a VPN connection 
Condition:
(1) ZyWALL start negotiating with remote VPN software.
(2) The remote VPN software sends too long VID size.
(3)device will crash.

- [BUG FIX] 050221575
Symptom: Max. Concurrent Sessions Per Host problem.
Condition: 
(1) In eWC->NAT , change Max. Concurrent Sessions Per Host to 300
(2) Use ipscan tool to make session
(3) Log show “192.168.1.33 exceeds the max. number of session per host! “ when exceeds 	the max. number of session per host, but Max. Concurrent Sessions Per Host (Historical high since last startup: 286), it’s not reach 300.

- [BUG FIX] 050407161
Symptom: PC cannot ping remote secure gateway's LAN IP via VPN tunnel
Condition:
PC A (1.33) – (1.1)ZW5 --- LAB ---- ZW70 (2.1) ----(2.33) PC B
(1) Add a VPN rule(ZW5), and in IPsec rule Local Network select Subnet Address, Starting IP is 192.168.1.0 / 255.255.255.0. Remote Network select Subnet Address Starting IP is 192.168.2.0 / 255.255.255.0.
(2) ZW70 had opposite setting.
(3) Build up this tunnel, PC A can ping PC B, but PC A can’t ping 192.168.2.1(ZW70 gateway LAN IP)

- [BUG FIX] 050302166
Symptom: Remote gateway Address can’t configure as domain type when ipsec Nail-Up option is on.
Condition: 
(1) Add a VPN rule(Static rule) with Remote gateway Address set as domain type.
(2) In Ipsec rule, enable Nail-Up option.
(3) Return to IKE rule page, change some fields and click Apply. The Status will show “This ipsec rule bounds to dynamic IKE rule. Please inactive nail up.” and it can’t be saved.

- [BUG FIX] 050309435
Symptom: Router crash when recieve UDP packets which comes from TfGen.
Condition:
(1) Restore default rom file.
(2) In WAN side, place a PC and open TfGen tool to send packets to router's WAN.
(3) The TfGen's setting in my PC is: Ultilization: 4kbps, Destion: 192.168.70.34, Port: 500.

- [BUG FIX] 050214258
Symptom: DNS inverse query causes system crash.
Condition:
(1) Set A PC on the device LAN site.
(2) The DNS server of the PC sets to the device.
(3) The PC sends DNS inverse query continually, the device will crash sometimes.

- [BUG FIX] 050204235
Symptom: Responder receive duplicate package when VPN tunnel established 
Condition:
(1) At Initiator edit one VPN rule and Extended Authentication=enable=client mode
(2) At responder edit one VPN rule and Extended Authentication=enable=server mode
(3) when VPN tunnel established ,Responder log show “Rule[IKE1] receives duplicate packet”

- [BUG FIX] 050412413
Symptom: There is no “Ping of Dead” log message when performing “Consolidate every 10 seconds（Attack: ping of death）” 
Condition: 
(1) Dos command ”ping 192.168.1.1 -l 2000”
(2) User can not see ”ping of death” consolidation log on eWC/LOGS page
(3) Bridge mode only.

- [BUG FIX] 050303203
Symptom: DNS inverse query causes memory leak.
Condition:
(1) Set A PC on the ZyWALL LAN site.
(2) The DNS server of the PC sets to the ZyWALL.
(3) The PC sends DNS inverse query continually (ex: 140.113.23.1), the system will generate memory leak.

- [BUG FIX] 050201041
Symptom: "Gateway Domain Name Update Timer" in eWC --> VPN --> Global Setting didn't work.
Condition:
(1) Set one IKE rule which secured gateway address is domain name. 
(2) Set "Gateway Domain Name Update Timer" to 15 minutes and apply.
(3) System will not update secured gateway domain name according to the setting unless system reboot.

- [BUG FIX] 050415693
Symptom: Resolving a domain name which start with number (for example 4youcard.com) will fail. 
Condition: CI command "ip ping 4youcard.com" and it will fail.

- [BUG FIX] 050406055
Symptom: ZyWALL VPN traffic will lose from time to time 
Condition:
(1) To create tunnel from zw5 to peer.
(2) To ping the LAN PC of peer VPN gateway fom the LAN PC of zw5 via the tunnel.
(3) About 1 min, it will re-key again.
(4) The tunnel loses packet.

- [BUG FIX] 041201001
Symptom: Router will crash when receive an unrecognizable DNS response
Condition:
Environment:
PC(192.168.1.33)-----(192.168.1.1)ZW5---Internet
(1) Set ZW5's system DNS server as "164.67.128.1"
(2) From PC, send a DNS query to ZW5. The DNS format is as following:
cf 07 01 00 00 01 00 00 00 00 00 00 04 75 63 6c 
61 03 65 64 75 00 00 ff 00 01
(3) ZW5 will relay the DNS query to "164.67.128.1".
(4) ZW5 will crash after receive DNS response from "164.67.128.1"

- [BUG FIX] 050311685
Symptom: Firewall WAN to DMZ Reject can’t work.
Condition: PC A ---- (W)ZW5 (DMZ) 10.1.1.1 --- 10.1.1.100 ZW10W
(1) In eWC Firewall Default Action WAN to DMZ select Reject. And enable Log
(2) One ZW10W connect to ZW5 DMZ port and IP is 10.1.1.100
(3) Add default server 10.1.1.100.
(4) PC A also can ftp to DMZ ZW10W. 
(5) Check Picture [ZW5]Firewall W2D item 3->1

- [BUG FIX] 050420986
Symptom: External content filter cannot work.
Condition
(1) Enable external content filter.
(2) Use external content filter for a long time.
(3) System cannot create socket anymore and external content filter cannot work. 
(4) Use CI command "ip ping 168.95.1.1", there will be a message "Can't create socket' in console.
(5) You can see there are many used sockets via CI command "sys socket".

26.   [BUG FIX] 050201045

Symptom: For firewall ACL schedule, if two rules have the same policies

except "schedule", only the first rule will work.

Condition:

1. Set two firewall rules have same policies except schedule.

2. Only the first rule will work.

27.   [BUG FIX] 050301081

Symptom: Subclass(FTP service) can not borrow all rest of parent bandwidth in

priority-base.

Condition:

1.Root bandwidth is 1000kbps

2.Add a FTP service subclass which bandwidth is 100kbps and can borrow from

parent.

3.Add a Custom service subclass which bandwidth is 100kbps and can borrow from

parent

4.Execute FTP，but FTP service bandwidth can not borrow all rest of parent

bandwidth

5.Send lots of UDP packet，but Custom service bandwidth can not borrow all rest

of parent bandwidth. Sometimes all traffic can not pass through DUT.

28.  [BUG FIX] 050128718

Symptom: The VT6105 Ethernet port may fail to receive any packet.

Condition:

1. Connect ZyWALL5's LAN port (using VT6105 Ethernet chip) to

an SMC hub and operate it in 100M/HALF mode.

2. Generate heavy traffic to go through the ZyWALL 5's LAN port.

3. After an indefinite period of time, the ZyWALL 5's LAN port may fail to

receive any packet. When this hang condition happens, the console will show

"enet0 stop NIC Rx never completed!"

29.  [BUG FIX]

Symptom: DDNS failed to update when PPPoE redial.

Condition:

1. Configure the DDNS host and enable it.

2. Configure WAN as PPPoE mode and idle timeout, and connected OK.

3. When the connection is doown, and connected again, IP is change, it failed to

update DDNS server.

Modifications in V3.64(XD.0) | 03/04/2005

Modify for formal release.

Modifications in V3.64(XD.0)b4 | 02/23/2005

1.     [BUG FIX]
Symptom: In PPPoE/PPTP mode, BWM can not classify the traffic of FTP, H323,     SIP.

2.     [BUG FIX] 
Symptom: Bandwidth Management, Priority based, FTP transfer speed slow down until to disconnect .
Condition:

(1) Edit web eWC/BW MGMT，WAN/Active=enable, WAN1/Speed (kbps)=1000, 
      Scheduler=Priority-Based
(2) Edit web eWC/BW MGMT/Class Setup, Interface=WAN1, Add Sub-Class, 
      Class Name=FTP, Bandwidth Budget=200,
      Priority=3, Borrow bandwidth from parent class=enable，Enable Bandwidth 
      Filter=enable, Service=FTP, Destination IP Address =192.168.10.0, Destination 
      Subnet Mask=255.255.255.0
(3) FTP upload file from LAN to WAN

3.     [BUG FIX]
Symptom: Custom traffic will send over 100 kbps in bridge mode.
Condition: 
(1) In bridge mode, set WAN as 1000 kbps with fairness mode.
(2) Create a custom class, budget=50, priority=2, no borrow.
(3) Create a ftp class, budget=200, priority=3, no borrow.
(4) Use tfgen to generate UDP traffic to match custom class.
(5) Use ftp to generate TCP traffic to match ftp class.
(6) In GUI statistics page, custom class will be over 100 kbps.

4.     [BUG FIX]
Symptom: VPN XAuth rule swap fail 
Condition:
DUT1:
(1) Edit web eWC/VPN, add gateway policy, Name=IKE1, Remote Gateway 
      Address=192.168.11.101,  Pre-Shared Key=12345678, Enable Extended 
      Authentication=enable, Client Mode/User Name=dut1, Client 
      Mode/Password=dut1
(2) Edit web eWC/VPN, add network policy for IKE1, Active=enable,
      Name=IPSec1, Local Network/Starting IP Address=192.168.1.33, Remote 
      Network/Starting IP Address=192.168.2.33
DUT2:
(1) Edit web eWC/AUTH SERVER/Local User Database, index1/Active=enable
(2) Edit web eWC/VPN, add gateway policy, Name=IKE1, Remote Gateway 
      Address=192.168.12.100, Pre-Shared Key=12345678
(3) Edit web eWC/VPN, add gateway policy, Name=IKE2, Remote Gateway 
      Address=0.0.0.0, Pre-Shared Key=12345678, Enable Extended 
      Authentication=enable, Client Mode/User Name=dut1, Client 
      Mode/Password=dut1
(4) Edit web eWC/VPN，add gateway policy, Name=IKE3, Remote Gateway 
      Address=0.0.0.0, Pre-Shared Key=12345678, Enable Extended 
      Authentication=enable, Server Mode=enable
(5) Edit web eWC/VPN, add network policy for IKE1, Active=enable,
      Name=IPSec1, Local Network/Starting IP Address=192.168.2.43, Remote 
      Network/Starting IP Address=192.168.1.33
(6) Edit web eWC/VPN，add network policy for IKE2, Active=enable,
      Name=IPSec2, Local Network/Starting IP Address=192.168.2.53
(7) Edit web eWC/VPN，add network policy for IKE3, Active=enable,
      Name=IPSec3, Local Network/Starting IP Address=192.168.2.33

5.     [BUG FIX]
Symptom: In eWC->Wireless, When select WPA or WPA PSK, the Authentication Databases field always says: Local User first then RADIUS. 
Condition: Go to eWC>WLAN>Wireless, when select WPA or WPA PSK, 
the Authentication Databases field always says: "Local User first then RADIUS". 
But it shouldn't.
(1) When selecting "WPA", we should show "Authentication Database = RADIUS" 
      instead of "Authentication Databases Local User first then RADIUS" 
(2) When selecting "WPA+PSK", "Authentication Databases" should be hidden.

Modifications in V3.64(XD.0)b3 | 02/03/2005

1.     [BUG FIX]
Symptom: OpenPhone H.323 traffic will be blocked by Firewall if connection is initiated from WAN side to LAN side.
Condition:
PC1(OpenPhone)--------(LAN) ZyWALL (WAN) --------- PC2(OpenPhone)

192.168.1.33
        (1) Enable Firewall, setup a WAN2LAN firewall rule for H.323 service
        (2) Enable NAT port forwarding for port 1720(H.323) to PC 192.168.1.33 
        (3) Enable H.323 ALG by "ip alg enable ALG_H323"
        (4) PC1 and PC2 use OpenPhone, PC2 call PC1.
        (5) OpenPhone application traffic will be blocked by Firewall, you will see a lot of 
              Firewall blocked log in Centralized LOG.

2.     [BUG FIX]
Symptom: DPD vendor ID is not correct.
Condition: VID value of DPD is not compatible with RFC3706.

3.    [FEATURE CHANGE]
WAS: The second datagram will use the last 8 octets of the first datagram as IV. This may cause IV "predictable". 
IS: All datagrams will use random IV to make IV unpredictable.

Modifications in V3.64(XD.0)b2 | 01/31/2005

1.     [BUG FIX]
    Symptom: The name of Domain name does not check properly in SMT 1. 
    Condition:

- In SMT 1->Edit Dynamic DNS->Edit Host, fill the record 1’s ”domain name” with ”xxx.dyndns.org”. and record 2’s ”domain name” with ”xxx.dyndns.org ”. (the domain name of record 2 contains a space at the end)

- The domain should not contain space, we should have a filter to check this.

- Set record 1’s ”Update policy” with ”Use WAN IP Addrsss” and record 2’s ”Update policy” with ”Let DDNS Server Auto Detect”.

- After the DDNS process updating, the domain name ”xxx.dyndns.org” will be resolved by the policy ”Let DDNS Server Auto Detect” not ”Use WAN IP Addrsss”. (the first DDNS query result was overwritten by the second executed, 
"xxx.dyndns.org" is the first, "xxx.dyndns.org " is the second)

2.     [ENHANCEMENT] On eWC>BW MGMT>Class Setup, add a popup warning 
message "Delete Class : class name ?" before user delete a Class.

3.     [ENHANCEMENT] Add a active checkbox for ipsec rule on VPN wizard.

4.     [BUG FIX]
Symptom: The wording of Dial Backup in SMT is not consistent with GUI. 
Condition:
(1) In "eWC->WAN->Dial Backup", one of the wordings in "Budget" is "Always On".

(2) In SMT, the wording is "Nailed-Up Connection".

5.    [BUG FIX]
Symptom: While performing “Chariot 128 application 48 hours stress testing”, ZyWALL crashed several . 
Condition: Chariot Server<-----DUT----->Chariot end point
(1) DUT reset default romfile, and only configured WAN and LAN IP address.
(2) Traffic direction: Server to end point.
(3) Execute Chariot (automation.exe) after load stress file (stress-all.txt)
(4) After a while, DUT crashed

6.     [BUG FIX]
    Symptom: The traffic redirect should have higher priority than dial backup. 
    Condition:
    (1) In eWC>WAN>Route, set Traffic Redirect priority smaller than Dial Backup, 
          then click Apply.
    (2) It can be saved.

7.     [BUG FIX]
Symptom: Enter special url will cause device crash.
Condition: Form LAN site, enter http://192.168.1.1/Forms/rpAuth_1?ZyXEL%20ZyWALL% 20Series<script>top.location.pathname=%20""</script> on browser, the device will crash.

8.     [BUG FIX]
Symptom: The CI command ”ip nat service irc” may display strange Enable state. Condition:
(1) Execute ”ip nat service irc he_is_good”.
(2) Execute ”ip nat service irc 0”.
(3) Execute ”ip nat service irc he_is_bad”.
After Step 3, you will see that a strange Enable state, e.g., ”IRC enable = 12”.

9.   [BUG FIX]
      Symptom: The eWC>Firewall>Rule Summary>EDIT RULE page might be 
      corrupted.
      Condition:
      (1) Go to eWC>Firewall>Rule Summary.
      (2) Add or Edit a firewall rule.
      (3) Try to delete a Source Address (or Destination Address) without first selecting an 
            address.
      (4) Or try to delete a Service without first selecting a service.
      (5) With 3 or 4, you will see an error message on the status bar.
      (6) Click on any button of this page, and then you will see that the values of some fields 
            on this page are lost. Also you won't be able to escape this page by clicking on the 
            Cancel button.

10.   [ENHANCEMENT] Add SIP protocol in service list in firewall rule edit page.

11.   [BUG FIX]
        Symptom: In SMT 15.1 address mapping rule error message not correct.
        Condition:
        (1) In SMT 15.1, configure NAT address mapping many to many overloads(or many 
              one to one).
        (2) Configure local address from 0.0.0.0 to 255.255.255.255.
        (3) Configure global address from 0.0.0.0 to 255.255.255.255.
        (4) Save the configuration =>error message show “The end IP address must be great 
              than the start IP address “ not correct.

12   [BUG FIX]
Symptom: Configure WAN page, and WAN  priority will become 1. 
Condition: 
(1) In "eWC->WAN->General", set WAN1 priority to 5.
(2) In "eWC->WAN->WAN"., set encapsulation type to PPTP or PPPoE.
(3) Go to "eWC->WAN->General", WAN's priority will become 1.

13   [ENHANCEMENT] Give a warning message when user configure FTP/SIP/H.323 filter on BWM but FTP/SIP/H.323 alg is not enabled.
GUI : Save the filter and show the warning message. Warning: This is a SIP(FTP, H.323) filter, you have to enable SIP(FTP, H.323) ALG by CI command "ip alg enable". 
CI command : After running "bm config save", the router will save the configuration and check all filters in all interface. Then show a list of filters which are conflicted.

14   [ENHANCEMENT] NAT address mapping need prevent user configure local IP 
range and global IP range overlap.

15   [BUG FIX]
Symptom: SIP WiFi-Phone's voice communication failed.
Condition: 
(1) Use following topology to test. 
      WiFi A–(L)ZW35(W)----Internet(SIP server)---(W)ZW5(L)----WiFi B

(2) Both zywall reset to default romfile.
(3) In SMT 24.8 CI command, both type “ip alg enable ALG_SIP” to enable SIP 
      ALG.
(4) WiFi A make a phone call to WiFi B, voice communication works fine.
(5) Terminate the phone call,then WiFi B make a phone call to WiFi A, voice 
      communication fail.
(6) Fail status: WiFi A can hear voice, but WiFi B can't.

16   [BUG FIX]
Symptom: The deivce crashes while the user is changing the SNMP access right 
configuration.
Condition:
(1) Restore default romfile.
(2) Set the SNMP Access = Disable.
(3) Use MS-SOFT to query the device.
(4) Before the query timeout, change Access = ALL, the device will crash.

17   [BUG FIX]
Symptom: In authentication server, the local user database should check if the input user name is duplicate. 
Condition: 
(1) Restore to default romfile.
(2) In record 1, active = yes, name = test, password = 1234 In record 2, active = yes, 
      name = test, password = 5678
(3) Press Save and this configuration will be accept by router.

18   [BUG FIX]
    Symptom: BWM linear search can not find first match filter.
    Condition:
    PC1 ------------  (LAN) Router (WAN) ----- PC2
    (1) In router, enable BWM on WAN, setup two classes for WAN Root class: 
          1000kbps
          |-------Class 1: 200kbps
          |-------Class 2: 200kbps

Filters table:

Class 1: FTP SrcIP = 192.168.1.0/24

Class 2: FTP DstIP = 192.168.70.0/24
        (2) FTP upload file from PC1 to PC2.
        (3) In this case, BWM will match Class 2's filter. But it's wrong, in linear search    
              algorithm, we should return the first match filter for traffic.

19.  [BUG FIX]
Symptom: When manual mode encapsulation is Tunnel, responder can’t build up tunnel. 
Condition:
(1) PC A – ZW70 ---- ZW5 – PC B
(2) On eWC/VPN/Manual add two manual rules in ZW70 and ZW5. Rule 1 is 
      inactive. Rule 2 is active and encapsulation is Tunnel.
(3) PC A ping PC B, check SA Monitor, ZW70 tunnel had been built up but no tunnel 
      is up in ZW5, vice versa.
(4) If PC B ping PC A this time, tunnel can be built up in both sides and traffic can be 
      transferred.

20.   [BUG FIX]
        Symptom: LAN static DHCP can save the same data.
        Condition:
        (1) Restore default rom file.
        (2) In GUI>LAN>Static DHCP, add two record as MAC: 01:01:01:01:01:01, IP: 
              192.168.1.33 MAC: 02:02:02:02:02:02, IP: 192.168.1.66 and apply it.
        (3) Change these two record as MAC: 03:03:03:03:03:03, IP: 192.168.1.99 and apply 
              it.
        (4) It can be saved and it is wrong.

21.   [BUG FIX]
Symptom: Nail up warning message does not show correctly in eWC->WAN->WAN. Condition: 
(1) Edit a VPN rule and enable nail up
(2) In eWC->WAN->WAN, set encapsulation with PPPoE and no nailed-up 
      enabled, click "apply" to save, the status will show "Warning: VPN Nailed-Up 
      may trigger dial WAN links."
(3) Click "apply" again,the status will show "Nothing changed; no need to perform 
      save"

22.   [BUG FIX]
Symptom: VPN tunnel cannot be disconnected.
Condition: 
(1) PC1—ZW5-----HUB-----ZW10W(V362WH7)--PC2
(2) ZW5 has one IKE and two IPSec rules
(3) ZW10W has two VPN rules
(4) ZW10W initiates these two VPN rules
(5) ZW10W delete these two VPN tunnels but one of ZW5 VPN tunnels can not be 
     disconnected

23.   [BUG FIX]
Symptom: When out of call schedule, the device still cannot send traffic out. Condition:
(1) Set WAN 1 encapsulation is Ethernet.
(2) Edit SMT menu 24.10, Time Protocol = Manual, New Time (hh:mm:ss) = 
      10:00:00, New Date (yyyy-mm-dd) = 2004-06-01.
(3) Edit SMT menu 26, enter Schedule Set Number to Configure = 1, Edit Name = 
      FD-Once.
      - How often = Once
      - Once Date = 2004-06-01
      - Start Time = 10:05
      - Duration = 00:02
      - Action = Force Down
(4) Edit SMT menu 11.1, schedule = 1.
(5) However, when out of schedule about 5 minutes, device still cannot send traffic 
      out.

24.   [ENHANCEMENT] Add "Session Table is Full!" log message, when tos session is full.

25.   [BUG FIX]
Symptom: Wireless CI command “wlan active 100” can be save.(The value should be 1 or 0) 
Condition:
(1) Plug in B120 and reboot router.
(2) Use "wlan active 100" and it can be save.
(3) Go to smt3-5, router will crash.

26.   [BUG FIX]
        Symptom: The centralized log shows the strange DHCP entry with hex IP address. 
        Condition: 
        (1) The device enables LAN DHCP server.
        (2) A PC is set on device LAN site with dynamic IP and no system hostname.
        (3) The PC sends DHCP request to device.
        (4) The device will show the strange log message have the hex IP address. (ex: 101 
              01/15/ 2005 10:15:50 DHCP server assigns 0xa0a01e6 to 00:0E:08:AA:B6:B3)

27.   [ENHANCEMENT] When router reset, console will display the reset date and time.
For example, .\sys_cmd.c:869 sysreset() ZyWALL 5 system reset at 01/18/2005 15:07:48

28.   [BUG FIX]
Symptom: VPN page cannot be configured.
Condition: 
(1) Go to eWC>VPN>GATEWAY POLICY>EDIT to add a GATEWAY POLICY 
      rule. 
(2) Go to eWC>VPN>NETWORK POLICY>EDIT to add 10 NETWORK POLICY
      rules and bind them with the GATEWAY POLICY rule which was added in 
      Step1.
(3) Delete the GATEWAY POLICY rule which was added in Step1 and 10 
      NETWORK POLICY rules will be put into the Recycle Bin
(4) VPN page can't be configured anymore.

29.   [BUG FIX]
Symptom: Enhance the VPN error description 
Condition: 
(1) On eWC VPN, add a IKE rule Dynamic rule (Remote Gateway Address is 0.0.0.0)
(2) Add an Ipsec rule, and  fill some value instead of 0.0.0.0 in "Remote Network" 
      fields.
(3) Status will show "This policy cannot bound to the dynamic rule"
(4) User may not know where is wrong.

30.   [FEATURE CHANGE] Enhance Gateway Domain Name Update Timer. If Gateway Domain Name Update Timer is enabled. The ZyWALL will resolve the IP from a VPN gateway policy whose IKE remote gateway is domain name type in every cycle. If the ZyWALL finds that the new remote gateway IP is different from the old one( which is used by tunnel now), the ZyWALL will delete this tunnel.

31.   [BUG FIX]
Symptom: Save a legal VPN gateway policy but the ZyWALL shows an error message.
Condition: 
(1) GO to eWC>VPN>GATEWAY POLICY – EDIT
(2) Save a GATEWAY POLICY whose name = GW, My Address = 
      www.abc.com.tw, Remote Gateway Address = www.cde.com.tw and Pre-Shared 
      Key = 12345678
(3) GO to eWC>VPN>NETWORK POLICY - EDIT 
(4) Save a NETWORK POLICY whose name = NW, Active = Yes, Starting IP 
      Address = 192.168.1.33, Starting IP Address = 192.168.2.33 and Pre-Shared Key 
      = 12345678
(5) Go back to eWC>VPN>Rules and edit rule "GW" and set its My Address as 
      0.0.0.0, then save
(6) The ZyWALL shows an error message "This IKE rule has static policy rules.", but 
      it should not.

32.   [BUG FIX]
Symptom: There are no logs in eWC>Logs>Log Settings when SMTP authentication fail . 
Condition: 
(1) Go to eWC>Logs>Log Settings. Configure a wrong Mail Server/Send Log 
      to/Send Alerts to/ User Name of SMTP Authentication/Password of SMTP 
      Authentication and save.
(2) Go to eWC>Logs>View Log. There are no logs about SMTP Auth failures/SMTP 
      failures.
(3) If the configuration is correct. There is also no log to tell users that the result is 
      successful.

33.   [ENHANCEMENT] Add port information in centralized log message when a netbios 
packet was blocked.

34.  [ENHANCEMENT] After the device rebooting, the system will synchronize Time 
server until any WAN is up or all WAN links are failed exceed 5 minutes. If NTP server is on LAN/DMZ subnet, DUT still won’t sync when WAN interface is down.

35.   [BUG FIX]
Symptom: VPN tunnel can be established but traffic cannot go through tunnel. 
Condition: PC1 -- ZyWALL -- Any Router/Internet -- ZyWALL -- PC2
(1) Configure corresponding VPN setting in both ZyWALLs.
(2) Dial VPN tunnel
(3) After tunnel established, PC1 cannot ping PC2 vice versa.

36.   [BUG FIX]
Symptom: The router cannot flush correctly in eWC->LOGS->Reports. 
Condition:
(1) In Bridge Mode.
(2) In eWC->LOGS->Reports, enable “Collect Statistics”, interface = LAN, Report 
      type= "Host IP Address".
(3) When pressing "Flush" button, there is still one record existing "192.168.70.123 
      Outgoing 3913 bytes". "192.168.70.123" is router's IP address.
(4) It has the same problem when changing interface from "LAN" to "DMZ" if we do 
      the same action.

37.   [BUG FIX]
Symptom: In bridge mode, SIP traffic cannot be managed by BWM. 
Condition: SIP Phone1 ----- (LAN)ZyWALL(WAN) ------ SIP Phone2 
(1) Change router to Bridge Mode.
(2) Enable BWM, and add a SIP filter at WAN interface.
(3) SIP Phone1 call SIP Phone2.
(4) After connection is established, go to eWC->BW MGMT->Monitor, you will see 
      SIP traffic falls into Default class, it's wrong.

38.   [BUG FIX] 
Symptom: Packet still can send out through NAT router when there is no unused port for it. 
Condition: 
(1) Configure an active port forwarding rule with incoming port range 10000 to 
      29999.
(2) Send a packet out of NAT router.
(3) The packet can still send out.

39.   [BUG FIX] 
Symptom: BWM highest priority class cannot borrow residual bandwidth from parent class (using tfgen tool) 
Condition: 
(1) In WAN interface. Enable Priority-based Scheduler.
(2) Class Setup on WAN. 
      Root 100000 Kbps
      |-----WAN 2000 Kbps (No Borrow, No Filter, Priority = 3) 
      |-----WAN1-1 500 Kbps (Borrow; Filter: SrcIP:0, DestIP:0, SrcPort:0, 
              DestPort:90: Protocol: 17; Priority = 3 ) 
      |-----WAN1-2 300 Kbps (Borrow, Filter: SrcIP:0, DestIP: 192.168.70.0/24, 
              SrcPort:0, DestPort:0, Protocol: 17; Priority= 6) 
(3) From LAN host, use tfgen (UDP packet generator) to generate two session to 
      match  class WAN1-1 and WAN1-2. 
      session 1: Utilization = 2000Kbps, Destination = WAN host (192.168.70.57), 
      port=90. This will match WAN1-1 class. 
      session 2: Utilization = 2000Kbps, Destination = WAN host(192.168.70.57), port 
      = default. This will match WAN1-2 class 
(4) From Monitor, WAN1-1 should be protected at 500Kbps, and WAN1-2 should 
      borrow  remaining bandwidth from parent class. 
But you will see WAN1-1 still borrow remaining bandwidth and WAN1-2 almost borrows nothing from parent class.

40.   [BUG FIX]
Symptom: There is no response from DMZ after set system name by SNMP. 
Condition: 
(1) Reset to factory default setting.
(2) Disable firewall.
(3) Ping router's DMZ IP address continuity.
(4) Set DUT’s system name by SNMP tool "MG-SOFT MIB browser".
(5) There is no response from DMZ anymore.

41.   [BUG FIX]
Symptom: BM filter cannot be deleted via CI command.
Condition: 
(1) On eWC->BW MGMT->Class Setup, create 3 classes on LAN interface. all 
      classes have filter enabled.
(2) Go to SMT 24.8, delete the third filter by "bm filter lan del 3" and then save data 
      by "bm config save"
(3) By typing, "bm show filter", you will see the third filter still exists.

42.   [BUG FIX] 
Symptom: Device will crash.
Condition: Use IXIA to simulate 1012 ip address to access web site ( every ip has 10 sessions ), device will crash.

43.   [BUG FIX]
Symptom: Memory leak in DNS query.
Condition:
(1) Set the device as the network gateway.
(2) Some PCs assign the DNS server to the device.
(3) After some days, the DNS query will cause memory leak.

44.   [BUG FIX] 
Symptom: Executing CI command ”ip nat service irc” will make the router crash. Condition:
(1) In SMT 24.8, type ”ip nat service irc” then press enter.
(2) The router crash.

45.   [BUG FIX]
        Symptom: NAT address mapping functionality fail. 
        Condition: 
        (1) Restore to factory default.
        (2) In SMT4, set ”Network Address Translation” as ”Full Feature”.
        (3) In SMT 15.1.1, insert a rule in rule 1. Take an example with my setting: Type: One 
              to One. Local IP: 192.168.1.33 Global IP: 192.168.70.111 (FTP server in 
              192.168.70.8)
        (4) In PC/192.168.1.33, ftp to server/192.168.70.8.
              In FTP server, you can find the incoming IP is 192.168.70.111. (This is right) 
              Then logout the ftp.
        (5) Repeat step 3 but change the Global IP: 192.168.70.123
        (6) Repeat step 4, you can find the incoming still 192.168.70.111. This is wrong, it 
              should be 192.168.70.123.

46.   [FEATURE CHANGE] Extend "devID" field to six hexadecimal numbers(12 
characters) in syslog format.

47.   [BUG FIX]
Symptom: Netmeeting H.323 traffic will be blocked by Firewall if connection is initiated from WAN side to LAN side.
Condition:
PC1(Netmeeting)--------(LAN) ZyWALL (WAN) --------- PC2(Netmeeting)
(1) Enable Firewall, setup a WAN2LAN firewall rule for H.323 service
(2) Enable NAT port forwarding for port 1720(H.323) to PC 192.168.1.33 
(3) PC1 and PC2 use Netmeeting, PC2 call PC1.
(4) Netmeeting application traffic will be blocked by Firewall, you will see a lot of 
      Firewall blocked log in Centralized LOG.

48.   [BUG FIX]
        Symptom: After VPN tunnel is established, user will see DPD packet while traffic 
        still can be transferred through tunnel.
        Condition:
        PC1-------- ZyWALL-A ============= ZyWALL-B ------- PC2 IPSec tunnel
       (1) Configure VPN tunnel between ZyWALL-A and ZyWALL-B.
       (2) In ZyWALL-A eWC->VPN->Global Setting, set Output Idle Timer = 120. 
       (3) Reboot ZyWALL-A.
       (4) PC1 ping PC2 to trigger tunnel.
       (5) after tunnel is established, users will see ZyWALL-A's LOG show DPD packets.

49. [ENHANCEMENT] BWM children's bandwidth's sum will not exceed parent’s.
For example, the bandwidth of WAN interface is 50000 kbps. The sum of all children's bandwidth can not exceed 50000 kbps

Modifications in V3.64(XD.0)b1 | 12/17/2004

- [ENHANCEMENT] Redesign IPSec mechanism to comply with ICSA Labs 1.1D 
  IPSec Certification Testing.
  New feature added :
  (1) Multiple Proposal.
  (2) Support Nail Up, Dead Peer Detection, Control Ping.
  (3) Separate IPSec SA (Phase 2) from IKE SA (Phase 1), multiple IPSec SAs can bind 
  to one the same IKE SA. (Multiple policy)
  (4) Add a “Global Setting” tab in eWC->VPN which contains some timer settings.
  (5) IKE and manual key rules have their setting pages respectively in eWC->VPN.
  (6) Remove the VPN setup page (SMT 27)
  (7) Redesign lots of IPSec CI command.

2.     [ENHANCEMENT] Support Port Restricted Cone NAT.
3.     [ENHANCEMENT] Redesign eWC->BW MGMT->Class Setup page.

4.     [ENHANCEMENT] Enable "ip alg" command in bridge mode.

5.     [ENHANCEMENT] Add the eWC>CONTENT FILTER>Cache and eWC>DNS>Cache GUI. 
(1) Add total cache entry number info.

(2) Remove the "Port" info in URL Cache Entry table.

(3) The "Action" in URL Cache Entry table shows "Blocked" first by default.

(4) The URL entry in URL Cache Entry table aligns to the left.

(5) On the URL Cache Entry table, if the length of a URL entry is over 50, it will be truncated to 50 characters, with three trailing dots (...) appended.

(6) To adjust the note font size in eWC>DNS>Cache GUI.

6.    [ENHANCEMENT] Popup message improvement: "Delete this rule?" => "Delete entry #[number] ?"

7.    [ENHANCEMENT] DNS adds CI command "ip dns system cache flush".

8.    [ENHANCEMENT] eWC>LOGS>Reports>Report Type>”LAN IP Address” renamed as ”Host IP Address”

9.    [ENHANCEMENT] In eWC>DNS>System>Address Record, add Wildcard.

10.  [ENHANCEMENT] Add length checking of DNS(Peer ID Type) content in VPN.

11.  [ENHANCEMENT] Integration of TOS & NAT information
(1) Current concurrent sessions = max(TOS current concurrent sessions, NAT current concurrent sessions)
(2) Historical high since last startup = max(TOS historical high since last startup, NAT historical high since last startup)

12.  [ENHANCEMENT] Add FQDN support in my IP address in IKE.

13.  [ENHANCEMENT] IPSec GUI enhancements
(1) On eWC>VPN>Global Settings, add IPSec timers configuration.
(2) On eWC>VPN>Network Policy Edit page, add Netbios passthrough field.
(3) On eWC>VPN>Gateway Policy Edit page, add FQDN field for My ZyWALL.

14.   [ENHANCEMENT] Enhance ZyWALL GUI.
(1) To allow more than two child windows open from multiple ZyWALLs, the
second parameter (windowName) of the JavaScript function Window.open() will
be the MAC address of the ZyWALL that is currently being managed. The child
windows include the following.

1) Wizards

2) Help

3) Show Statistics

4) Show DHCP Table

5) VPN Status

6) BWM statistics
(2) For identification purpose, the title of the eWC parent window, as well
as its child windows, will contain the system FQDN of the ZyWALL that is
currently being managed.

15.  [ENHANCEMENT]
(1) In eWC>Home>System Time, add GMT timezone + DST offset.
(2) In eWC>Date&Time>Current Time, GMT add timezone + DST offset.

16.  [ENHANCEMENT] Add GUI for LAN DHCP Relay feature.

17.  [ENHANCEMENT] Auth Server/Local User Database needs long time to save all entries, enhance the saving policy to speed up this action.

18.  [ENHANCEMENT] In SMT 24.6, the menu adds the reminding message "You can enter ctrl-x to terminate operation any time."

19.  [ENHANCEMENT] Add a API function to move rules for NAT address mapping table. CI command: ip nat acl move <set#> <rule# from> <rule# to>

20.  [ENHANCEMENT] For Manual IPSec rule, the "My ZyWALL" and "Remote Gateway Address" should not have FQDN fields. (Remove My Domain Name and change Secure Gateway Address into IP field)

21.  [ENHANCEMENT]
(1) In eWC>MAINTENANCE>General, change the type of the "Administrator
Inactivity Timer" field from ASCII to integer.
(2) Add a JavaScript Global function to avoid filling any character in the
specific fields on both IE and Netscape. (allow number only)

22.  [ENHANCEMENT] Add a "Log" check box for "VPN connectivity check". in eWC>VPN>NETWORK POLICY>EDIT.

23.  [FEATURE CHANGE] Modify CI command "ip arp add" from hidden to visible.

24.  [ENHANCEMENT] For single WAN, the WAN cannot receive an IP from DHCP server with the same subnet with other interfaces.

25.  [ENCHANCEMENT] The new DST feature allows user to know the start/end date. It will be nice if the ZyWALL shows what date ’1st Sun in April’ is ----.  And there is some spare space on the screen on that line.

26.  [ENHANCEMENT] User can use telnet/ping/ via VPN in SMT menu 24.8.
(1) If you telnet/ping/... from your ZyWALL to an IP on the VPN "remote network" and the ZyWALL’s LAN IP (including alias IP) is on the VPN "local network", the ZyWALL uses LAN IP as source.
(2) If you telnet/ping/... from your ZyWALL to an IP on the VPN "remote network" and the ZyWALL’s DMZ IP (including alias IP) is on the VPN "local network", the ZyWALL uses DMZ IP as source.
(3) (For future wireless enhancement) If you telnet/ping/... from your ZyWALL to an IP on the VPN "remote network" and the ZyWALL’s WLAN IP (including alias IP) is on the VPN "local network", the ZyWALL uses WLAN IP as source.
(4) Otherwise the ZyWALL uses any appropriate interface IP as source depending on the routing table.
Note: If there are more than one appropriate local interfaces, router will use the first matched local interface IP address as the source IP address.

27.  [ENHANCEMENT] In GUI>NAT>Port Forwarding, router will now check if the translated end port is out of 65535.

28.  [ENHANCEMENT] On eWC>HOME>VPN wizard, My ZyWALL address support Domain name.

29.  [ENHANCEMENT]
(1) In eWC>MAINTENANCE>F/W Upload, the warning message title
should be red in order to be consistent with the style of other warning message.
(2) In eWC>MAINTENANCE>Restore Configuration, the warning message title
should be red in order to be consistent with the style of other warning message.

30.  [ENHANCEMENT] On eWC>NAT>AddressMapping, add dynamic display for "Go To Page". If there are less than 10 address mapping rules, then hide "Go To Page",
else display "Go To Page".

31.  [ENHANCEMENT] When we receive a non-encrypt initial content payload in IKE, we will ignore it.

32.  [ENHANCEMNET] Add payload information in IKE LOG. Besides reason, we also show which payload caused the IKE LOG.

33.  [ENHANCEMENT] HOME>Internet Access, the "First DNS Server", "Second DNS  
Server" is inconsistent with DNS>Name Server Record. 
The specified "First DNS Server", "Second DNS Server" will be updated in  
eWC>DNS>Name Server Record.

34.  [ENHANCEMENT] In GUI>WAN, add "Authentication Type" field.

35.  [ENHANCEMENT] For DHCP server, if the requested client does not have a host 
name, the log will show MAC address instead of nothing.

36.  [ENHANCEMENT]
(1) In eWC>CONTENT FILTER>Cache, if users click Action/URL/Remaining Time to sort the cache entries, the page will not jump to the top of this page before it refreshes.
(2) By using Firefox/Netscape in eWC>CONTENT FILTER>Cache, if useres click
Action/URL/Remaing Time to sort the cache entries, the page will refresh
immediately.

37.  [ENHANCEMENT] In the past, we can delete a tunnel in SMT 27 and can only do this in eWC. Now, Add a CI command "ipsec drop <policy index>" to delete a tunnel and
"ipsec show_runtime list" to list the active VPN tunnel.

38.  [ENHANCEMENT] Consolidate "Receive IPSec packet, but no corresponding tunnel exists" logs.

Modifications in V3.62(XD.2) | 09/24/2004

Modify for formal release.

Modifications in V3.62(XD.2)b3 | 09/21/2004

- [BUG FIX] 
Symptom: LAN host will get wrong DNS server.
Condition:
1. Set SMT 3.2 DNS first DNS server as user defined 1.1.1.1. Others are none.
2. Unplug WAN port and reboot.
3. LAN host get IP address and DNS server and the DNS server is LAN IP.

Modifications in V3.62(XD.2)b2 | 09/17/2004

- [BUG FIX]
Symptom: LAN host ping device LAN IP a period time, then PPPoE/PPTP will be triggered dial.
Condition:
1. Set WAN 1  are PPPoE.
2. LAN host ping device LAN IP a period time, then WAN 1 will be triggered dial.

- [BUG FIX]
Symptom: Firewall sends TCP RST after it blocks traffic period of time.  
Condition:
1. Configure Firewall LAN to WAN blocked and enable log
2. Generate one TCP SYN packet from LAN to WAN
3. Firewall will block this packet and generate block log
4. After period of time (30 seconds), Firewall log shows it sent TCP RST to both client and server side

- [BUG FIX]
Symptom: System has a lot of long timeout UDP sessions. 
Condition: 
1. Enable firewall. 
2. Display TOS sessions.
3. A lot of long timeout UDP sessions.

- [BUG FIX]
Symptom: ZyWALL crashes very often in bridge mode.
Condition:
1. Switch to bridge mode.
2. Enable Firewall.
3. ZyWALL crashes very often.

- [ENHANCEMENT] Enhance "cnm keepalive" ci command. Add "cnm keepalive 0" command to stop sending of keepalive packet to Vantage.

- [BUG FIX] Symptom: Symptom: FTP from WAN to LAN does not work.
Condition:
1. Set a FTP server on a host in the LAN side and configure a default server to this host.
2. Using FTP from WAN to the default server with port mode.
3. After typing username and password, "ls" command does not work.

- [BUG FIX] Symptom: LAN host will get wrong DNS server.
Condition:
1. Set SMT 3.2 DNS first DNS server as user defined 1.1.1.1. Others are none.
2. Unplug WAN port and reboot.
3. LAN host get IP address and DNS server and the DNS server is LAN IP.

- [BUG FIX] Symptom: System Crash when change encryption key in Vantage.
Condition:
1. Device register to Vantage in router mode under DES and PPPoE.
2. configuration>>general>>system change the original encryption key and apply
3. Device receives data but soon the system crash.

- [BUG FIX] Symptom: WAN Gateway will be reset to 0.0.0.0.
Condition:
1. In Vantage CNM add a device (the device have a static IP),when it register to Vantage. Vantage set default value to device.
2. After the device reset, WAN Gateway will be reset to 0.0.0.0.

- [BUG FIX] Symptom: CNM agent accepts wrong CI command ”cnm keepalive -32312312222222222222222”.
Condition:
1. In SMT 24.8, type ”cnm keep -32312312222222222222222”.
2. The system accepts it and saves with the value.

- [BUG FIX] Symptom: CNM agent accepts wrong CI command ”cnm encrymode 1231223”.
Condition:
1. In SMT 24.8, type ”cnm encrymode 1231223”.
2. The system accepts it and read it as ”65535”.

- [BUG FIX] Symptom: [Vantage] Configuration>>VPN: When delete a active VPN tunnel successfully. Device sends VPN tunnel status "Destroy" to vantage.
Condition:
1. Create and dial up a VPN tunnel via Vantage.
2. Delete this active rule in Vantage.
3. Vantage server will have exception.

- [BUG FIX]
Symptom: eWC will fill the "Connection ID/Name" field with "C:1" when the fetch data is empty.
Condition:
1. In eWC, set "Connection ID/Name" as empty in PPTP mode and apply it.
2. Go go another page and go back the WAN page, the "Connection ID/Name" field is filled with "C:1" even we set the field as empty.

Modifications in V3.62(XD.2)b1 | 08/16/2004

- [ENHANCEMENT]
Add Unified ALG for SIP and H.323.

- [ENHANCEMENT]
Each unified ALG can be enabled/disabled. The default ALG setting for SIP and H.323 is disabled.

- [ENHANCEMENT]
Firewall can bypass AX.25 (protocol #93) & IPv6 (protocol #41) protocols.

- [BUG FIX]
Symptom: Bandwidth management with ALG_H.323 cause system crash.

Condition:

1. Create a class with a Service-H.323 filter in WAN1 interface.

2. Unplug all WAN’s cable
3. Lanch the ”Openphone” application that supports H.323 and make a call.

4. Router crashes.

- [BUG FIX]
Symptom: Router block trusted web content.
Condition:
1). In "eWC->CONTENT FILTER->General", enable content filter.
2). In "eWC->CONTENT FILTER->Customization", select check boxes of "Enable Web site customization" and "Disable all Web traffic except for trusted Web sites".
3). In "eWC->CONTENT FILTER->Customization", set "www.hellowork.go.jp" as trusted web site.
4). Open browser and access http://www.hellowork.go.jp/kensaku/servlet/kensaku?pageid=001
5). In the new page, select third and fourth radio bottom and click "search" bottom.
6). In the new page, click "next page" bottom.
7). The new page will be blocked.

- [BUG FIX]
Symptom: External Content Filtering cannot block the URL belonging to restricted category.
Condition:
1). In "eWC->CONTENT FILTER->Customization", unselect "Enable Web site customization".
2). Add a URL to "trusted web sites".
3). In "eWC->CONTENT FILTER-Customization>, select "Block Web sites which contain these keywords".
4). In "eWC->CONTENT FILTER->Categories", select the category which the URL belongs to.
5). Access the trusted URL.
6). The URL will not be blocked.

- [BUG FIX]
Symptom: System crash by memory leak.
Condition:
1). Enable bandwidth management.
2). Into eWC->Bandwidth Management->Monitor and wait for a period time.
3). System crash by memory leak.

- [BUG FIX]
Symptom: Remote node CI command crashes.
Condition:
1). Goto SMT 24.8
2). Load dial backup remote node to working buffer.
3). Type CI command "sys rn accessblock 0".
4). Save this remote.
5). System crashes.

- [BUG FIX]
Symptom: System crash when someone want to configure NAT mapping rules.

Condition:

1. Use the terminal program to login the console.

2. Enter SMT 15, NAT Setup

3. Select 1 to enter SMT 15.1, Address Mapping Sets.

4. The system crash

- [BUG FIX]
Symptom: eWC>NAT>ADDRESS MAPPING edit page leaks memory.

Condition:

1. Log on to eWC.

2. Go to eWC>NAT>ADDRESS MAPPING edit page, and then click Cancel.

3. Repeat Step 2 for several times.

4. Check system memory info by the CI command: system memu ms You will observe abnormal increases of memory sections, indicating memory leaks.

- [BUG FIX]
Symptom: Trigger port will disappear after system reboot.
Condition:
1. Configure Trigger port rule.
2. System reboot.
3. The configured Trigger port rule disappear.

- [BUG FIX]
Symptom: The system might crash when enabling IPSec.
Condition: During IKE negotiation the system might crash.

- [BUG FIX]
Symptom: MSN Messenger's "Ask for Remote Assistance" function causes system crash.
Condition:
1. Enable UPnP.
2. Set PC(A) and router(B) in intranet and PC(C) connects to LAN port of router(B).
3. Test MSN Messenger's "Ask for Remote Assistance" function from PC(A) to PC(C).
4. After PC(C) accepts the PC(A) request by "Ask for Remote Assistance" then the device will crash.

- [BUG FIX]
Symptom: System out of memory.
Condition:
1. Let the ZyWALL be a DNS proxy for LAN hosts.
2. Do a lot of DNS inverse queries by running IPScan tool continuously from LAN host.
3. After a long time, the ZyWALL will out of memory.

- [FEATURE CHANGE]
Change UPnP device name for ZyWALL35 and ZyWALL5
WAS: "ZyXEL ZyWALL 35 Internet Security Gateway"
IS:  "ZyXEL ZyWALL 35 Internet Security Appliance"

- [BUG FIX]
Symptom: Packets cannot pass through NAT router to LAN hosts.
Condition:
1. NAT default server is on
2. Procotol of the packet is not TCP, UDP, ICMP, ESP, GRE.
3. Packets from WAN to router.
4. Packets cannot pass through NAT router to LAN hosts (NAT default server)

- Symptom: External Content filtering cannot register.
Condition: 
1. In "eWC->content filter->categories", click "register" to connect to ZSSW.
2. Do the registration on ZSSW.
3. The registration will fail in the final step.

- [ENHANCEMENT]
External content filtering support full URL checking.
Was: External content filtering only take domain name or IP address of URL into category checking.
Is: External content filtering put entire URL into category checking.

- [ENHANCEMENT]
CI command to turn off triangle route log, multicast log and broadcast log.
1. Add CI commands:
a. "sys logs switch".
b. "sys logs switch display".
c. Triangle route log switch: "sys logs switch bmlog <0:no|1:yes>" 
d. Broadcast/Multicast log switch: "sys logs switch trilog <0:no|1:yes>".

- [BUG FIX]
Symptom: System time problem.
Condition:
1. enter SMT24.10, configure time server.
2. open daylight saving, configure the start time and end time so that current time is within the daylight saving time.
3. after writing to rom file, router ask you to calibrate the system clock, answer yes.
4. If system failed to connect time server, system time will add one hour, every time you enter smt 24.1,system time add 1 hour automatically.

- [FEATURE CHANGE]
Change external content filtering message on centralized log and blocked page for some error events.

- [BUG FIX]
Symptom: Router will crash.
Condition: When user continuously accesses eWC and press "Apply" button, sometimes router will crash.

- [BUG FIX]
Symptom: The system crashes after it receives a url that contains more than three "/"s behind the ip address (or domain name).

- [BUG FIX]
Symptom: Sometimes when connect to router by TCP, FTP or HTTP will fail.
Condition:
1. One user connects to router by FTP, TELNET or HTTP.
2. In TCP handshake, client doesn't receive SYN ACK. i.e., router is in SYN RECEIVE state.
3. Client timeout and send RESET to router.
4. Related socket in router is still alive and other users can't login router until this socket timeout.

- [BUG FIX]
Symptom: eWC spelling error: eWC->FirewallDefault Rule: Allow Asymetrical should be “Asymmetric”

- [BUG FIX]
Symptom: System out of memory and reboot when firewall enable.
Condition:
1. Enable firewall, then generate traffic.
2. The memory will slowly leak until it uses up all the memory, then reboot.

- [BUG FIX]
Symptom: Generate a lot of TCP port 80 sessions to ZyWALL will cause device to hang and reboot by hardware watchdog.
Condition:
1. Use session.exe to generate a lot of TCP port 80 sessions to ZyWALL's LAN or WAN interface
2. After several hundreds of sessions are established, the ZyWALL will hang and finally reboot.

- [ENHANCEMENT]
1. Support user config for SIP session timeout value.
2. Support SIP SDP multiple RTP port.
3. Delete unused ALG type.
4. Command for ALG enable/disable and sip timeout.

- [BUG FIX]
Symptom: Sometimes the ZyWALL reboots by software watchdog.
Condition:
1.Put the ZyWALL on the network for a long time.
2.Sometimes the ZyWALL will reboot by software watchdog.

- [BUG FIX]
Symptom: XAUTH with rule swap doesn’t work.
Condition:
1. In initiator, set up a VPN rule with XAUTH in client mode.
2. In responder, there are three VPN rules:
a. Rule 1 is XAUTH off.
b. Rule 2 is XAUTH with client mode.
c. Rule 3 is XAUTH with server mode (this rule corresponds to client rule).
3. Dial from initiator, and the tunnel will never be up.

- [BUG FIX]
Symptom: Content filter timeout problem. 
Condition:
1. A router is register the content filter (CF) server.
2. Enable the CF feature.
3. Enable the external database content filtering.
4. The router log often record "Waiting content filter server (server name) timeout!".
5. A PC in lan fetch web from internet often hang for a while.

Modifications in V3.62(XD.1) | 06/25/2004

1. Formal release.

Modifications in V3.62(XD.1)b1 | 06/16/2004

- [ENHANCEMENT] Suppout Vantage CNM 2.0 (Vantage Centralized Network Management).

Modifications in V3.62(XD.0) | 05/18/2004

1. Formal release.

Modifications in V3.62(XD.0)b5 | 05/14/2004

- [BUG FIX] Symptom: The ZyWALL might crash or hang when users browse eWCFirewallRule Summary.

Condition:

  - Log on to eWC.

  - Browse EwcFirewallRule Summary

  - The ZyWALL might crash or hang.

Modifications in V3.62(XD.0)b4 | 04/27/2004

- [FEATURE CHANGE]
Remove Policy Route feature from ZyWALL 5 because Policy Route is not defined in    product specification.

- [FEATURE CHANGE]
Maximum concurrent VPN tunnel number  is changed from 5 to 10.

- [FEATURE CHANGE]
The following default settings is changed:
(1) eWCFirewallAnti-Probing
WAS: Anti-Probing Respond Ping to LAN
IS: Anti-Probing Response Ping to LAN&WAN&DMZ
(2) eWCFirewallThreshold
WAS: TCP Maximum Incomplete Sessions = 10
IS: TCP Maximum Incomplete Sessions = 30
(3) eWCWANRoute
WAS: WAN Priority = 2
IS: WAN Priority = 1

- [BUG FIX]
Symptom: External Content Filtering cannot be registered.
Condition: 
(1) In eWCCONTENT FILTERCategories", click "register" to connect to ZSSW.
(2) Do the registration on ZSSW.
(3) Browser display "Please wait....." and the page of "Register successfully" does not appear.

- [BUG FIX]
Symptom: Traffic Redirect does not work.
Condition: 

  Internet ------- Router A ------ ZyWALL ----- gateway B ----- Internet
                                              WAN      LAN  
(1) Let ZyWALL WAN port connect to another router A and A is connected to Internet.
(2) Setup Traffic Redirect to backup gateway B located at LAN side.
(3) Disconnect the connection between router A and Internet.
(4) The ZyWALL can not do Traffic Redirect to gateway B located at LAN side.

- [BUG FIX]
CI command “ip igmp” is lost.

- [BUG FIX]
Symptom: The behavior in priority-based Bandwidth Management is not correct.
Condition: 
(1) In eWCBW MGMTSummary, activates WAN1 root class with Speed = 1500 kbps and Scheduler = Priority-Based
(2) In eWCBW MGMTClass Setup, Adds two sub-classes under WAN1 root class.
Where WAN1-1 : Bandwidth Budget = 200, Priority = 7(higher than WAN1-2), and “Borrow bandwidth from parent class” is selected; WAN1-2 : Bandwidth Budget = 500, Priority = 1, “Borrow bandwidth from parent class” is also selected.
(3) First generates traffic that satisfies WAN1-2 class, users will find WAN1-2 borrow the whole available bandwidth from parent, and the traffic is bound at about 1500kbps. 
(4) Then generates traffic that satisfies WAN1-1 class. Users will find WAN1-1 can not borrow bandwidth from parent class and bandwidth is bound at about 200kbps even though WAN1-1 has higher priority than WAN1-2.

- [BUG FIX]
Symptom: In eWCMAINTENANCEGeneral, set a number which is bigger than 1000 for Administrator Inactivity Timer. The label string 'Administrator Inactivity Timer' will disappear.
Condition: 
(1) Go to eWCMAINTENANCEGeneral, set a number which is bigger than 1000 for Administrator Inactivity Timer. 
(2) Click 'Apply'. 
(3) The label string 'Administrator Inactivity Timer' will disappear.

- [BUG FIX]
Symptom: ZyWALL ping sometimes fails.
Condition:
(1) Turn on Firewall.
(2) Go to SMT 24.8
(3) Ping to exist host, but it sometimes fails.

- [BUG FIX]
Symptom: In SMT 3.2, the subnet of ZyWALL LAN IP can be different from the subnet of DHCP client ip and ZyWALL LAN IP can be set within DHCP Client IP pool range.
Condition:
First case: 
(1) Go to SMT 3.2	
(2) Set DHCP client IP Starting address to be 192.168.2.3
(3) Set LAN IP Address to be 192.168.1.1, then confirm to save.
(4) These setting can be saved and no error message. 
Second case:
(1) In SMT 3.2, set DHCP client ip Starting address to be 192.168.1.3
(2) Set Size of Client IP Pool to be 10
(3) Set LAN IP Address to be 192.168.1.3, then confirm to save.
(4) These setting can be saved and no error message.

- [BUG FIX]
Symptom: Remote access control cannot work properly.
Condition: 
(1) Turn on bridge mode
(2) Configure telnet server access control from WAN only by SMT 24.11
(3) Telnet to device via WAN side
(4) The telnet connection fails.

- [BUG FIX]
Symptom: System crashes. 
Condition: Configure device by eWC sometimes cause crash.

- [BUG FIX]
Symptom: In bridge mode ZyWALL at eWCBridge, Bridge IP address settings can not be saved successfully.
Condition:
(1) Switch the ZyWALL to bridge mode.
(2) Go to eWCBridge page.
(3) Change "IP Address", "IP Subnet Mask", or "Gateway IP Address" then click "Apply" 
(4) Status shows "Configuration updated successfully" but the changes was not really saved.

- [BUG FIX]
Symptom: In SMT 24.11, the setting of DNS Service is displayed under bridge mode Condition:
(1) Go to SMT 1, change Device Mode to bridge mode.
(2) After reboot, go to SMT 24.11, DNS Service incorrectly appear.

Modifications in V3.62(XD.0)b3 | 04/04/2004

- [BUG FIX] 
Symptom: CI command error, ZyWALL will show some CI commands which don't belong to current command set. 
Condition:
(1) Go to SMT 24.8, CI command mode.
(2) Type "ip dns system", ZyWALL will correctly print two available commands, "edit" and "display".
(3) Type "ip dns sys", ZyWALL will unexpectedly print nine available commands instead of two. Those extra seven commands are not under "ip dns system".

- [BUG FIX]
Symptom: DHCP client cannot get address from router.
Condition: 
(1) In eWCLANLAN, configure router as a DHCP server and set IP pool starting address as 192.168.1.33.
(2) In eWCLANStatic DHCP, configure all rules in static DHCP table and the IP addresses are 192.168.1.33~192.168.1.40.
(3) Use a PC which MAC address is not in the static DHCP table to get a IP address from router.
(4) The PC cannot get the IP address.

- [BUG FIX]
Symptom: The ZyWALL will reset the current eWC HTTP session even when the LAN IP configuration is not successfully changed. Under this situation, users have to re-log in the ZyWALL. 
Condition: 
(1) Log in ZyWALL eWC, and go to eWCLAN.
(2) Deliberately configure the LAN IP address as within the WAN subnet.  
(3) Click Apply, then the status will show an error message indicating address conflict.
(4) The ZyWALL will then automatically break the current eWC HTTP session. To access the ZyWALL, users have to log in again.

- [BUG FIX]
Symptom: Router will crash when entering SMT menu 3.5 
Condition:
(1) Insert WLAN card. 
(2) In CI command, enter "wlan active 11" instead of "wlan active 1" to activate WLAN on router. 
(3) Enter SMT 3.5, router will crash.

- [ENHANCEMENT]
Supports Vantage CNM 2.0(Vantage Centralized Network Management)

- [BUG FIX]
Symptom: The Content Filtering blocks cookies even if it is not in the blocked schedule. Condition: 
(1) In eWCCONTENT FILTERGeneral, select "Block Cookies".
(2) In eWCCONTENT FILTERGeneral, set "Schedule to Block" with a time period NOT including the current time.
(3) Access a web site which contains cookies.
(4) The cookies will be blocked by the Content Filtering.

- [BUG FIX]
Symptom: WAN status in SMT 24.1 shows wrong information in bridge mode. Condition: 
(1) Configure Internet access as PPTP or PPPoE encapsulation in router mode.
(2) Switch ZyWALL to bridge mode.
(3) WAN status in SMT 24.1 shows idle and IP address is "0.0.0.0".

- [BUG FIX]
Symptom: Device cannot transfer Ethernet frame in bridge mode. 
Condition: 
(1) ZyWALL enables bridge mode.
(2) The Internet connection is under DMZ port.
(3) Plug Ethernet cable between one host and ZyWALL DMZ port. 
(4) This host starts to transfer packets to Internet.
(5) Unplug the Ethernet cable from DMZ port and plug in LAN port.
(6) This host cannot transfer packets to Internet anymore.

- [BUG FIX] 
Symptom: PPPoE connection sometimes fails in France.
Condition: Since France Telecom changes their core network setup to BRAS, ZyWALL PPPoE connection on authentication phase most of the time fails.

- [ENHANCEMENT]
Updates help pages for ZyWALL 5.

- [BUG FIX]
Symptom: On the eWCWIZARDInternet Access page, the System DNS Servers configuration is not available when the ZyWALL is not a DHCP server for its LAN hosts.
Condition:
(1) Log onto eWC, and go to eWCLAN. Uncheck the "DHCP Server" option to stop ZyWALL from being a DHCP server to its LAN hosts.
(2) Go to eWCHOMEWIZARDInternet Access. The System DNS Servers configuration is not available in the wizard.

- [ENHANCEMENT]
The ZyWALL 5 Firewall GUI are enhanced as follows.
(1) On eWCFirewallRule SummaryEdit Rule, a basic sanity check on the firewall rule is performed.
(2) On eWCFirewallRule SummaryEdit Rule, the selected service for a new rule is empty by default.
(3) On eWCFirewallRule SummaryEdit Rule, the useless headers "####### Source IP Address #######" and "#### Destination IP Address #####" are removed.
(4). On eWCFirewallRule SummaryEdit Rule, when a specific address is added to the Source/Destination Address list, the "Any" address will automatically be deleted.
(5) On eWCFirewallRule SummaryEdit Rule, the firewall action radio buttons are replaced by a dropdown list.
(6) On eWCFirewallThreshold, the "Cancel" button is replaced by "Reset" button.
(7) On eWCFirewallDeafult Rule, the wording "Default Rule Settings" is replaced by "Deafult Rule Setup".
(8) On eWCFirewallAnti-Probing, the wording "Anti-Probing Settings" is replaced by "Anti-Probing Setup".
(9) "ACCESS POLICY" is renamed as "FIREWALL".
(10) "CUSTOM PORT" is renamed as "CUSTOM SERVICE".
(11) Users can expand or collapse "Source Address", "Destination Address" and "Service Type" drop down lists by clicking the [+]/[-] icon at the beginning of each rule in Firewall Rule Summary Table.

Modifications in V3.62(XD.0)b2 | 03/26/2004

- [BUG FIX] 
Symptom: In eWCFIREWALLACCESS POLICYEDIT RULE, Action for Matched Packets can't be saved correctly. 
Condition:
(1) Go to eWCFIREWALLACCESS POLICYEDIT RULE
(2) Choose the type of Action for Matched Packets as Block, and then click Apply. 
(3) Leave this page and then re-enter this page again, Action for Matched Packets always shows Forward.

- [ENHANCEMENT]
Supports Intel TE28F640 J3C120 Flash ROM.

Modifications in V3.62(XD.0)b1 | 03/11/2004

First Release.

Appendix 1 Remote Management Enhancement (Add SNMP & DNS Control)

New function

- You can change the server port.

- You can set the security IP address for each type of server.

- You can define the rule for server access. (WAN only/LAN only, None, ALL).

- The secure IP and port of the SNMP server is read only

- The port of the SNMP and DNS server is read only.

- The default server access of the SNMP and DNS is ALL.

Modification

- The default value for Server access rule is ALL.

- Under the default setting: You can setup the Menu 15 to forwarding the server to LAN IP address. Thus you can configure the router through the WAN and you don’t need to modify the server management or filter.

Note

(1) DNS Service is not available in Bridge Mode.

Menu 24.11 - Remote Management Control TELNET Server: Port = 23 Access = ALL Secure Client IP = 0.0.0.0 FTP Server: Port = 21 Access = ALL Secure Client IP = 0.0.0.0 SSH Server: Certificate = auto_generated_self_signed_cert Port = 22 Access = ALL Secure Client IP = 0.0.0.0 HTTPS Server: Certificate = auto_generated_self_signed_cert Authenticate Client Certificates = No Port = 443 Access = ALL Secure Client IP = 0.0.0.0 HTTP Server: Port = 80 Access = ALL Secure Client IP = 0.0.0.0 SNMP Service: Port = 161 Access = ALL Secure Client IP = 0.0.0.0 DNS Service: Port = 53 Access = ALL Secure Client IP = 0.0.0.0 Press ENTER to Confirm or ESC to Cancel:

Appendix 2 Trigger Port

Introduction

Some routers try to get around this "one port per customer" limitation by using "triggered" maps.  Triggered maps work by having the router watch outgoing data for a specific port number and protocol.  When the router finds a match, it remembers the IP address of the computer that sent the matching data.  When the requested data wants to come back in through the firewall, the router uses the port mapping rules that are linked to the trigger, and the IP address of the computer that "pulled" the trigger, to get the data back to the proper computer.

These triggered events can be timed so that they erase the port mapping as soon as they are done with the data transfer, so that the port mapping can be triggered by another Client computer.  This gives the illusion that multiple computers can use the same port mapping at the same time, but the computers are really just taking turns using the mapping.

How to use it

Following table is a configuration table.

| # Name | # Incoming | # Trigger |
| --- | --- | --- |
| # Napster | # 6699 | # 6699 |
| # Quicktime 4 Client | # 6970-32000 | # 554 |
| # Real Audio | # 6970-7170 | # 7070 |
| # User | # 1001-1100 | # 1-100 |

How it works

For example, you are running a FTP Server on port 21 of machine A. And you may want this server accessible from the Internet without enabling NAT-based firewall. There are one Web Server on port 80 of machine B and another client C on the Internet.

- As Prestige receives a packet from a local client A destined for the outside Internet machine B, it will check the destination port in the TCP/UDP header to see if it matches the setting in "Trigger Port" (80). If it matches, Prestige records the source IP of A (192.168.1.33) in its internal table.

- Now client C (or client B) tries to access the FTP server in machine A. When Prestige to forward any un-requested traffic generated from Internet, it will first check the rules in port forwarding set. When no matches are found, it will then check the "Incoming Port". If it matches, Prestige will forward the packet to the recorded IP address in the internal table for this port. (This behavior is the same as we did for port forwarding.)

- The recorded IP in the internal table will be cleared if machine A disconnect from the sessions that matches the "Trigger Port".

Notes

- Trigger events can't happen on data coming from outside the firewall because the NAT router's sharing function doesn't work in that direction.

- Only one computer can use a port or port range at a time on a given real (ISP assigned) IP address.

Appendix 3 Hard-coded packet filter for "NetBIOS over TCP/IP" (NBT)

The new set C/I commands is under "sys filter netbios" sub-command. Default values of any direction are “Forward”, and trigger dial is “Disabled”.

There are two CI commands:

- "sys filter netbios disp": It will display the current filter mode.

Example ouput:

=============== NetBIOS Filter Status ===============

LAN to WAN:         	Block

WAN to LAN:         	Forward

IPSec Packets:      	Forward

Trigger Dial:       	Disabled

- "sys filter netbios config <type> {on|off}": To configure the filter mode for each type.

Current filter types and their description are:

| Type | Description | Default mode |
| --- | --- | --- |
| 0 | LAN to WAN | Forward |
| 1 | WAN to LAN | Forward |
| 6 | IPSec pass through | Forward |
| 7 | Trigger dial | Disabled |

Example commands:

sys filter netbios config 0 on	=> block LAN to WAN NBT packets

sys filter netbios config 1 on	=> block WAN to LAN NBT packets

sys filter netbios config 6 on 	=> block IPSec NBT packets

sys filter netbios config 7 off	=> disable trigger dail

Appendix 4 Traffic Redirect/Static Route Application Note

Why traffic redirect/static route be blocked by ZyWALL

ZyWALL is the ideal secure gateway for all data passing between the Internet and the LAN. For some reasons (load balance or backup line), users want traffics be re-routed to another Internet access devices while still be protected by ZyWALL. The network topology is the most important issue. Here is the common example that people misemploy the LAN traffic redirect and static route.

Figure 4-1 Triangle Route

Figure 4-1 indicates the triangle route topology. It works fine with turn off firewall. Let’s take a look into the perspective toward this situation.

- PC sends outgoing traffics through ZyWALL because default gateway assigned to it.

- Then, ZyWALL will redirect the traffics to another gateway (ISDN/Router) as we expect.

- But the return traffics do not go through ZyWALL because the gateway (say, P201) and the PC are on the same IP network. Any traffic will easily inject into the protected network area through the unprotected gateway.

- When firewall turns on, it could be worse. ZyWALL will check the outgoing traffics by ACL and create dynamic sessions to allow legal return traffics. For Anti-DoS reason, ZyWALL will send RST packets to the PC and the peer because it never received TCP SYN/ACK packet.

That causes all of outgoing TCP traffics being reset!

How traffic redirect/static route works under protection - Solutions

- Gateway on alias IP network

IP alias allows you to partition a physical network into different logical IP networks over the same Ethernet interface. The ZyWALL supports three logical LAN interfaces via its single physical Ethernet interface with the ZyWALL itself as the gateway for each LAN network. Division of protected LAN and the other gateway into different subnets will trigger the incoming traffic back to ZyWALL and it can work as normal function.

Figure 4-2 Gateway on alias IP network

- Gateway on WAN side

A working topology is suggested as below.

Figure 5-3 Gateway on WAN side

Appendix 5 IPSec FQDN support

ZyWALL A-------------Router C (with NAT) ------------ZyWALL B

(WAN)	  (WAN)			   (LAN)     (WAN)

If ZyWALL A wants to build a VPN tunnel with ZyWALL B by passing through Router C with NAT, A can not see B. It has to secure gateway as C. However, ZyWALL B will send it packet with its own IP and its ID to ZyWALL A. The IP will be NATed by Router C, but the ID will remain as ZyWALL B sent.

In FQDN design, all three types, IP, DNS, E-Mail, can set ID content. For ID type is DNS or E-mail, the behavior is simple. ZyWALL A and ZyWALL B only checks the ID contents are consistent and they can connect.

Basically the story is the same when ID type is IP. If user configures ID content, then ZyWALL will use it as a check. So the ID content also has to match each other. For example, ID type and ID content of incoming packets must match “Peer ID Type” and “Peer ID content”. Or ZyWALL will reject the connection.

However, user can leave “ID content” blank if the ID type is IP. ZyWALL will put proper value in it during IKE negotiation. This appendix describes all combinations and behaviors of ZyWALL.

We can put all combinations in to these two tables:

(Local ID Type is IP):

| Configuration |  | **Run-time status |  |
| --- | --- | --- | --- |
| My IP Addr | Local ID Content | My IP Addr | Local ID Content |
| 0.0.0.0 | *blank | My WAN IP | My WAN IP |
| 0.0.0.0 | a.b.c.d (it can be 0.0.0.0) | My WAN IP | a.b.c.d ( 0.0.0.0, if user specified it) |
| a.b.c.d (not 0.0.0.0) | *blank | a.b.c.d | a.b.c.d |
| a.b.c.d (not 0.0.0.0) | e.f.g.h (or 0.0.0.0) | a.b.c.d | e.f.g.h (or 0.0.0.0) |

*Blank: User can leave this field as empty, doesn’t put anything here.

**Runtime status: During IKE negotiation, ZyWALL will use “My IP Addr” field as source IP of IKE packets, and put “Local ID Content” in the ID payload.

(Peer ID Type is IP):

| Configuration |  | *Run-time check |
| --- | --- | --- |
| Secure Gateway Addr | Peer ID Content |  |
| 0.0.0.0 | blank | Just check ID types of incoming packet and machine’s peer ID type. If the peer’s ID is IP, then we accept it. |
| 0.0.0.0 | a.b.c.d | System checks both type and content |
| a.b.c.d | blank | 1. System will check the ID type and the content. 2. The contents will match only if the ID content of coming packet is a.b.c.d because system will put Secure Gateway Address as Peer ID content. |
| a.b.c.d | e.f.g.h | 1. System will check the ID type and the content. 2. The contents will match only if the ID content of coming packet is e.f.g.h. |

*Runtime Check: During IKE negotiation, we will check ID of incoming packet and see if it matches our setting of “Peer ID Type” and “Peer ID Content”.

Summary:

- When Local ID Content is blank which means user doesn’t type anything here, during IKE negotiation, my ID content will be “My IP Addr” (if it’s not 0.0.0.0) or local’s WAN IP.

- When “Peer ID Content” is not blank, ID of incoming packet has to match our setting. Or the connection request will be rejected.

- When “Secure Gateway IP Addr” is 0.0.0.0 and “Peer ID Content” is blank, system can only check ID type. This is a kind of “dynamic rule” which means it accepts incoming request from any IP, and these requests’ ID type is IP. So if user put a such kind of rule in top of rule list, it may be matched first. To avoid this problem, we will enhance it in the future.

Appendix 6 Embedded HTTPS proxy server

HTTPS (Hypertext Transfer Protocol over Secure Socket Layer, or HTTP over SSL) is a Web protocol developed by Netscape and built into its browser that encrypts and decrypts user page requests as well as the pages that are returned by the Web server. HTTPS is really just the use of Netscape's Secure Socket Layer (SSL) as a sublayer under its regular HTTP application layering.

The ZyWALL’s embedded HTTPS proxy server is basically an SSL server which performs SSL transactions, on behalf of the embedded HTTP server, with an SSL client such as MSIE or Netscape. As depicted by the figure below, when receiving a secure HTTPS request from an SSL-aware Web browser, the HTTPS proxy server converts it into a non-secure HTTP request and sends it to the HTTP server. On the other hand, when receiving a non-secure HTTP response from the HTTP server, the HTTPS proxy server converts it into a secure HTTPS response and sends it to the SSL-aware Web browser.

By default, the HTTPS proxy server listens on port 443 instead of the HTTP default port 80. If the ZyWALL’s HTTPS proxy server port is changed to a different number, say 8443, then the URL for accessing the ZyWALL’s Web user interface should be changed to https://hostname:8443/ accordingly.

Appendix 7 Wi-Fi Protected Access

Wi-Fi Protected Access(WPA) is a subset of the IEEE 802.11i.  WPA improves data encryption by using TKIP, MIC and IEEE 802.1X. Because WPA applies 802.1X to authenticate WLAN users by using an external RADIUS server, so you can not use the Local User Database for WPA authentication.

For those users in home or small office, they have no RADIUS server, WPA provides the benefit of WPA through the simple “WPA-PSK”. Pre-Shared Key(PSK) is manually entered in the client and ZyWALL for authentication.  ZyWALL will check the client PSK and allow it join the network if it’s PSK is matched.  After the client pass the authentication, ZyWALL will derived and distribute key to the client, and both of then will use TKIP process to encrypt exchanging data.

Appendix 8 IPSec IP Overlap Support

##### Figure 1

The ZyWALL uses the network policy to decide if the traffic matches a VPN rule. But if the ZyWALL finds that the traffic whose local address overlaps with the remote address range, it will be confused if it needs to trigger the VPN tunnel or just route this packet.

So we provide a CI command “ipsec swSkipOverlapIp” to trigger the VPN rule. For example, you configure a VPN rule on the ZyWALL A as below:

Local IP Address Start= 1.1.1.1      End= 1.1.2.254

Remote IP Address Start= 1.1.2.240   End = 1.1.2.254

You can see that the Local IP Address and the remote IP address overlap in the range from 1.1.2.240 to 1.1.2.254.

- Enter “ipsec swSkipOverlapIp off”:

To trigger the tunnel for packets from 1.1.1.33 to 1.1.2.250. If there is traffic from LAN to IP Alias (Like the traffic from PCA to PCB in Figure 1), the traffic still will be encrypted as VPN traffic and routed to WAN, you will find their traffic disappears on LAN.

- Enter “ipsec swSkipOverlapIp on”:

Not to trigger the tunnel for packets from 1.1.1.33 to 1.1.2.250. Even the tunnel has been built up, the traffic in this overlapped range still cannot be passed.

[Note]

If you configure a rule on the ZyWALL A whose

Local IP Address Start= 0.0.0.0

Remote IP Address Start= 1.1.2.240 End = 1.1.2.254

No matter swSkipOverlapIp is on or off, any traffic from any interfaces on the ZyWALL A will match the tunnel. Thus swSkipOverlapIp is not applicable in this case.

Appendix 9 VPN Local IP Address Limitation

Figure 1

There is a limitation when you configure the VPN network policy to use any Local IP address. When you set the Local address to 0.0.0.0 and the Remote address to include any interface IP of the ZyWALL at the same time, it may cause the traffic related to remote management or DHCP between PCs and the ZyWALL to work incorrectly. This is because the traffic will all be encrypted and sent to WAN.

For example, you configure a VPN rule on the ZyWALL A as below:

Local IP Address Start= 1.1.1.1      End= 1.1.2.254

Remote IP Address Start= 1.1.2.240  End = 1.1.2.254

ZyWALL LAN IP = 1.1.1.10

ZyWALL LAN IP falls into the Local Address of this rule, when you want to manage the ZyWALL A from PCA, you will find that you cannot get a DHCP Client IP from the ZyWALL anymore. Even if you set your IP on PCA as static one, you cannot access the ZyWALL.

Appendix 10 VPN rule swap limitation with VPN Client on XAuth

Example  1:
	ZyWALL (WAN)------------------- VPN Client

(IP:1.1.1.1)                     (IP:1.1.1.2)

| ZyWALL VPN Rule: Two IKE rule |  |
| --- | --- |
| - Dynamic IKE rule: Security Gateway: 0.0.0.0 X-Auth: Server - Policy one: - Name: “Rule_A” - Local: 192.168.2.0/24 - Remote: 0.0.0.0 | - Static IKE rule: Security Gateway: 1.1.1.2 X-Auth: None - Policy one: - Name: “Rule_B” - Local: 192.168.1.0/24 - Remote: 1.1.1.2/32 |

ZyXEL VPN Client

Security Gateway: 1.1.1.1 Phase one Authentication method: Preshare Key Remote: 192.168.1.0/24

In example 1, user may wonder why ZyWALL swap to dynamic rule even VPN client only set authentication method as “Preshare Key” not “Preshare Key+XAuth”. The root cause is that currently ZyXEL VPN Cient will send XAuth VID no matter what authentication mode that him set. Because of the XAuth VID, ZyWALL will swap to dynamic rule.

This unexpected rule swap result is a limitation of our design. For ZyWALL, when we got initiator’s XAuth VID in IKE Phase One period, we know initiator can support XAuth. To take account of security, we will judge that initiator want to do XAuth, and we will search one matched IKE Phase One rule with XAuth server mode as the top priority. To our rule swap scheme, we search static rule first then dynamic rule. In example 1, we will find the static rule, named “Rule_B”, to build phase one tunnel at first. After finished IKE phase one negotiation, we known initiator want to do XAuth. Since Rule_B has no XAuth server mode, we try to search another rule with correct IKE Phase One parameter and XAuth server mode. The search result will lead us to swap rule to dynamic rule, named “Rule_A”. Thus to build VPN tunnel will fail by Phase Two local ip mismatch.

To avoid this scenario, the short-term solution is that we recommend user to set two IKE rule with different Phase One parameter. The long-term solution is that VPN Client needs to modify the XAuth VID behavior. VPN Client should not send XAuth VID when authentication method is “Preshare key”, but send XAuth VID when authentication method is “Preshare key+XAuth”.

Appendix 11 The mechanism of Gratuitous ARP in the ZyWALL

In the past, if the ZyWALL gets a gratuitous ARP it will not update the sender's MAC mapping into its ARP table. In current design, if you turn on 'ip arp ackGratuitous active yes', the ZyWALL will response such packet depends on two case: 'ip arp ackGratuitous forceUpdate on' or 'ip arp ackGratuitous forceUpdate off'. if you turn on forceUpdate, then the ZyWALL gets gratuitous ARP, it will force to update MAC mapping into the ARP table, otherwise if turn off forceUpdate, then the ZyWALL gets gratuitous ARP, it will update MAC mapping into the ARP table only when there is no such MAC mapping in the ARP table.

Give an example for its purpose, there is a backup gateway on the network as the picture. One day, the gateway shuts down and the backup gateway is up, the backup gateway is set a static IP as original gateway's IP, it will broadcast a gratuitous ARP to ask who is using this IP. If ackGratuitous is on, the ZyWALL receive the gratuitous ARP from the backup gateway, it will also send an ARP request to ask who is using this IP. Once the ZyWALL gets a reply from backup gateway, it will update its ARP table so that the ZyWALL can keep a correct gateway ARP entry to forward packets. If ackGratuitous is off, the ZyWALL will not keep a correct gateway ARP entry to forward packets.

There is one thing need to be noticed: update the ARP entry might still have dangers more or less if there is a spoofing attack. So we suggest if you have no opportunity to meet the problem, you can turn off ackGratuitous. forceUpdate on will be more dangerous than forceUpdate off because it update ARP table even when ARP entry is existing.

Appendix 12 The mechanism when the ZyWALL receives a IKE packets with IC

[RFC 2407]The INITIAL-CONTACT(IC) status message may be used when one side wishes to inform the other that this is the first SA being established with the remote system. The receiver of this Notification Message might then elect to delete any existing SA's it has for the sending system under the assumption that the sending system has rebooted and no longer has access to the original SA's and their associated keying material.

The ZyWALL has two ways to delete SA when it receives IC, it is switched by a global option 'ipsec initContactMode gateway/tunnel':

(1)ipsec initContactMode gateway

When the ZyWALL receives a IKE packets with IC, it deletes all tunnels with the same secure gateway IP. It is default option because the ZyWALL is site to site VPN device. Take the picture 1 as example, there are three VPN tunnels are created between ZWA and ZWB, but ZWA reboots for some reasons, and after rebooting, the ZWA will send a IKE with IC to the ZWB, then the ZWB will delete all existing tunnels whose security gateway IP is the same as this IKE's one and build a new VPN tunnel for the sender.

(2)ipsec initContactMode tunnel
    When the ZyWALL receives a IKE packets with IC, it deletes only one existing tunnel, whose security gateway IP is not only the same as this IKE's one and also its phase 2 ID(network policy) should match. It is suitable when your tunnel is created from a VPN peer to ZyWALL and there are more than two this kind of VPN peers build tunnels behind the same NAT router. Take the picture 2 as example, PC 1, PC2 and PC3 has it's own VPN software to create tunnels with ZW. Suppose that the PC1, PC2 and PC3 separately create different tunnels with ZW for the traffic to PC4, PC5 and PC6, once the PC1 reboots for some reasons, and after rebooting, the PC1 sends a IKE with IC to the ZWB, then the ZWB will only delete the tunnel which is used by PC1 and PC4 and build a new VPN tunnel for it. So other tunnels will not be disconnected.

Appendix 13 SIP NAT Traversal

Previously, the ZyWALL supports most of SIP topologies except:

- SIP server on the ZyWALL’s LAN/DMZ/WLAN.

- Two SIP clients behind the ZyWALL and talk to each other.

Now we have solved these two problems, all directions of SIP calls can work. You can refer to the Figure 1, all of the SIP clients in the picture can register to the SIP server behind the ZyWALL and any two SIP clients can talk to each other.

Figure 1.

But there are still some limitations remain that we need to overcome in the future. When you deploy your SIP server on LAN for SIP service, please make sure that prevent your topology from any case listed as below.

- When SIP client is on LAN, do not use NAT lookback on SIP server.

When there is a SIP server on the LAN, for those SIP clients on WAN, we can set a port forwarding rule or address mapping rule to let them to use WAN IP to access the SIP server behind the ZyWALL, but for those SIP clients which is behind the ZyWALL, please just use the SIP server’s LAN IP and DON’T use the public IP as their SIP server IP, the ZyWALL doesn’t support such a loopback case on SIP registration/proxy server.

For instance, in Figure 2, there is a SIP server on LAN, and there are also two SIP phones E and F on LAN want to talk to each other. Although there is a NAT port forwarding rule for outside SIP clients to use 211.72.158.200 to connect to SIP server, but please let phone E and F use SIP server’s LAN IP 192.168.1.200 to connect to SIP server directly.

Figure 2.

- Try not use different global IPs for SIP client and SIP server on NAT.

Currently, there are still some limitations when use different global IPs for SIP client and SIP server. For instance, in Figure 3, SIP server and a SIP client B are on the same LAN. If we use different global IP for SIP server and the SIP client, the SIP client A which is behind another NAT router will fail to communication with SIP client B.

Figure 3.

- We do not support that SIP client sends ACK directly to a peer client.

For instance, in Figure 4, when SIP phone A want to send ACK request direct to SIP phone B, because of the limitation, this ACK request will not successfully transmit to SIP phone B. Thus will be fail on call setup. This limitation is SIP client related issue, some SIP clients will send ACK request direct to the remote clients, some may send through proxy server.

Figure 4.

- We do not support multiple SIP proxies in the middle of way.

We haven’t implemented or take care on this kind topology (Figure 5), so the result is still unknown.

Figure 5

Annex A CI Command List

Last Updated: 2006/04/18

| Command Class List Table |  |  |
| --- | --- | --- |
| System Related Command | Exit Command | Device Related Command |
| Ethernet Related Command | POE Related Command | PPTP Related Command |
| AUX Related Command | Configuration Related Command | IP Related Command |
| IPSec Related Command | PPP Related Command | Bandwidth Management |
| Firewall Related Command | Certificate Management (PKI) Command | Load Sharing Command |
| Bridge Related Command | myZyXEL.com Command | Anti-Spam Command |
| IDP Command | Anti-Virus Command |  |

System Related Command								Home

| Command |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| sys |  |  |  |  |  |
|  | atsh |  |  |  | Display system information |
|  | cbuf |  |  |  |  |
|  |  | display | [a¦f¦u] |  | display cbuf a: all f: free u: used |
|  |  | cnt |  |  | cbuf static |
|  |  |  | Display |  | display cbuf static |
|  |  |  | Clear |  | clear cbuf static |
|  | baud |  | <1..5> |  | change console speed |
|  | callhist |  |  |  |  |
|  |  | display |  |  | display call history |
|  |  | remove | <index> |  | remove entry from call history |
|  | clear |  |  |  | clear the counters in GUI status menu |
|  | countrycode |  | [countrycode] |  | set country code |
|  | datetime |  | [year month date] |  | set/display date |
|  | domainname |  |  |  | display domain name |
|  | edit |  | <filename> |  | edit a text file |
|  | enhanced |  |  |  | return OK if commands are supported for PWC purposes |
|  | errctl |  | [level] |  | set the error control level 0:crash no save,not in debug mode (default) 1:crash no save,in debug mode 2:crash save,not in debug mode 3:crash save,in debug mode |
|  | event |  |  |  |  |
|  |  | display |  |  | display tag flags information |
|  |  | trace |  |  | display system event information |
|  |  |  | display |  | display trace event |
|  |  |  | clear <num> |  | clear trace event |
|  | extraphnum |  |  |  | maintain extra phone numbers for outcalls |
|  |  | add | <set 1-3> <1st phone num> [2nd phone num] |  | add extra phone numbers |
|  |  | display |  |  | display extra phone numbers |
|  |  | node | <num> |  | set all extend phone number to remote node <num> |
|  |  | remove | <set 1-3> |  | remove extra phone numbers |
|  |  | reset |  |  | reset flag and mask |
|  | feature |  |  |  | display feature bit |
|  | fid |  |  |  |  |
|  |  | display |  |  | display function id list |
|  | firmware |  |  |  | display ISDN firmware type |
|  | hostname |  | [hostname] |  | display system hostname |
|  | iface |  |  |  |  |
|  |  | disp | [#] |  | display iface list |
|  | interrupt |  |  |  | display interrupt status |
|  | logs |  |  |  |  |
|  |  | category |  |  |  |
|  |  |  | access [0:none/1:log/2:alert/3:both] |  | record the access control logs |
|  |  |  | attack [0:none/1:log/2:alert/3:both] |  | record and alert the firewall attack logs |
|  |  |  | display |  | display the category setting |
|  |  |  | error [0:none/1:log/2:alert/3:both] |  | record and alert the system error logs |
|  |  |  | ipsec [0:none/1:log/2:alert/3:both] |  | record the access control logs |
|  |  |  | ike [0:none/1:log/2:alert/3:both] |  | record the access control logs |
|  |  |  | javablocked [0:none/1:log] |  | record the java etc. blocked logs |
|  |  |  | mten [0:none/1:log] |  | record the system maintenance logs |
|  |  |  | packetfilter [0:none/1:log] |  | record the packet filter logs |
|  |  |  | pki [0:none/1:log/2:alert/3:both] |  | record the pki logs |
|  |  |  | tcpreset [0:none/1:log] |  | record the tcp reset logs |
|  |  |  | upnp [0:none/1:log] |  | record upnp logs |
|  |  |  | urlblocked [0:none/1:log/2:alert/3:both] |  | record and alert the web blocked logs |
|  |  |  | urlforward [0:none/1:log] |  | record web forward logs |
|  |  | clear |  |  | clear log |
|  |  | display | [access¦attack¦error¦ipsec¦ike¦javablocked¦mten¦packetfilter¦pki¦tcpreset¦urlblocked¦urlforward] |  | display all logs or specify category logs |
|  |  | dispSvrIP |  |  | Display the IP address of email log server and syslog server |
|  |  | errlog |  |  |  |
|  |  |  | clear |  | display log error |
|  |  |  | disp |  | clear log error |
|  |  |  | online |  | turn on/off error log online display |
|  |  | load |  |  | load the log setting buffer |
|  |  | mail |  |  |  |
|  |  |  | alertAddr [mail address] |  | send alerts to this mail address |
|  |  |  | display |  | display mail setting |
|  |  |  | logAddr [mail address] |  | send logs to this mail address |
|  |  |  | schedule display |  | display mail schedule |
|  |  |  | schedule hour [0-23] |  | hour time to send the logs |
|  |  |  | schedule minute [0-59] |  | minute time to send the logs |
|  |  |  | schedule policy [0:full/1:hourly/2:daily/3:weekly/4:none] |  | mail schedule policy |
|  |  |  | schedule week [0:sun/1:mon/2:tue/3:wed/4:thu/5:fri/6:sat] |  | weekly time to send the logs |
|  |  |  | server [domainName/IP] |  | mail server to send the logs |
|  |  |  | subject [mail subject] |  | mail subject |
|  |  | save |  |  | save the log setting buffer |
|  |  | syslog |  |  |  |
|  |  |  | active [0:no/1:yes] |  | active to enable unix syslog |
|  |  |  | display |  | display syslog setting |
|  |  |  | facility [Local ID(1-7)] |  | log the messages to different files |
|  |  |  | server [domainName/IP] |  | syslog server to send the logs |
|  |  | updateSvrIP | <minute> |  | If there is one parameter <minute>, it will change the dns timer task timeout value. Otherwise, do dns resolve to find email log server and syslog server IP. |
|  |  | consolidate |  |  |  |
|  |  |  | switch <0:on¦1:off> |  | active to enable log consolidation |
|  |  |  | period |  | consolidation period (seconds) |
|  |  |  | msglist |  | display the consolidated messages |
|  |  | switch |  |  |  |
|  |  |  | bmlog <0:no¦1:yes> |  | active to enable broadcast/multicast log |
|  |  |  | display |  | display switch setting |
|  |  |  | trilog <0:no¦1:yes> |  | active to enable triangle route log |
|  |  | lastAlert | <index> |  | display the last #index alert in the centralized log. |
|  | mbuf |  |  |  |  |
|  |  | link | link |  | list system mbuf link |
|  |  | pool | <id> [type][num] |  | list system mbuf pool |
|  |  | status |  |  | display system mbuf status |
|  |  | disp | <address>[1¦0] |  | display mbuf status |
|  |  | cnt |  |  |  |
|  |  |  | disp |  | display system mbuf count |
|  |  |  | clear |  | clear system mbuf count |
|  |  | debug | [on¦off] |  |  |
|  | md5 |  | <string> |  | This command will hash the string by MD5. Tha maximum length of the string is 64. |
|  | memwrite |  | <address> <len> [data list ...] |  | write some data to memory at <address> |
|  | memutil |  |  |  |  |
|  |  | usage |  |  | display memory allocate and heap status |
|  |  | mqueue | <address> <len> |  | display memory queues |
|  |  | mcell | mid [f¦u] |  | display memory cells by given ID |
|  |  | msecs | [a¦f¦u] |  | display memory sections |
|  |  | mtstart | <n-mcell> |  | start memory test |
|  |  | mtstop |  |  | stop memory test |
|  |  | mtalloc | <size> [n-mcell] |  | allocate memory for testing |
|  |  | mtfree | <start-idx> [end-idx] |  | free the test memory |
|  | mode | <router/bridge> |  |  | switch router and bridge mode |
|  | model |  |  |  | display server model name |
|  | mwan |  |  |  |  |
|  |  | load |  |  | Load the multiple wan common data to the memory |
|  |  | mode | <0:Active/Passive¦1:Active/Active> |  | Change the Multiple WAN operation mode. |
|  |  | Save |  |  | Save the configuration |
|  |  | Disp |  |  | Display the data |
|  | ProbeType |  | [icmp ¦ arp] |  | DHCP server probing type |
|  | proc |  |  |  |  |
|  |  | display |  |  | Display all process information. State: process state. Pri: priority, a_usg: accumulated cpu usage, p_usg: profiling cpi usage.(take count after do clear command). Size: (lowest available stack size)/(total stack size). |
|  |  | stack | [tag] |  | display process's stack by a give TAG |
|  |  | pstatus |  |  | display process's status by a give TAG |
|  |  | clear |  |  | Restart cpu usage measurement. (Result will be in p_usg column from display command. |
|  | pwc |  |  |  | sends information to PWC via telnet |
|  | pwdHash |  | <on ¦ off> [newPassword] [oldPassword] |  | The password saved in ROM file can be hashed by MD5. |
|  | queue |  |  |  |  |
|  |  | display | [a¦f¦u] [start#] [end#] |  | display queue by given status and range numbers |
|  |  | ndisp | [qid] |  | display a queue by a given number |
|  | quit |  |  |  | quit CI command mode |
|  | reboot |  | [code] |  | reboot system code = 0 cold boot, = 1 immediately boot = 2 bootModule debug mode |
|  | reslog |  |  |  |  |
|  |  | disp |  |  | display resources trace |
|  |  | clear |  |  | clear resources trace |
|  | rn |  |  |  |  |
|  |  | load | <entry no.> |  | load remote node information |
|  |  | disp | <entry no.>(0:working buffer) |  | display remote node information |
|  |  | nat | <none¦sua¦full_feature> |  | config remote node nat |
|  |  | nailup | <no¦yes> |  | config remote node nailup |
|  |  | mtu | <value> |  | set remote node mtu |
|  |  | accessblock | [1¦0] |  | Enable/disable block specific remote note packet. |
|  |  | trigger | [on¦off] |  |  |
|  |  | save | [entry no.] |  | save remote node information |
|  | smt |  |  |  | not support in this product |
|  | stdio |  | [second] |  | change terminal timeout value |
|  | time |  | [hour [min [sec]]] |  | display/set system time |
|  | timer |  |  |  |  |
|  |  | disp |  |  | display timer cell |
|  | tos |  |  |  |  |
|  |  | display |  |  | display all runtime TOS |
|  |  | listPerHost |  |  | display all host session count |
|  |  | debug | [on¦off] |  | turn on or off TOS debug message |
|  |  | sessPerHost | <number> |  | configure session per host value |
|  |  | timeout |  |  |  |
|  |  |  | display |  | display all TOS timeout information |
|  |  |  | icmp <idle timeout> |  | set idle timeout value |
|  |  |  | igmp <idle timeout> |  | set idle timeout value |
|  |  |  | tcpsyn <idle timeout> |  | set idle timeout value |
|  |  |  | tcp <idle timeout> |  | set idle timeout value |
|  |  |  | tcpfin <idle timeout> |  | set idle timeout value |
|  |  |  | udp <idle timeout> |  | set idle timeout value |
|  |  |  | gre <idle timeout> |  | set idle timeout value |
|  |  |  | esp <idle timeout> |  | set idle timeout value |
|  |  |  | ah <idle timeout> |  | set idle timeout value |
|  |  |  | other <idle timeout> |  | set idle timeout value |
|  |  | tempTOSDisplay |  |  | display temporal TOS records. |
|  |  | tempTOSTimeout | [timeout value] |  | set/display temporal timeout value |
|  | trcdisp | parse, brief, disp |  |  | monitor packets |
|  | trclog |  |  |  |  |
|  |  | switch | [on¦off] |  | set system trace log |
|  |  | online | [on¦off] |  | set on/off trace log online |
|  |  | level | [level] |  | set trace level of trace log #:1-10 |
|  |  | type | <bitmap> |  | set trace type of trace log |
|  |  | disp |  |  | display trace log |
|  |  | clear |  |  | clear trace |
|  |  | call |  |  | display call event |
|  |  | encapmask | [mask] |  | set/display tracelog encapsulation mask |
|  | trcpacket |  |  |  |  |
|  |  | create | <entry> <size> |  | create packet trace buffer |
|  |  | destroy |  |  | packet trace related commands |
|  |  | channel | <name> [none¦incoming¦outgoing¦bothway] |  | <channel name>=enet0,sdsl00, fr0 set packet trace direction for a given channel |
|  |  | string |  |  | enable smt trace log |
|  |  | switch | [on¦off] |  | turn on/off the packet trace |
|  |  | disp |  |  | display packet trace |
|  |  | udp |  |  | send packet trace to other system |
|  |  |  | switch [on¦off] |  | set tracepacket upd switch |
|  |  |  | addr <addr> |  | send trace packet to remote udp address |
|  |  |  | port <port> |  | set tracepacket udp port |
|  |  | parse | [[start_idx], end_idx] |  | parse packet content |
|  |  | brief |  |  | display packet content briefly |
|  | syslog |  |  |  |  |
|  |  | server | [destIP] |  | set syslog server IP address |
|  |  | facility | <FacilityNo> |  | set syslog facility |
|  |  | type | [type] |  | set/display syslog type flag |
|  |  | mode | [on¦off] |  | set syslog mode |
|  | version |  |  |  | display RAS code and driver version |
|  | view |  | <filename> |  | view a text file |
|  | wdog |  |  |  |  |
|  |  | switch | [on¦off] |  | set on/off wdog |
|  |  | cnt | [value] |  | display watchdog counts value: 0-34463 |
|  | romreset |  |  |  | restore default romfile |
|  | pwderrtm |  | [minute] |  | Set or display the password error blocking timeout value. |
|  | mrd |  |  |  |  |
|  |  | atwe | <mac> [country code] [debug flag] [featurebit] |  | configure mac, country code, debug flag, featurebit in the boot module |
|  |  | atse |  |  | generate the engeneering debug flag password seed |
|  |  | aten | <password> |  | enter the engeneering debug flag password |
|  |  | atfl | <0:1> |  | set engeneering debug flag |
|  |  | atsh |  |  | show mrd setting |
|  | server |  |  |  |  |
|  |  | access | <telnet¦ftp¦web¦icmp¦snmp¦dns> <value> |  | set server access type |
|  |  | load |  |  | load server information |
|  |  | disp |  |  | display server information |
|  |  | port | <telnet¦ftp¦web¦snmp> <port> |  | set server port |
|  |  | save |  |  | save server information |
|  |  | secureip | <telnet¦ftp¦web¦icmp¦snmp¦dns> <ip> |  | set server secure ip addr |
|  |  | certificate | <https¦ssh> [certificate name] |  | set server certificate |
|  |  | auth_client | <https> [on¦off] |  | specifies whether the server authenticates the client |
|  | fwnotify |  |  |  |  |
|  |  | load |  |  | load fwnotify entry from spt |
|  |  | save |  |  | save fwnotify entry to spt |
|  |  | url | <url> |  | set fwnotify url |
|  |  | days | <days> |  | set fwnotify days |
|  |  | active | <flag> |  | turn on/off fwnotify flag |
|  |  | disp |  |  | display firmware notify information |
|  |  | check |  |  | check firmware notify event |
|  |  | debug | <flag> |  | turn on/off firmware notify debug flag |
|  | spt |  |  |  |  |
|  |  | dump |  |  | dump spt raw data |
|  |  |  | root |  | dump spt root data |
|  |  |  | rn |  | dump spt remote node data |
|  |  |  | user |  | dump spt user data |
|  |  |  | slot |  | dump spt slot data |
|  |  | set | <offset> <len> <value...> |  | set spt value in memory address |
|  |  | save |  |  | save spt data |
|  |  | size |  |  | display spt record size |
|  |  | clear |  |  | clear spt data |
|  | cmgr |  |  |  |  |
|  |  | trace |  |  |  |
|  |  |  | disp <ch-name> |  | show the connection trace of this channel |
|  |  |  | clear <ch-name> |  | clear the connection trace of this channel |
|  |  | data | <ch-name> |  | show channel connection related data |
|  |  | cnt | <ch-name> |  | show channel connection related counter |
|  | socket |  |  |  | display system socket information |
|  | filter |  |  |  |  |
|  |  | clear |  |  | clear filter statistic counter |
|  |  | disp |  |  | display filter statistic counters |
|  |  | sw | [on¦off] |  | set filter status switch |
|  |  | rule | <iface> |  | display iface filter flag |
|  |  | set | <set> |  | display filter rule |
|  |  | addNetBios |  |  | add netbios filter |
|  |  | removeNetBios |  |  | remove netbios filter |
|  |  | netbios |  |  |  |
|  |  |  | disp |  | display netbios filter status |
|  |  |  | config <0:Between LAN and WAN, 1: Between LAN and DMZ, 2: Between WAN and DMZ, 3:IPSec passthrough, 4:Trigger Dial> <on¦off> |  | config netbios filter |
|  |  | blockbc | [on¦off] |  | set/display broadcast filter mode |
|  | roadrunner |  |  |  |  |
|  |  | debug | <level> |  | enable/disable roadrunner service 0: diable <default> 1: enable |
|  |  | display | <iface name> |  | display roadrunner information iface-name: enif0, wanif0 |
|  |  | restart | <iface name> |  | restart roadrunner |
|  |  | logout | <iface name> |  | logout roadrunner |
|  |  | set | <iface name> |  | set roadrunner |
|  | ddns |  |  |  |  |
|  |  | debug | <level> |  | enable/disable ddns service |
|  |  | display | <iface name> |  | display ddns information |
|  |  | restart | <iface name> |  | restart ddns |
|  |  | logout | <iface name> |  | logout ddns |
|  | cpu |  |  |  |  |
|  |  | display |  |  | display CPU utilization |
|  | upnp |  |  |  |  |
|  |  | active | [0:no/1:yes] |  | Activate or deactivate the saved upnp settings |
|  |  | config | [0:deny/1:permit] |  | Allow users to make configuration changes. through UPnP |
|  |  | display |  |  | display upnp information |
|  |  | firewall | [0:deny/1:pass] |  | Allow UPnP to pass through Firewall. |
|  |  | load |  |  | save upnp information |
|  |  | reserve | [0:no/1:yes] |  | Reserve UPnP NAT rules in flash after system bootup. |
|  |  | save |  |  | save upnp information |
|  | threatReport |  |  |  |  |
|  |  | idp |  |  |  |
|  |  |  | active |  | Active/inactive threat report functionality for IDP |
|  |  |  | dump |  | Dump all entry in memory |
|  |  |  | flush |  | Flush all data and update time stamp |
|  |  |  | summary |  | Show summary |
|  |  |  | statistic | id | Show top N statistic records for id field |
|  |  |  | statistic | src | Show top N statistic records for source IP field |
|  |  |  | statistic | dst | Show top N statistic records for destination IP field |
|  |  | av |  |  |  |
|  |  |  | active |  | Active/inactive URM report functionality for AV |
|  |  |  | dump |  | Dump all entry in memory |
|  |  |  | flush |  | Flush all data and update time stamp |
|  |  |  | summary |  | Show summary |
|  |  |  | statistic | id | Show top N statistic records for id field |
|  |  |  | statistic | src | Show top N statistic records for source IP field |
|  |  |  | statistic | dst | Show top N statistic records for destination IP field |
|  |  | as |  |  |  |
|  |  |  | active |  | Active/inactive threat report functionality for AS |
|  |  |  | dump |  | Dump all entry in memory |
|  |  |  | flush |  | Flush all data and update time stamp |
|  |  |  | summary |  | Show summary |
|  |  |  | statistic | sender | Show top N statistic records for sender mail address field |
|  |  |  | statistic | src | Show top N statistic records for source IP field |
|  |  |  | statistic | score | Show score distribution for AS |
|  | atmu |  |  |  | Show multiboot client version |

Exit Command										Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| exit |  |  |  | exit smt menu |

Device Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| dev |  |  |  |  |
|  | channel |  |  |  |
|  |  | name | <all¦use> | list channel name |
|  |  | drop | <channel_name> | drop channel |
|  |  | disp | <channel_name> [level] | display channel |
|  |  | threshold | <channel_name> [number] | set channel threshold |
|  | dial |  | <node#> | dial to remote node |

Ethernet Related Command								Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| ether |  |  |  |  |
|  | config |  |  | display LAN configuration information |
|  | driver |  |  |  |
|  |  | cnt |  |  |
|  |  |  | disp <name> | display ether driver counters |
|  |  |  | clear <name> | clear ether driver counters |
|  |  | iface | <ch_name> <num> | send driver iface |
|  |  | ioctl | <ch_name> | Useless in this stage. |
|  |  | mac | <ch_name> <mac_addr> | Set LAN Mac address |
|  |  | reg | <ch_name> | display LAN hardware related registers |
|  |  | rxmod | <ch_name> <mode> | set LAN receive mode. mode: 1: turn off receiving 2: receive only packets of this interface 3: mode 2+ broadcast 5: mode 2 + multicast 6: all packets |
|  |  | status | <ch_name> | see LAN status |
|  |  | init | <ch_name> | initialize LAN |
|  | version |  |  | see ethernet device type |
|  | pkttest |  |  |  |
|  |  | disp |  |  |
|  |  |  | packet <level> | set ether test packet display level |
|  |  |  | event <ch> [on¦off] | turn on/off ether test event display |
|  |  | sap | [ch_name] | send sap packet |
|  |  | arp | <ch_name> <ip-addr> | send arp packet to ip-addr |
|  | test |  | <ch_id> <test_id> [arg3] [arg4] | do LAN test |
|  | ipmul |  | <num> | only receive ip multicast and broadcast packet |
|  | pncconfig |  | <ch_name> | do pnc config |
|  | mac |  | <src_ch> <dest_ch> <ipaddr> | fake mac address |
|  | edit |  |  |  |
|  |  | load | <ether no.> | load ether data from spt |
|  |  | mtu | <value> | set ether data mtu |
|  |  | speed | <speed> | set ether data speed |
|  |  | save |  | save ether data to spt |
|  | dynamicPort |  |  |  |
|  |  | dump |  | display the relation between physical port and channel. |
|  |  | set | <port> <type> | set physical port belongs to which channel. |
|  |  | spt |  | display channel setting stored in SPT. |

POE Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| poe |  |  |  |  |
|  | debug |  | [on¦off] | switch poe debug |
|  | retry |  |  |  |
|  |  | count | [count] | set/display poe retry count |
|  |  | interval | [interval] | set/display poe retry interval |
|  | status |  | [ch_name] | see poe status |
|  | master |  |  |  |
|  |  | promiscuous | [on¦off] | provide pppoe server list to client |
|  |  | easy | [on¦off] | response for no service name request |
|  | service |  |  |  |
|  |  | add | <service-name> | add poe service |
|  |  | show |  | show poe service |
|  | dial |  | <node> | dial a remote node |
|  | drop |  | <node> | drop a pppoe call |
|  | channel |  |  |  |
|  |  | enable | <channel> | enable a channel to carry pppoe traffic |
|  |  | disable | <channel> | disable a pppoe channel |
|  |  | show |  | show pppoe channel |
|  | padt |  | [limit] | set/display pppoe PADT limit |
|  | inout |  | <node_name> | set call direction to both |
|  | ippool |  | [ip] [cnt] | set/display pppoe ippool information |
|  | ether |  | [rfc¦3com] | set /display pppoe ether type |
|  | proxy | disp |  | Display PPPoE proxy client session table |
|  |  | active | [on ¦ off] | Turn on / off PPPoE proxy function |
|  |  | debug | [on ¦ off] | Turn on / off PPPoE proxy debug function |
|  |  | time | <interval> | Set the time out interval, it’s a count. Actual time is count * 5 seconds. |
|  |  | init |  | Initialize PPPoE proxy client session table |
|  |  | flush |  | Clear PPPoE proxy client session table |

PPTP Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| pptp |  |  |  |  |
|  | debug |  | [on¦off] | switch pptp debug flag |
|  | dial |  | <rn-name> | dial a remote node |
|  | drop |  | <rn-name> | drop a remote node call |
|  | tunnel |  | <tunnel id> | display pptp tunnel information |
|  | enque |  | [size] | set pptp max en-queued size |

AUX Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| aux |  |  |  |  |
|  | atring |  | <device name> | Command the AT command to the device. |
|  | clearstat |  | <device name> | reset channel statistics |
|  | cnt |  |  |  |
|  |  | disp | <device name> | display aux counter information |
|  |  | clear | <device name> | clear aux counter information |
|  | cond |  |  |  |
|  |  | disp | <device name> | display aux condition information |
|  |  | clear | <device name> | clear aux condition information |
|  | config |  |  | display aux config, board, line, channel information |
|  | data |  |  |  |
|  | drop |  | <device name> | disconnect |
|  | event |  |  |  |
|  |  | disp |  | aux event trace display |
|  |  | clear |  | aux event trace clear |
|  | init |  | <device name> | initialize aux channel |
|  | mstatus |  | <device name> | display modem last call status |
|  | mtype |  | <device name> | display modem type |
|  | netstat |  | <device name> | prints upper layer packet information |
|  | rate |  | <device name> | show tx rx rate |
|  | ringbuf |  |  |  |
|  |  | cmd |  |  |
|  |  |  | clear <device name> | clear ringbuffer |
|  |  |  | disp <device name> | display ringbuffer |
|  |  | data |  |  |
|  |  |  | clear | clear command ringbuffer |
|  |  |  | disp <start> <len> | display command ringbuffer |
|  | signal |  | <device name> | show aux signal |
|  | speed |  | <device name> <type> [value] | display/set aux speed |
|  | usrmdn | flag | [1¦0] | Enable/disable USR modem capability. |

Configuration Related Command								Home

| Command |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| config |  |  |  |  | The parameters of config are listed below. |
| edit | firewall | active <yes¦no> |  |  | Activate or deactivate the saved firewall settings |
|  | custom-service <entry#> | name <string> |  |  | Configure selected custom-service with name = <string> |
|  |  | ip-protocol < icmp ¦ tcp ¦ udp ¦ tcp/udp ¦ user-defined> |  |  | Configure IP Protocol Type for selected custom-service |
|  |  | port-range <start port> <end port> |  |  | When ip-protocol = “tcp ¦ udp ¦ tcp/udp “. configure port range for custom-service entry #. For single port configuration, start port equals to end port. |
|  |  | user-defined-ip <1~65535> |  |  | When ip-protocol = “user-defined”. Configure user defined IP protocol. |
|  |  | icmp-type <0~255> |  |  | When ip-protocol = “icmp”, configure ICMP type. |
|  |  | icmp-code <0~255> |  |  | When ip-protocol = “icmp”, configure ICMP code. This field is optional for ICMP. |
| retrieve | firewall |  |  |  | Retrieve current saved firewall settings |
| save | firewall |  |  |  | Save the current firewall settings |
|  | custom-service <entry#> |  |  |  | Save the custom service entry specified by <entry#> |
|  | anti-spam |  |  |  | Save current AntiSpam settings |
|  | all |  |  |  | Save all working SPT buffer into flash. |
| display | firewall |  |  |  | Displays all the firewall settings |
|  |  | set <set#> |  |  | Display current entries of a set configuration; including timeout values, name, default-permit, and number of rules in the set. |
|  |  | set <set#> | rule <rule#> |  | Display current entries of a rule in a set. |
|  |  | attack |  |  | Display all the attack alert settings in PNC |
|  |  | e-mail |  |  | Display all the e-mail settings in PNC |
|  |  | ? |  |  | Display all the available sub commands |
|  | custom-service |  |  |  | Display all configured custom services. |
|  | custom-service <entry #> |  |  |  | Display custom service <entry #> |
|  | anti-spam |  |  |  | Display AntiSpam settings |
| edit |  | e-mail | mail-server <mail server IP> |  | Edit the mail server IP to send the alert |
|  |  |  | return-addr <e-mail address> |  | Edit the mail address for returning an email alert |
|  |  |  | e-mail-to <e-mail address> |  | Edit the mail address to send the alert |
|  |  |  | policy <full ¦ hourly ¦daily ¦ weekly> |  | Edit email schedule when log is full or per hour, day, week. |
|  |  |  | day <sunday ¦ monday ¦ tuesday ¦ wednesday ¦ thursday ¦ friday ¦ saturday> |  | Edit the day to send the log when the email policy is set to Weekly |
|  |  |  | hour <0~23> |  | Edit the hour to send the log when the email policy is set to daily or weekly |
|  |  |  | minute <0~59> |  | Edit the minute to send to log when the email policy is set to daily or weekly |
|  |  |  | Subject <mail subject> |  | Edit the email subject |
|  |  | attack | send-alert <yes¦no> |  | Activate or deactivate the firewall DoS attacks notification emails |
|  |  |  | block <yes¦no> |  | Yes: Block the traffic when exceeds the tcp-max-incomplete threshold |
|  |  |  |  |  | No: Delete the oldest half-open session when exceeds the tcp-max-incomplete threshold |
|  |  |  | block-minute <0~255> |  | Only valid when sets 'Block' to yes. The unit is minute |
|  |  |  | minute-high <0~255> |  | The threshold to start to delete the old half-opened sessions to minute-low |
|  |  |  | minute-low <0~255> |  | The threshold to stop deleting the old half-opened session |
|  |  |  | max-incomplete-high <0~255> |  | The threshold to start to delete the old half-opened sessions to max-incomplete-low |
|  |  |  | max-incomplete-low <0~255> |  | The threshold to stop deleting the half-opened session |
|  |  |  | tcp-max-incomplete <0~255> |  | The threshold to start executing the block field |
|  |  | set <set#> | name <desired name> |  | Edit the name for a set |
|  |  |  | default-permit <forward¦block> |  | Edit whether a packet is dropped or allowed when it does not match the default set |
|  |  |  | icmp-timeout <seconds> |  | Edit the timeout for an idle ICMP session before it is terminated |
|  |  |  | udp-idle-timeout <seconds> |  | Edit the timeout for an idle UDP session before it is terminated |
|  |  |  | connection-timeout <seconds> |  | Edit the wait time for the SYN TCP sessions before it is terminated |
|  |  |  | fin-wait-timeout <seconds> |  | Edit the wait time for FIN in concluding a TCP session before it is terminated |
|  |  |  | tcp-idle-timeout <seconds> |  | Edit the timeout for an idle TCP session before it is terminated |
|  |  |  | pnc <yes¦no> |  | PNC is allowed when 'yes' is set even there is a rule to block PNC |
|  |  |  | log <yes¦no> |  | Switch on/off sending the log for matching the default permit |
|  |  |  | logone <yes¦no> |  | Switch on/off for one packet that create just one log message. |
|  |  |  | rule <rule#> | permit <forward¦block> | Edit whether a packet is dropped or allowed when it matches this rule |
|  |  |  |  | active <yes¦no> | Edit whether a rule is enabled or not |
|  |  |  |  | protocol <0~255> | Edit the protocol number for a rule. 1=ICMP, 6=TCP, 17=UDP... |
|  |  |  |  | log <none¦match¦not-match¦both> | Sending a log for a rule when the packet none¦matches¦not match¦both the rule |
|  |  |  |  | alert <yes¦no> | Activate or deactivate the notification when a DoS attack occurs or there is a violation of any alert settings. In case of such instances, the function will send an email to the SMTP destination address and log an alert. |
|  |  |  |  | srcaddr-single <ip address> | Select and edit a source address of a packet which complies to this rule |
|  |  |  |  | srcaddr-subnet <ip address> <subnet mask> | Select and edit a source address and subnet mask if a packet which complies to this rule. |
|  |  |  |  | srcaddr-range <start ip address> <end ip address> | Select and edit a source address range of a packet which complies to this rule. |
|  |  |  |  | destaddr-single <ip address> | Select and edit a destination address of a packet which complies to this rule |
|  |  |  |  | destaddr-subnet <ip address> <subnet mask> | Select and edit a destination address and subnet mask if a packet which complies to this rule. |
|  |  |  |  | destaddr-range <start ip address> <end ip address> | Select and edit a destination address range of a packet which complies to this rule. |
|  |  |  |  | tcp destport-single <port#> | Select and edit the destination port of a packet which comply to this rule. For non-consecutive port numbers, the user may repeat this command line to enter the multiple port numbers. |
|  |  |  |  | tcp destport-range <start port#> <end port#> | Select and edit a destination port range of a packet which comply to this rule. |
|  |  |  |  | udp destport-single <port#> | Select and edit the destination port of a packet which comply to this rule. For non-consecutive port numbers, users may repeat this command line to enter the multiple port numbers. |
|  |  |  |  | udp destport-range <start port#> <end port#> | Select and edit a destination port range of a packet which comply to this rule. |
|  |  |  |  | desport-custom <desired custom port name> | Type in the desired custom port name |
|  |  |  |  | custom-ip <desired custom service name> | Type in the desired User Defined IP Protocol custom service. |
|  |  |  |  | custom-icmp <desired custom service name> | Type in the desired ICMP custom service |
|  | anti-spam |  |  |  |  |
|  |  | action | <0¦1> |  | Set the action for Spam Mail: add tag(0) or discard mail(1). |
|  |  | markString | <spam tag> |  | Set the Spam tag string. This tag will add to the subject of spam mail. |
|  |  | externDB | <0¦1> |  | Enable(1)/Disable(0) External Database Query. |
|  |  | query | <0¦1> |  | Set the action for no spam score: add tag(0) ot discard mail(1). |
|  |  | queryString | <no spam score tag> |  | Set the tag string for no spam score. This tag will add to the subject of spam mail. |
|  |  | threshold | <threshold> |  | Set the spam score threshold. If the spam score is higher than this threshold, this mail will be judge as spam mail. |
|  |  | switch | <0¦1> |  | Enable(1)/Disable(0) AntiSpam function. |
|  |  | whiteRule | <0¦1> |  | Enable(1)/Disable(0) AntiSpam White Rule Filter. |
|  |  | blackRule | <0¦1> |  | Enable(1)/Disable(0) AntiSpam Black Rule Filter. |
|  |  | phishingString | <Phishing tag> |  | Set the phishing tag string. This tag will add to the subject of spam mail. |
|  |  | rule | <rule number> | ip <index> active <0¦1> address <ip address> netmask <netmask> | Set the While(1)/Black(2) Rule IP Filter. The <index> is start from 0. |
|  |  |  |  | email <index> active <0¦1> data <email address> | Set the While(1)/Black(2) Rule Email Filter. The <index> is start from 0. |
|  |  |  |  | mime <index> active <0¦1> header <MIME Header> value <MIME Value> | Set the While(1)/Black(2) Rule MIME Filter. The <index> is start from 0. |
| delete | firewall | e-mail |  |  | Remove all email alert settings |
|  |  | attack |  |  | Reset all alert settings to defaults |
|  |  | set <set#> |  |  | Remove a specified set from the firewall configuration |
|  |  | set <set#> | rule <rule#> |  | Remove a specified rule in a set from the firewall configuration |
|  | anti-spam | blackRule |  |  | Remove the AntiSpam Black Rule. |
|  |  | whiteRule |  |  | Remove the AntiSpam White Rule. |
| insert | firewall | e-mail |  |  | Insert email alert settings |
|  |  | attack |  |  | Insert attack alert settings |
|  |  | set <set#> |  |  | Insert a specified rule set to the firewall configuration |
|  |  | set <set#> | rule <rule#> |  | Insert a specified rule in a set to the firewall configuration |
| cli |  |  |  |  | Display the choices of command list. |
| debug | <1¦0> |  |  |  | Turn on¦off trace for firewall debug information. |

IP Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| ip |  |  |  |  |
|  | address |  | [addr] | display host ip address |
|  | alias |  | <iface> | alias iface |
|  | aliasdis |  | <0¦1> | disable alias |
|  | alg |  |  |  |
|  |  | disp |  | Show ALG enable disable status |
|  |  | enable | <ALG_FTP¦ALG_H323¦ALG_SIP> | Enable ALG command |
|  |  | disable | <ALG_FTP¦ALG_H323¦ALG_SIP> | Disable ALG command |
|  |  | siptimeout | <timeout in second> or 0 for no timeout | Configure SIP timeout command |
|  |  | ftpPortNum | [port number] | Support a different port number on FTP ALG. |
|  | arp |  |  |  |
|  |  | status | <iface> | display ip arp status |
|  |  | add | <hostid> ether <ether addr> | add arp information |
|  |  | resolve | <hostid> | resolve ip-addr |
|  |  | replydif | [<0:No¦1:yes>] | reply different inteface ip-addr's arp request |
|  |  | drop | <hostid> [hardware] | drop arp |
|  |  | flush |  | flush arp table |
|  |  | publish |  | add proxy arp |
|  |  | period | < value: 30~3000> | Set arp period. |
|  |  | attpret | <on¦off> | Switch receive APR from the different network or not. |
|  |  | force | <on¦off> | Switch the time out function of the APR. |
|  |  | gratuitous | <on¦off> | Switch the duplicate IP address detection based on Gratuitous ARP |
|  |  | ackGratuitous | active [yes¦no] | Let DUT accept gratuitous ARP request. |
|  |  |  | forceUpdate [on¦off] | Update the exist MAC mapping to new one. |
|  | dhcp |  | <iface> |  |
|  |  | client |  |  |
|  |  |  | release | release DHCP client IP |
|  |  |  | renew | renew DHCP client IP |
|  |  | mode | <server¦relay¦none¦client> | set dhcp mode |
|  |  | relay | server <serverIP> | set dicp relay server ip-addr |
|  |  | reset |  | reset dhcp table |
|  |  | server |  |  |
|  |  |  | probecount <num> | set dhcp probe count |
|  |  |  | dnsserver <IP1> [IP2] [IP3] | set dns server ip-addr |
|  |  |  | winsserver <winsIP1> [<winsIP2>] | set wins server ip-addr |
|  |  |  | gateway <gatewayIP> | set gateway |
|  |  |  | hostname <hostname> | set hostname |
|  |  |  | initialize | fills in DHCP parameters and initializes (for PWC purposes) |
|  |  |  | leasetime <period> | set dhcp leasetime |
|  |  |  | netmask <netmask> | set dhcp netmask |
|  |  |  | pool <startIP> <numIP> | set dhcp ip pool |
|  |  |  | renewaltime <period> | set dhcp renew time |
|  |  |  | rebindtime <period> | set dhcp rebind time |
|  |  |  | reset | reset dhcp table |
|  |  |  | server <serverIP> | set dhcp server ip for relay |
|  |  |  | dnsorder [router¦isp] | set dhcp dns order |
|  |  |  | release <entry num> | release specific entry of the dhcp server pool |
|  |  | status | [option] | show dhcp status |
|  |  | static |  |  |
|  |  |  | Delete <num>¦all | delete static dhcp mac table |
|  |  |  | display | display static dhcp mac table |
|  |  |  | update <num> <mac> <ip> | update static dhcp mac table |
|  | dns |  |  |  |
|  |  | query |  |  |
|  |  |  | address <ipaddr> [timeout] | resolve ip-addr to name |
|  |  |  | Debug <num> | enable dns debug value |
|  |  |  | Name <hostname> [timeout] | resolve name to multiple IP addresses |
|  |  |  | Status | display dns query status |
|  |  |  | Table | display dns query table |
|  |  | server | <primary> [secondary] [third] | set dns server |
|  |  | stats |  |  |
|  |  |  | Clear | clear dns statistics |
|  |  |  | Disp | display dns statistics |
|  |  | table |  | display dns table |
|  |  | default | <ip> | Set default DNS server |
|  |  | system |  |  |
|  |  |  | display | display dns system information |
|  |  |  | edita <record idx> <name> <0:FQDN¦1:wildcard> <0:from ISP group¦1:user defined> <isp group idx¦ip address> | edit dns A record |
|  |  |  | editns <record idx> <*¦domain name> <0:from ISP¦1:user defined(public)¦2: user defined(private)> <isp group idx¦dns server ip> | edit dns NS record |
|  |  |  | inserta <before record idx¦-1:new> <name> <0:FQDN¦1:wildcard> <0:from ISP group¦1:user defined> <isp group idx¦ip address> | insert dns A record |
|  |  |  | insertns <before record idx¦-1:new> <*¦domain name> <0:from ISP¦1:user defined(public)¦2: user defined(private)> <isp group idx¦dns server ip> | insert dns NS record |
|  |  |  | movea <record idx> <record idx> | move dns A record |
|  |  |  | movens <record idx> <record idx> | move dns NS record |
|  |  |  | dela <record idx> | delete DNS A record |
|  |  |  | delns <record idx> | delete DNS NS record |
|  |  | system cache |  |  |
|  |  |  | disp <0:none¦1:name¦2:type¦3:IP¦4:refCnt¦5:ttl> [0:increase¦1:decrease] | display DNS cache table |
|  |  |  | flush | flush DNS cache |
|  |  |  | negaperiod <second(60 ~ 3600)> | set negative cache period |
|  |  |  | negative <0: disable¦1: enable> | enable/disable dns negative cache |
|  |  |  | positive <0: disable¦1: enable> | enable/disable dns positive cache |
|  |  |  | ttl <second(60 ~ 3600)> | set positive cache maximum ttl |
|  | Httpd |  |  |  |
|  |  | debug | [on¦off] | set http debug flag |
|  | icmp |  |  |  |
|  |  | echo | [on¦off] | set icmp echo response flag |
|  |  | data | <option> | select general data type |
|  |  | status |  | display icmp statistic counter |
|  |  | trace | [on¦off] | turn on/off trace for debugging |
|  |  | discovery | <iface> [on¦off] | set icmp router discovery flag |
|  | ifconfig |  | [iface] [ipaddr] [broadcast <addr> ¦mtu <value>¦dynamic] | configure network interface |
|  | ping |  | <hostid> | ping remote host |
|  | route |  |  |  |
|  |  | status | [if] | display routing table |
|  |  | add | <dest_addr¦default>[/<bits>] <gateway> [<metric>] | add route |
|  |  | addiface | <dest_addr¦default>[/<bits>] <gateway> [<metric>] | add an entry to the routing table to iface |
|  |  | drop | <host addr> [/<bits>] | drop a route |
|  |  | flush |  | flush route table |
|  |  | lookup | <addr> | find a route to the destination |
|  |  | errcnt |  |  |
|  |  |  | disp | display routing statistic counters |
|  |  |  | clear | clear routing statistic counters |
|  | status |  |  | display ip statistic counters |
|  | stroute |  |  |  |
|  |  | display | [rule # ¦ buf] | display rule index or detail message in rule. |
|  |  | load | <rule #> | load static route rule in buffer |
|  |  | save |  | save rule from buffer to spt. |
|  |  | config |  |  |
|  |  |  | name <site name> | set name for static route. |
|  |  |  | destination <dest addr>[/<bits>] <gateway> [<metric>] | set static route destination address and gateway. |
|  |  |  | mask <IP subnet mask> | set static route subnet mask. |
|  |  |  | gateway <IP address> | set static route gateway address. |
|  |  |  | metric <metric #> | set static route metric number. |
|  |  |  | private <yes¦no> | set private mode. |
|  |  |  | active <yes¦no> | set static route rule enable or disable. |
|  | adjTcp |  | <iface> [<mss>] | adjust the TCP mss of iface |
|  | adjmss |  | [mss] | adjust all system TCP mss of iface |
|  | udp |  |  |  |
|  |  | status |  | display udp status |
|  | rip |  |  |  |
|  |  | accept | <gateway> | drop an entry from the RIP refuse list |
|  |  | activate |  | enable rip |
|  |  | merge | [on¦off] | set RIP merge flag |
|  |  | refuse | <gateway> | add an entry to the rip refuse list |
|  |  | request | <addr> [port] | send rip request to some address and port |
|  |  | reverse | [on¦off] | RIP Poisoned Reverse |
|  |  | status |  | display rip statistic counters |
|  |  | trace |  | enable debug rip trace |
|  |  | mode |  |  |
|  |  |  | <iface> in [mode] | set rip in mode |
|  |  |  | <iface> out [mode] | set rip out mode |
|  |  | dialin_user | [show¦in¦out¦both¦none] | show dialin user rip direction |
|  | tcp |  |  |  |
|  |  | ceiling | [value] | TCP maximum round trip time |
|  |  | floor | [value] | TCP minimum rtt |
|  |  | irtt | [value] | TCP default init rtt |
|  |  | kick | <tcb> | kick tcb |
|  |  | limit | [value] | set tcp output window limit |
|  |  | max-incomplete | [number] | Set the maximum number of TCP incomplete connection. |
|  |  | mss | [value] | TCP input MSS |
|  |  | reset | <tcb> | reset tcb |
|  |  | rtt | <tcb> <value> | set round trip time for tcb |
|  |  | status | [tcb] [<interval>] | display TCP statistic counters |
|  |  | syndata | [on¦off] | TCP syndata piggyback |
|  |  | trace | [on¦off] | turn on/off trace for debugging |
|  |  | window | [tcb] | TCP input window size |
|  | samenet |  | <iface1> [<iface2>] | display the ifaces that in the same net |
|  | uninet |  | <iface> | set the iface to uninet |
|  | telnet |  | <host> [port] | execute telnet clinet command |
|  | tftp |  |  |  |
|  |  | support |  | pritn if tfpt is support |
|  |  | stats |  | display tftp status |
|  | traceroute |  | <host> [ttl] [wait] [queries] | send probes to trace route of a remote host |
|  | xparent |  |  |  |
|  |  | join | <iface1> [<iface2>] | join iface2 to iface1 group |
|  |  | break | <iface> | break iface to leave ipxparent group |
|  | anitprobe |  | <0¦1> 1:yes 0:no | set ip anti-probe flag |
|  | forceproxy |  | <display¦set> [on¦off] [servicePort] [proxyIp] [proxyport] | enable TCP forceproxy |
|  | ave |  |  | anti-virus enforce |
|  | urlfilter |  |  |  |
|  |  | bypass | [LAN/DMZ/WAN] [ON/OFF] | Let lan to lan(for example) packet bypass content filter. |
|  |  | enable |  | enable/disable url filter function |
|  |  | reginfo |  |  |
|  |  |  | display | display urlfilter registration information |
|  |  |  | name | set urlfilter registration name |
|  |  |  | eMail <size> | set urlfilter registration email addr |
|  |  |  | country <size> | set urlfilter registration country |
|  |  |  | clearAll | clear urlfilter register information |
|  |  | category |  |  |
|  |  |  | display | display urlfilter category |
|  |  |  | webFeature [block/nonblock] [activex/java/cookei/webproxy] | block or unblock webfeature |
|  |  |  | logAndBlock [log/logAndBlock] | set log only or log and block |
|  |  |  | blockCategory [block/nonblock] [all/type(1-14)] | block or unblock type |
|  |  |  | timeOfDay [always/hh:mm] [hh:mm] | set block time |
|  |  |  | clearAll | clear all category information |
|  |  | listUpdate |  |  |
|  |  |  | display | display listupdate status |
|  |  |  | actionFlags [yes/no] | set listupdate or not |
|  |  |  | scheduleFlag [pending] | set schedule flag |
|  |  |  | dayFlag [pending] | set day flag |
|  |  |  | time [pending] | set time |
|  |  |  | clearAll | clear all listupdate information |
|  |  | exemptZone |  |  |
|  |  |  | display | display exemptzone information |
|  |  |  | actionFlags [type(1-3)][enable/disable] | set action flags |
|  |  |  | add [ip1] [ip2] | add exempt range |
|  |  |  | delete [ip1] [ip2] | delete exempt range |
|  |  |  | reset | clear exemptzone information |
|  |  | customize |  |  |
|  |  |  | display | display customize action flags |
|  |  |  | actionFlags [filterList/disableAllExceptTrusted/unblockRWFToTrusted/keywordBlock/fullPath/caseInsensitive/fileName][enable/disable] | set action flags |
|  |  |  | logFlags [type(1-3)][enable/disable] | set log flags |
|  |  |  | add [string] [trust/untrust/keyword] | add url string |
|  |  |  | delete [string] [trust/untrust/keyword] | delete url string |
|  |  |  | reset | clear all information |
|  |  | logDisplay |  | display cyber log |
|  |  | ftplist |  | update cyber list data |
|  |  | listServerIP | <ipaddr> | set list server ip |
|  |  | listServerName | <name> | set list server name |
|  |  | general |  |  |
|  |  |  | enable | enable/disable url filter function |
|  |  |  | display | display content filer’s general setting |
|  |  |  | webFeature | [block/nonblock] [activex/java/cookei/webproxy] |
|  |  |  | timeOfDay[always/hh:mm] [hh:mm] | set block time |
|  |  |  | exemptZone display | display exemptzone information |
|  |  |  | exemptZone actionFlags [type(1-3)][enable/disable] | set action flags |
|  |  |  | exemptZone add [ip1] [ip2] | add exempt range |
|  |  |  | exemptZone delete [ip1] [ip2] | delete exempt range |
|  |  |  | exemptZone reset | clear exemptzone information |
|  |  |  | reset | reset content filter’s general setting |
|  |  | webControl |  |  |
|  |  |  | enable | enable cbr_filter |
|  |  |  | display | display cbr_filter’s setting |
|  |  |  | logAndBlock [log/block/both] | set log or block on matched web site |
|  |  |  | category | set blocked categories |
|  |  |  | serverList display | display current cbr_filter servers |
|  |  |  | serverList refresh | refresh cbr_filter servers |
|  |  |  | queryURL [url][Server/localCache] | query url need to block or forward according the database on server or local cache |
|  |  |  | cache display | display the local cache entries |
|  |  |  | cache delete [entrynum/All] | delete the local cache entries |
|  |  |  | cache timeout [hour] | Set timeout value of cache entries |
|  |  |  | blockonerror [log/block][on/off] | choose log or block when server is unavailable |
|  |  |  | unratedwebsite[block¦log][on¦off] | hoose log or block for unrated web site |
|  |  |  | waitingTime [sec] | set waiting time for server |
|  |  |  | reginfo display | display the license key with cerberian |
|  |  |  | reginfo refresh | Check whether device had been registered and write the original license key to flash |
|  |  |  | zssw | change the zssw’s URL |
|  | tredir |  |  |  |
|  |  | failcount | <count> | set tredir failcount |
|  |  | partner | <ipaddr> | set tredir partner |
|  |  | target | <ipaddr> | set tredir target |
|  |  | timeout | <timeout> | set tredir timeout |
|  |  | checktime | <period> | set tredir checktime |
|  |  | active | <on¦off> | set tredir active |
|  |  | save |  | save tredir information |
|  |  | disp |  | display tredir information |
|  |  | debug | <value> | set tredir debug value |
|  | rpt |  |  |  |
|  |  | active | [0:lan¦1:dmz][1:yes¦0:no] | active report |
|  |  | start | [0:lan¦1:dmz] | start report |
|  |  | stop | [0:lan¦1:dmz] | stop report |
|  |  | url | [0:lan¦1:dmz] [num] | top url hit list |
|  |  | ip | [0:lan¦1:dmz] [num] | top ip addr list |
|  |  | srv | [0:lan¦1:dmz] [num] | top service port list |
|  | dropIcmp |  | [0 ¦ 1] | to drop ICMP fragment packets |
|  | nat |  |  |  |
|  |  | period | [period] | set nat timer period |
|  |  | port | [port] | set nat starting external port number |
|  |  | checkport |  | verify all server tables are valid |
|  |  | timeout |  |  |
|  |  |  | gre [timeout] | set nat gre timeout value |
|  |  |  | iamt [timeout] | set nat iamt timeout value |
|  |  |  | generic [timeout] | set nat generic timeout value |
|  |  |  | reset [timeout] | set nat reset timeout value |
|  |  |  | tcp [timeout] | set nat tcp timeout value |
|  |  |  | tcpother [timeout] | set nat tcp other timeout value |
|  |  |  | udp [port] <value> | set nat udp timeout value of specific port |
|  |  |  | display | display all the timeout values |
|  |  | update |  | create nat system information from spSysParam |
|  |  | iamt | <iface> | display nat iamt information |
|  |  | lookup | <rule set> | display nat lookup rule |
|  |  | loopback | [on¦off] | turn on/off nat loopback flag |
|  |  | reset | <iface> | reset nat table of an iface |
|  |  | server |  |  |
|  |  |  | disp | display nat server table |
|  |  |  | load <set id> | load nat server information from ROM |
|  |  |  | save | save nat server information to ROM |
|  |  |  | clear <set id> | clear nat server information |
|  |  |  | edit active <yes¦no> | set nat server edit active flag |
|  |  |  | edit svrport <start port> [end port] | set nat server server port |
|  |  |  | edit intport <start port> [end port] | set nat server forward port |
|  |  |  | edit remotehost <start ip> [end ip] | set nat server remote host ip |
|  |  |  | edit leasetime [time] | set nat server lease time |
|  |  |  | edit rulename [name] | set nat server rule name |
|  |  |  | edit forwardip [ip] | set nat server server ip |
|  |  |  | edit protocol [protocol id] | set nat server protocol |
|  |  |  | edit clear | clear one rule in the set |
|  |  | service |  |  |
|  |  |  | irc [on¦off] | turn on/off irc flag |
|  |  |  | xboxlive [on¦off] | turn on/off xboxlive flag |
|  |  |  | sip debug | enable/disable sip debug flag |
|  |  |  | sip display | display the sip call buffer |
|  |  |  | aol [on¦off] | Turn on/off aol flag |
|  |  | resetport |  | reset all nat server table entries |
|  |  | incikeport | [on¦off] | turn on/off increase ike port flag |
|  |  | session | [session per host] | set nat session per host value |
|  |  | deleteslot | <iface> <slot> | delete specific slot of iface |
|  |  | debug |  |  |
|  |  |  | natTraversal [on¦off] | set NAT traversal debug flag |
|  |  |  | hash [on¦off] | set NAT hash table debug flag |
|  |  |  | session [on¦off] | set NAT session debug flag |
|  |  | hashtable | <enifX, X=0, 1, 2, …> | show the NAT hash table of enifX |
|  |  | natTable | [enifX, X=0, 1, 2, …] | show the NAT global information |
|  |  | simulation | <enifX, X=0, 1, 2, …> | for engineer debug only |
|  |  | acl |  |  |
|  |  |  | display | display all NAT acl set and rule information |
|  |  |  | load <set number> | load a specific acl of set number |
|  |  |  | move <set#> <rule# from> <rule# to> | Move specific acl rule to specific position. |
|  |  |  | save <set number> | save a specific acl of set number |
|  |  | routing | [0:LAN¦1:DMZ] [0:no¦1:yes] | set NAT routing attributes |
|  |  | historicalCHigh |  | Display the historical highest count of concurrent NAT sessions |
|  |  | historicalHigh |  | Display the historical highest count of NAT sessions based on per host. |
|  | igmp |  |  |  |
|  |  | debug | [level] | set igmp debug level |
|  |  | forwardall | [on¦off] | turn on/off igmp forward to all interfaces flag |
|  |  | querier | [on¦off] | turn on/off igmp stop query flag |
|  |  | iface |  |  |
|  |  |  | <iface> grouptm <timeout> | set igmp group timeout |
|  |  |  | <iface> interval <interval> | set igmp query interval |
|  |  |  | <iface> join <group> | join a group on iface |
|  |  |  | <iface> leave <group> | leave a group on iface |
|  |  |  | <iface> query | send query on iface |
|  |  |  | <iface> rsptime [time] | set igmp response time |
|  |  |  | <iface> start | turn on of igmp on iface |
|  |  |  | <iface> stop | turn off of igmp on iface |
|  |  |  | <iface> ttl <threshold> | set ttl threshold |
|  |  |  | <iface> v1compat [on¦off] | turn on/off v1compat on iface |
|  |  | robustness | <num> | set igmp robustness variable |
|  |  | status |  | dump igmp status |
|  | pr |  |  |  |
|  |  | clear |  | clear ip pr table counter information |
|  |  | disp |  | display policy route set and rule information |
|  |  | move |  | move specific policy route rule to another rule |
|  |  | dispCnt |  | dump ip pr table counter information |
|  |  | switch |  | turn on/off ip pr table counter flag |

IPSec Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| ipsec |  |  |  |  |
|  | debug | type | <0:Disable ¦ 1:Original on¦off ¦ 2:IKE on¦off ¦ 3: IPSec [SPI]¦on¦off ¦ 4:XAUTH on¦off ¦ 5:CERT on¦off ¦ 6: All> | Turn on¦off trace for IPsec debug information |
|  |  | level | <0:None ¦ 1:User ¦ 2:Low ¦ 3:High> | Set the debug level. Higher number means more detailed. |
|  |  | display |  | Show debugging information, include type and level. |
|  | route | dmz | <on¦off> | After a packet is IPSec processed and will be sent to DMZ side, this switch is to control if this packet can be applied IPSec again. |
|  |  | lan | <on¦off> | After a packet is IPSec processed and will be sent to LAN side, this switch is to control if this packet can be applied IPSec again. |
|  |  |  |  | Remark: Command available since 3.50(WA.3) |
|  |  | wan | <on¦off> | After a packet is IPSec processed and will be sent to WAN side, this switch is to control if this packet can be applied IPSec again. |
|  |  | wan2 | <on¦off> | After a packet is IPSec processed and will be sent to WAN2 side, this switch is to control if this packet can be applied IPSec again. |
|  |  | wlan | <on¦off> | After a packet is IPSec processed and will be sent to WLAN side, this switch is to control if this packet can be applied IPSec again. |
|  | show_runtime | sa |  | display runtime phase 1 and phase 2 SA information |
|  |  | spd |  | When a dynamic rule accepts a request and a tunnel is established, a runtime SPD is created according to peer local IP address. This command is to show these runtime SPD. |
|  |  | list |  | Display brief runtime phase 1 and phase 2 SA information |
|  | switch | <on¦off> |  | As long as there exists one active IPSec rule, all packets will run into IPSec process to check SPD. This switch is to control if a packet should do this. If it is turned on, even there exists active IPSec rules, packets will not run IPSec process. |
|  | timer | chk_conn. | <0~255> | - Adjust auto-timer to check if any IPsec connection has “only outbound traffic but no inbound traffic” for certain period. If yes, system will disconnect it. |
|  |  |  |  | - Interval is in minutes |
|  |  |  |  | - Default is 2 minuets |
|  |  |  |  | - 0 means never timeout |
|  |  | update_peer | <0~255> | - Adjust auto-timer to update IPSec rules which use domain name as the secure gateway IP. |
|  |  |  |  | - Interval is in minutes |
|  |  |  |  | - Default is 30 minutes |
|  |  |  |  | - 0 means never update |
|  |  | chk_input | <0~255> | - Adjust input timer to check if any IPSec connection has no inbound traffic for a certain period. If yes, system will disconnect it. |
|  |  |  |  | - Interval is in minutes |
|  |  |  |  | - Default is 2 minuets |
|  |  |  |  | - 0 means never timeout |
|  | updatePeerIp |  |  | Force system to update IPSec rules which use domain name as the secure gateway IP right away. |
|  | dial | <policy index> |  | Initiate IPSec rule <policy index> from ZyWALL box |
|  | enable | <on¦off> |  | Turn on/off IPSec feature |
|  | ikeDisplay | <rule #> |  | Display IKE rule #, if no rule number assigned, this command will show current working buffer. NOTE: If working buffer is null, it will show error messages. Please ADD or EDIT an IKE rule before display. |
|  | ikeAdd |  |  | Create a working buffer for IKE rule. |
|  | ikeEdit | <rule #> |  | Edit an existing IKE rule # |
|  | ikeSave |  |  | Save working buffer of IKE rule to romfile. |
|  | ikeList |  |  | List all IKE rules |
|  | ikeDelete | <rule #> |  | Delete IKE rule # |
|  | ikeConfig | name | <string> | Set rule name (max length is 31) |
|  |  | negotiationMode | <0:Main ¦ 1:Aggressive> | Set negotiation mode |
|  |  | natTraversal | <Yes¦ No> | Enable NAT traversal or not. |
|  |  | multiPro | <Yes¦No> | Enable multiple proposals in IKE or not |
|  |  | lcIdType | <0:IP ¦ 1:DNS ¦ 2:Email> | Set local ID type |
|  |  | lcIdContent | <string> | Set local ID content |
|  |  | myIpAddr | <IP address> | Set my IP address |
|  |  | peerIdType | <0:IP ¦ 1:DNS ¦ 2:Email> | Set peer ID type |
|  |  | peerIdContent | <string> | Set peer ID content |
|  |  | secureGwAddr | <IP address ¦ Domain name> | Set secure gateway address or domain name |
|  |  | authMethod | <0:PreSharedKey ¦ 1:RSASignature ¦ 2:preShare Key+XAUTH ¦ 3:RSASignature+XAUTH> | Set authentication method in phase 1 in IKE |
|  |  | preShareKey | <ASCII ¦ 0xHEX> | Set pre shared key in phase 1 in IKE |
|  |  | certificate | <certificate name> | Set certificate file if using RSA signature as authentication method. |
|  |  | encryAlgo | <0:DES ¦ 1:3DES ¦ 2:AES> | Set encryption algorithm in phase 1 in IKE |
|  |  | authAlgo | <0:MD5 ¦ 1:SHA1> | Set authentication algorithm in phase 1 in IKE |
|  |  | saLifeTime | <seconds> | Set sa life time in phase 1 in IKE |
|  |  | keyGroup | <0:DH1 ¦ 1:DH2> | Set key group in phase 1 in IKE |
|  |  | xauth | type <0:Client Mode ¦ 1:Server Mode> | Set client or server mode. |
|  |  |  | username <name> | Set xauth user name |
|  |  |  | password <password> | Set xauth password |
|  |  |  | radius <username> <password> | Ser radius username and password |
|  |  | ha | enable <on¦off> | Enable / disable IPSec HA |
|  |  |  | redunSecGwAddr <IP address ¦ Domain name> | Configure redundant remote secure gateway address or domain name |
|  |  |  | failback enable <on¦off> | Enable or disable “Fail back to primary secure gateway when possible” |
|  |  |  | failback interval <number> | Configure the check interval for fail back detection |
|  |  |  | failover display | Display current fail over detection method |
|  |  |  | failover dpd <on¦off> | Enable / disable fail over by DPD |
|  |  |  | failover outputIdleTime <on¦off> | Enable / disable fail over by output idle timer |
|  |  |  | failover pingCheck <on¦off> | Enable / disable fail over by ping check |
|  | ipsecDisplay | <rule #> |  | Display IPSec rule #, if no rule number assigned, this command will show current working buffer. NOTE: If working buffer is null, it will show error messages. Please ADD or EDIT an IPSec rule before display. |
|  | ipsecAdd |  |  | Create a working buffer for IPSec rule. |
|  | ipsecEdit | <rule #> |  | Edit IPSec rule # |
|  | ipsecSave |  |  | Save working buffer of IPSec rule to romfile. |
|  | ipsecList |  |  | List all IPSec rules |
|  | ipsecDelete | <rule #> |  | Delete IPSec rule # |
|  | ipsecConfig | name | <string> | Set rule name. (max length is 31) |
|  |  | active | <Yes ¦ No> | Set active or not |
|  |  | saIndex | <index> | Bind to which IKE rule. |
|  |  | multiPro | <Yes ¦ No> | Enable multiple proposals in IPSec or not |
|  |  | nailUp | <Yes ¦ No> | Enable nailed-up or not |
|  |  | activeProtocol | <0:AH ¦ 1:ESP> | Set active protocol in IPSec |
|  |  | encryAlgo | <0:Null ¦ 1:DES ¦ 2:3DES ¦ 3:AES> | Set encryption algorithm in IPSec |
|  |  | encryKeyLen | <0:128 ¦ 1:192 ¦ 2:256> | Set encryption key length in IPSec |
|  |  | authAlgo | <0:MD5 ¦ 1:SHA1> | Set authentication algorithm in IPSec |
|  |  | saLifeTime | <seconds> | Set sa life time in IPSec |
|  |  | encap | <0:Tunnel ¦ 1:Transport> | set encapsulation in IPSec |
|  |  | pfs | <0:None ¦ 1:DH1 ¦ 2:DH2> | set pfs in phase 2 in IPSec |
|  |  | antiReplay | <Yes ¦ No> | Set anitreplay or not |
|  |  | controlPing | <Yes¦No> | Enable control ping or not |
|  |  | logControlPing | <Yes¦No> | Enable logging control ping events or not |
|  |  | controlPingAddr | <IP> | Set control ping address |
|  |  | protocol | <1:ICMP ¦ 6:TCP ¦ 17:UDP> | Set protocol |
|  |  | lcAddrType | <0:single ¦ 1:range ¦ 2:subnet> | Set local address type |
|  |  | lcAddrStart | <IP> | Set local start address |
|  |  | lcAddrEndMask | <IP> | Set local end address or mask |
|  |  | lcPortStart | <port> | Set local start port |
|  |  | lcPortEnd | <port> | Set local end port |
|  |  | rmAddrType | <0:single ¦ 1:range ¦ 2:subnet> | Set remote address type |
|  |  | rmAddrStart | <IP> | Set remote start address |
|  |  | rmAddrEndMask | <IP> | Set remote end address or mask |
|  |  | rmPortStart | <port> | Set remote start port |
|  |  | rmPortEnd | <port> | Set remote end port |
|  | policyList |  |  | List all IPSec policies |
|  | manualDisplay | <rule #> |  | Display manual rule # |
|  | manualAdd |  |  | Add manual rule |
|  | manualEdit | <rule #> |  | Edit manual rule # |
|  | manualSave |  |  | Save IPSec rules |
|  | manualList |  |  | List all IPSec rule |
|  | manualDelete | <rule #> |  | Delete IPSec rule # |
|  | manualConfig | name | <string> | Set rule name |
|  |  | active | <Yes ¦ No> | Set active or not |
|  |  | myIpAddr | <IP address> | Set my IP address |
|  |  | secureGwAddr | <IP address> | Set secure gateway |
|  |  | protocol | <1:ICMP ¦ 6:TCP ¦ 17:UDP> | Set protocol |
|  |  | lcAddrType | <0:single ¦ 1:range ¦ 2:subnet> | Set local address type |
|  |  | lcAddrStart | <IP> | Set local start address |
|  |  | lcAddrEndMask | <IP> | Set local end address or mask |
|  |  | lcPortStart | <port> | Set local start port |
|  |  | lcPortEnd | <port> | Set local end port |
|  |  | rmAddrType | <0:single ¦ 1:range ¦ 2:subnet> | Set remote address type |
|  |  | rmAddrStart | <IP> | Set remote start address |
|  |  | rmAddrEndMask | <IP> | Set remote end address or mask |
|  |  | rmPortStart | <port> | Set remote start port |
|  |  | rmPortEnd | <port> | Set remote end port |
|  |  | activeProtocol | <0:AH ¦ 1:ESP> | Set active protocol in manual |
|  |  | ah | encap <0:Tunnel ¦ 1:Transport> | Set encapsulation in ah in manual |
|  |  |  | spi <decimal> | Set spi in ah in manual |
|  |  |  | authAlgo <0:MD5 ¦ 1:SHA1> | Set authentication algorithm in ah in manual |
|  |  |  | authKey <string> | Set authentication key in ah in manual |
|  |  | esp | encap <0:Tunnel ¦ 1:Transport> | Set encapsulation in esp in manual |
|  |  |  | spi <decimal> | Set spi in esp in manual |
|  |  |  | encryAlgo <0:Null ¦ 1:DES ¦ 2:3DES> | Set encryption algorithm in esp in manual |
|  |  |  | encryKey <string> | Set encryption key in esp in manual |
|  |  |  | authAlgo <0:MD5 ¦ 1:SHA1> | Set authentication algorithm in esp in manual |
|  |  |  | authKey < string> | Set authentication key in esp in manual |
|  | manualPolicyList |  |  | List all manual policy |
|  | swSkipOverlapIp |  | <on¦off> | - When a VPN rule with remote range overlaps with local range, the switch decides if a local to local packet should apply this rule. - Default value is “off” which means “no skip”. |
|  | swFwScan | <on¦off> |  | Enable / disable to skip firewall packet inspection for IPSec packet. |
|  | swIdpScan | <on¦off> |  | Enable / disable the IDP for IPSec packet. |
|  | swAvScan | <on¦off> |  | Enable / disable the Anti Virus for IPSec packet. |
|  | swAsScan | <on¦off> |  | Enable / disable the Anti Spam for IPSec packet. |
|  | swCfScan | <on¦off> |  | Enable / disable the Content Filter for IPSec packet. |
|  | adjTcpMss |  | <off¦auto¦user defined value> | After a tunnel is established, system will automatically adjust TCP MSS. After all tunnels are drops, the MSS will adjust to the original value. The default value is auto. |
|  | ha | pingRetryCnt | <value> (1~10) | Ping retry fail tolerence |
|  |  | debug | <on¦off¦runtime¦spt> | On: turn on debug message Off: turn on debug message Runtime: show runtime data structure Spt: show SPT record data |
|  | Drop |  | <policy index> | Drop an active tunnel. |
|  | swSkipPPTP |  | [on¦off] | Enable / disable to skip PPTP packets to go in ipsec tunnel. |
|  | initContactMode |  | <gateway¦tunnel> | Set initial contact mode to base on tunnel or gateway. Change to tunnel mode can support multiple VPN client which located at same NAT router. |
|  | async | active | <on¦off> | Enable / disable the asynchronous mode |
|  |  | utility |  | Crypto engine utility rate |
|  |  | queue | <on¦off> | Enable / disable the asynchronous queue function |
|  |  | display |  | Asynchronous mode function status |
|  |  | debug | <on¦off> | Show asynchronous debug message |
|  | swDevTri |  | <on¦off> | Enable / disable device trigger tunnel |

PPP Related Command									Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| ppp |  |  |  |  |
|  | bod |  |  |  |
|  |  | remote | <iface> | show remote bod information |
|  |  | reset |  | reset bod |
|  |  | setremote | <iface> | set remote bod |
|  |  | status | <wan_iface> | show wan port bod status |
|  |  | clear | <wan_iface> | clear wan port bod data |
|  |  | on |  | set bod flag on |
|  |  | off |  | set bod flag off |
|  |  | node | <node> <dir> | config the statistic method for remote node bod traffic data |
|  |  | debug | [on¦off] | show bod debug flag |
|  |  | cnt |  |  |
|  |  |  | disp | show bod state |
|  |  |  | clear | clear bod state |
|  | ccp |  | [on¦off] | set/display dial-in ccp switch |
|  | lcp |  |  |  |
|  |  | acfc | [on¦off] | set address/control field compression flag |
|  |  | pfc | [on¦off] | set protocol field compression flag |
|  |  | mpin | [on¦off] | set incoming call MP flag |
|  |  | callback | [on¦off] | set callback flag |
|  |  | bacp | [on¦off] | set bandwidth allocation control flag |
|  |  | echo |  |  |
|  |  |  | retry <retry_count> | set/display retry count to send echo-request |
|  |  |  | time <interval> | set/display time interval to send echo-request |
|  | ipcp |  |  |  |
|  |  | close |  | close connection on ppp interface |
|  |  | list | <iface> | show ipcp state |
|  |  | open |  | open fsm link |
|  |  | timeout | [value] | set timeout interval when waiting for response from remote peer |
|  |  | try |  |  |
|  |  |  | configure [value] | set/display fsm try config |
|  |  |  | failure [value] | set/display fsm try failure |
|  |  |  | terminate [value] | set/display fsm try terminate |
|  |  | compress | [on¦off] | set compress flag |
|  |  | slots | [slot_num] | set number of slots |
|  |  | idcompress | [on¦off] | set/display slot id compress |
|  |  | address | [on¦off] | set/display ip one address option |
|  | mp |  |  |  |
|  |  | default |  | show link default flag |
|  |  |  | rotate | set link default to rotate |
|  |  |  | split | set link default to split |
|  |  | split | [0¦1] | set/display link split |
|  |  | rotate | [0¦1] | set/display link rotate |
|  |  | sequence |  | set/display mp start sequence |
|  | configure |  |  |  |
|  |  | ipcp |  |  |
|  |  |  | compress [on¦off] | enable/disable compress |
|  |  |  | slots [slot_num] | select number of slots |
|  |  |  | idcompress [on¦off] | enable/disable slot id compress |
|  |  |  | address [on¦off] | set/display ip one address option |
|  |  | atcp |  | apple talk feature not supported anymore |
|  |  | ccp |  |  |
|  |  |  | ascend [on¦off] | set/display ascend stac flag |
|  |  |  | history <count> | set/display stac history count |
|  |  |  | check [argv] | set/display stac check mode |
|  |  |  | reset <mode> | set/display stac reset mode |
|  |  |  | pfc [on¦off] | set/display pfc flag |
|  |  |  | debug [on¦off] | set/display ccp debug flag |
|  | iface |  |  |  |
|  |  |  | <iface> ipcp | show the ipcp status of the given iface |
|  |  |  | <iface> ipxcp | show the ipxcp status of the given iface |
|  |  |  | <iface> atcp |  |
|  |  |  | <iface> ccp [reset¦skip¦flush] | show the ccp status of the given iface |
|  |  |  | <iface> mp | show the mp status of the given iface |
|  | show |  | <channel> | show the ppp channel status |
|  | fsm |  |  |  |
|  |  | trace |  |  |
|  |  |  | break [num] [count] [flag] | set the fsm log break value |
|  |  |  | clear | clear the fsm log data |
|  |  |  | disp | display the fsm log data |
|  |  |  | filter [mask] [protocol] | set the fsm log filter value |
|  |  | tdata |  |  |
|  |  |  | filter [protocol1] [protocol2] … | set the fsm filter data |
|  |  |  | disp | display the fsm data |
|  |  |  | clear | clear the fsm data |
|  |  | struc |  | dump fsm data structure |
|  | delay |  | [inteval] | set the delay timer for sending first PPP packet after call answered |

Firewall Related Command								Home

| Command |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| sys | Firewall |  |  |  |  |
|  |  | acl |  |  |  |
|  |  |  | disp |  | Display specific ACL set # rule #, or all ACLs. |
|  |  | active | <yes¦no> |  | Active firewall or deactivate firewall |
|  |  | cnt |  |  |  |
|  |  |  | disp |  | Display firewall log type and count. |
|  |  |  | clear |  | Clear firewall log count. |
|  |  | dynamicrule |  |  | SUPPORT_DYNAMIC_PORT |
|  |  |  | timeout |  | Set dynamic ACL rule timeout value |
|  |  | dos |  |  |  |
|  |  |  | smtp |  | Set SMTP DoS defender on/off |
|  |  |  | display |  | Display SMTP DoS defender setting. |
|  |  |  | ignore |  | Set if firewall ignore DoS in lan/wan1/wan2/dmz/wlan/vpn |
|  |  | ignore |  |  |  |
|  |  |  | logBroadcast | <from> <to> <on¦off> | Set ignore log broadcast flag. The <from> and <to> parameters include lan/wan1/wan2/dmz/wlan/vpn. |
|  |  |  | triangle |  | Set if firewall ignore triangle route in lan/wan/dmz/wlan |
|  |  | schedule |  |  |  |
|  |  |  | load [ set # rule #] |  | Load firewall ACL schedule by rule. |
|  |  |  | display |  | Display ACL schedule in buffer. |
|  |  |  | save |  | Save buffer date and update runtime firewall ACL rule. |
|  |  |  | week |  |  |
|  |  |  |  | monday [on/off] | Set schedule on or off by day – Monday. |
|  |  |  |  | tuesday [on/off] | Set schedule on or off by day – Tuesday. |
|  |  |  |  | wednesday [on/off] | Set schedule on or off by day – Wednesday. |
|  |  |  |  | thursday [on/off] | Set schedule on or off by day – Thursday. |
|  |  |  |  | friday [on/off] | Set schedule on or off by day – Friday. |
|  |  |  |  | saturday [on/off] | Set schedule on or off by day – Saturday. |
|  |  |  |  | sunday [on/off] | Set schedule on or off by day – Sunday. |
|  |  |  |  | allweek [on/off] | Quick set schedule on or off by week. |
|  |  |  | timeOfDay [always/hh:mm] |  | Set firewall ACL schedule block time of day. |

Certificate Management (PKI) Command							Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| certificates |  |  |  |  |
|  | my_cert |  |  |  |
|  |  | create |  |  |
|  |  |  | self_signed <name> <subject> [key size] | Create a self-signed local host certificate. <name> specifies a descriptive name for the generated certificate. <subject> specifies a subject name (required) and alternative name (required). The format is "subject-name-dn;{ip,dns,email}=value". If the name contains spaces, please put it in quotes. [key size] specifies the key size. It has to be an integer from 512 to 2048. The default is 1024 bits. |
|  |  |  | request <name> <subject> [key size] | Create a certificate request and save it to the router for later manual enrollment. <name> specifies a descriptive name for the generated certification request. <subject> specifies a subject name (required) and alternative name (required). The format is "subject-name-dn;{ip,dns,email}=value". If the name contains spaces, please put it in quotes. [key size] specifies the key size. It has to be an integer from 512 to 2048. The default is 1024 bits. |
|  |  |  | scep_enroll <name> <CA addr> <CA cert> <auth key> <subject> [key size] | Create a certificate request and enroll for a certificate immediately online using SCEP protocol. <name> specifies a descriptive name for the enrolled certificate. <CA addr> specifies the CA server address. <CA cert> specifies the name of the CA certificate. <auth key> specifies the key used for user authentication. If the key contains spaces, please put it in quotes. To leave it blank, type "". <subject> specifies a subject name (required) and alternative name (required). The format is "subject-name-dn;{ip,dns,email}=value". If the name contains spaces, please put it in quotes. [key size] specifies the key size. It has to be an integer from 512 to 2048. The default is 1024 bits. |
|  |  |  | cmp_enroll <name> <CA addr> <CA cert> <auth key> <subject> [key size] | Create a certificate request and enroll for a certificate immediately online using CMP protocol. <name> specifies a descriptive name for the enrolled certificate. <CA addr> specifies the CA server address. <CA cert> specifies the name of the CA certificate. <auth key> specifies the id and key used for user authentication. The format is "id:key". To leave the id and key blank, type ":". <subject> specifies a subject name (required) and alternative name (required). The format is "subject-name-dn;{ip,dns,email}=value". If the name contains spaces, please put it in quotes. [key size] specifies the key size. It has to be an integer from 512 to 2048. The default is 1024 bits. |
|  |  | import [name] |  | Import the PEM-encoded certificate from stdin. [name] specifies the descriptive name (optional) as which the imported certificate is to be saved. For my certificate importation to be successful, a certification request corresponding to the imported certificate must already exist on ZyWALL. After the importation, the certification request will automatically be deleted. If a descriptive name is not specified for the imported certificate, the certificate will adopt the descriptive name of the certification request. |
|  |  | export <name> |  | Export the PEM-encoded certificate to stdout for user to copy and paste. <name> specifies the name of the certificate to be exported. |
|  |  | view <name> |  | View the information of the specified local host certificate. <name> specifies the name of the certificate to be viewed. |
|  |  | verify <name> [timeout] |  | Verify the certification path of the specified local host certificate. <name> specifies the name of the certificate to be verified. [timeout] specifies the timeout value in seconds (optional). The default timeout value is 20 seconds. |
|  |  | delete <name> |  | Delete the specified local host certificate. <name> specifies the name of the certificate to be deleted. |
|  |  | list |  | List all my certificate names and basic information. |
|  |  | rename <old name> <new name> |  | Rename the specified my certificate. <old name> specifies the name of the certificate to be renamed. <new name> specifies the new name as which the certificate is to be saved. |
|  |  | def_self_signed [name] |  | Set the specified self-signed certificate as the default self-signed certificate. [name] specifies the name of the certificate to be set as the default self-signed certificate. If [name] is not specified, the name of the current self-signed certificate is displayed. |
|  |  | replace_factory |  |  |
|  | ca_trusted |  |  |  |
|  |  | import <name> |  | Import the PEM-encoded certificate from stdin. <name> specifies the name as which the imported CA certificate is to be saved. |
|  |  | export <name> |  | Export the PEM-encoded certificate to stdout for user to copy and paste. <name> specifies the name of the certificate to be exported. |
|  |  | view <name> |  | View the information of the specified trusted CA certificate. <name> specifies the name of the certificate to be viewed. |
|  |  | verify <name> [timeout] |  | Verify the certification path of the specified trusted CA certificate. <name> specifies the name of the certificate to be verified. [timeout] specifies the timeout value in seconds (optional). The default timeout value is 20 seconds. |
|  |  | delete <name> |  | Delete the specified trusted CA certificate. <name> specifies the name of the certificate to be deleted. |
|  |  | list |  | List all trusted CA certificate names and basic information. |
|  |  | rename <old name> <new name> |  | Rename the specified trusted CA certificate. <old name> specifies the name of the certificate to be renamed. <new name> specifies the new name as which the certificate is to be saved. |
|  |  | crl_issuer <name> [on¦off] |  | Specify whether or not the specified CA issues CRL. <name> specifies the name of the CA certificate. [on¦off] specifies whether or not the CA issues CRL. If [on¦off] is not specified, the current crl_issuer status of the CA. |
|  | remote_trusted |  |  |  |
|  |  | import <name> |  | Import the PEM-encoded certificate from stdin. <name> specifies the name as which the imported remote host certificate is to be saved. |
|  |  | export <name> |  | Export the PEM-encoded certificate to stdout for user to copy and paste. <name> specifies the name of the certificate to be exported. |
|  |  | view <name> |  | View the information of the specified trusted remote host certificate. <name> specifies the name of the certificate to be viewed. |
|  |  | verify <name> [timeout] |  | Verify the certification path of the specified trusted remote host certificate. <name> specifies the name of the certificate to be verified. [timeout] specifies the timeout value in seconds (optional). The default timeout value is 20 seconds. |
|  |  | delete <name> |  | Delete the specified trusted remote host certificate. <name> specifies the name of the certificate to be deleted. |
|  |  | list |  | List all trusted remote host certificate names and basic information. |
|  |  | rename <old name> <new name> |  | Rename the specified trusted remote host certificate. <old name> specifies the name of the certificate to be renamed. <new name> specifies the new name as which the certificate is to be saved. |
|  | dir_service |  |  |  |
|  |  | add <name> <addr[:port]> [login:pswd] |  | Add a new directory service. <name> specifies a descriptive name as which the added directory server is to be saved. <addr[:port]> specifies the server address (required) and port (optional). The format is "server-address[:port]". The default port is 389. [login:pswd] specifies the login name and password, if required. The format is "[login:password]". |
|  |  | delete <name> |  | Delete the specified directory service. <name> specifies the name of the directory server to be deleted. |
|  |  | view <name> |  | View the specified directory service. <name> specifies the name of the directory server to be viewed. |
|  |  | edit <name> <addr[:port]> [login:pswd] |  | Edit the specified directory service. <name> specifies the name of the directory server to be edited. <addr[:port]> specifies the server address (required) and port (optional). The format is "server-address[:port]". The default port is 389. [login:pswd] specifies the login name and password, if required. The format is "[login:password]". |
|  |  | list |  | List all directory service names and basic information. |
|  |  | rename <old name> <new name> |  | Rename the specified directory service. <old name> specifies the name of the directory server to be renamed. <new name> specifies the new name as which the directory server is to be saved. |
|  | cert_manager |  |  |  |
|  |  | reinit |  | Reinitialize the certificate manager. |

Bandwidth management Related Command							Home

| Command |  |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| bm |  |  |  |  |  |  |
|  | interface | lan | enable | <bandwidth xxx> |  | Enable bandwidth management in LAN with bandwidth xxx bps. If the user doesn’t set the bandwidth, the default value is 100Mbps. |
|  |  |  |  | <wrr¦prr> |  | Select fairness-based(WRR) or priority-based(PRR) mechanism. the default value is fairness-based. |
|  |  |  |  | <efficient> |  | Enable work-conserving feature. |
|  |  |  | disable |  |  | Disable bandwidth management in LAN |
|  |  | wan | enable | <bandwidth xxx> |  | Enable bandwidth management in WAN with bandwidth xxx bps. If the user doesn’t set the bandwidth, the default value is 100Mbps. |
|  |  |  |  | <wrr¦prr> |  | Select fairness-based(WRR) or priority-based(PRR) mechanism. the default value is fairness-based. |
|  |  |  |  | <efficient> |  | Enable work-conserving feature. |
|  |  |  | disable |  |  | Disable bandwidth management in WAN |
|  |  | dmz | enable | <bandwidth xxx> |  | Enable bandwidth management in DMZ with bandwidth xxx bps. If the user doesn’t set the bandwidth, the default value is 100Mbps. |
|  |  |  |  | <wrr¦prr> |  | Select fairness-based(WRR) or priority-based(PRR) mechanism. the default value is fairness-based. |
|  |  |  |  | <efficient> |  | Enable work-conserving feature. |
|  |  |  | disable |  |  | Disable bandwidth management in DMZ |
|  |  | wlan | enable | <bandwidth xxx> |  | Enable bandwidth management in WLAN with bandwidth xxx bps. If the user doesn’t set the bandwidth, the default value is 100Mbps. |
|  |  |  |  | <wrr¦prr> |  | Select fairness-based(WRR) or priority-based(PRR) mechanism. the default value is fairness-based. |
|  |  |  |  | <efficient> |  | Enable work-conserving feature. |
|  |  |  | disable |  |  | Disable bandwidth management in WLAN |
|  | class | lan | add # | bandwidth xxx | <name xxx> | Add a class with bandwidth xxx bps in LAN. The name is for users’ information. |
|  |  |  |  |  | <priority x> | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The default value is 3. |
|  |  |  |  |  | <borrow on¦off> | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The default value is off. |
|  |  |  | mod # | <bandwidth xxx> |  | Modify the parameters of the class in LAN. The bandwidth is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <name xxx> |  | Set the class’ name. |
|  |  |  |  | <priority x> |  | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The priority is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <borrow on¦off> |  | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The borrow is unchanged if the user doesn’t set a new value. |
|  |  |  | del # |  |  | Delete the class # and its filter and all its children class and their filters in LAN. |
|  |  | wan | add # | bandwidth xxx | <name xxx> | Add a class with bandwidth xxx bps in WAN. The name is for users’ information. |
|  |  |  |  |  | <priority x> | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The default value is 3. |
|  |  |  |  |  | <borrow on¦off> | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The default value is off. |
|  |  |  | mod # | <bandwidth xxx> |  | Modify the parameters of the class in WAN. The bandwidth is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <name xxx> |  | Set the class’ name. |
|  |  |  |  | <priority x> |  | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The priority is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <borrow on¦off> |  | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The borrow is unchanged if the user doesn’t set a new value. |
|  |  |  | del # |  |  | Delete the class # and its filter and all its children class and their filters in WAN. |
|  |  | dmz | add # | bandwidth xxx | <name xxx> | Add a class with bandwidth xxx bps in DMZ. The name is for users’ information. |
|  |  |  |  |  | <priority x> | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The default value is 3. |
|  |  |  |  |  | <borrow on¦off> | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The default value is off. |
|  |  |  | mod # | <bandwidth xxx> |  | Modify the parameters of the class in DMZ. The bandwidth is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <name xxx> |  | Set the class’ name. |
|  |  |  |  | <priority x> |  | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The priority is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <borrow on¦off> |  | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The borrow is unchanged if the user doesn’t set a new value. |
|  |  |  | del # |  |  | Delete the class # and its filter and all its children class and their filters in DMZ. |
|  |  | wlan | add # | bandwidth xxx | <name xxx> | Add a class with bandwidth xxx bps in WLAN. The name is for users’ information. |
|  |  |  |  |  | <priority x> | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The default value is 3. |
|  |  |  |  |  | <borrow on¦off> | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The default value is off. |
|  |  |  | mod # | <bandwidth xxx> |  | Modify the parameters of the class in WLAN. The bandwidth is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <name xxx> |  | Set the class’ name. |
|  |  |  |  | <priority x> |  | Set the class’ priority. The range is between 0 (the lowest) to 7 (the highest). The priority is unchanged if the user doesn’t set a new value. |
|  |  |  |  | <borrow on¦off> |  | The class can borrow bandwidth from its parent class when the borrow is set on, and vice versa. The borrow is unchanged if the user doesn’t set a new value. |
|  |  |  | del # |  |  | Delete the class # and its filter and all its children class and their filters in WLAN. |
|  | filter | lan | add # | Daddr <mask Dmask> Dport Saddr <mask Smask> Sport protocol |  | Add a filter for class # in LAN. The filter contains destination address (netmask), destination port, source address (netmask), source port and protocol. You may set the value as 0 if you do not care the item. |
|  |  |  | del # |  |  | Delete a filter which belongs to class # in LAN. |
|  |  | wan | add # | Daddr <mask Dmask> Dport Saddr <mask Smask> Sport protocol |  | Add a filter for class # in WAN. The filter contains destination address (netmask), destination port, source address (netmask), source port and protocol. You may set the value as 0 if you do not care the item. |
|  |  |  | del # |  |  | Delete a filter which belongs to class # in WAN. |
|  |  | dmz | add # | Daddr <mask Dmask> Dport Saddr <mask Smask> Sport protocol |  | Add a filter for class # in DMZ. The filter contains destination address (netmask), destination port, source address (netmask), source port and protocol. You may set the value as 0 if you do not care the item. |
|  |  |  | del # |  |  | Delete a filter which belongs to class # in DMZ. |
|  |  | wlan | add # | Daddr <mask Dmask> Dport Saddr <mask Smask> Sport protocol |  | Add a filter for class # in WLAN. The filter contains destination address (netmask), destination port, source address (netmask), source port and protocol. You may set the value as 0 if you do not care the item. |
|  |  |  | del # |  |  | Delete a filter which belongs to class # in WLAN. |
|  | show | interface | lan |  |  | Show the interface settings of LAN |
|  |  |  | wan |  |  | Show the interface settings of WAN |
|  |  |  | dmz |  |  | Show the interface settings of DMZ |
|  |  |  | wlan |  |  | Show the interface settings of WLAN |
|  |  | class | lan |  |  | Show the classes settings of LAN |
|  |  |  | wan |  |  | Show the classes settings of WAN |
|  |  |  | dmz |  |  | Show the classes settings of DMZ |
|  |  |  | wlan |  |  | Show the classes settings of WLAN |
|  |  | filter | lan |  |  | Show the filters settings of LAN |
|  |  |  | wan |  |  | Show the filters settings of WAN |
|  |  |  | dmz |  |  | Show the filters settings of DMZ |
|  |  |  | wlan |  |  | Show the filters settings of WLAN |
|  |  | statistics | lan |  |  | Show the statistics of the classes in LAN |
|  |  |  | wan |  |  | Show the statistics of the classes in WAN |
|  |  |  | dmz |  |  | Show the statistics of the classes in DMZ |
|  |  |  | wlan |  |  | Show the statistics of the classes in WLAN |
|  | monitor | lan | <#> |  |  | Monitor the bandwidth of class # in LAN. If the class is not specific, all the classes in LAN will be monitored. The first time you key the command will set it on; the second time you will set it off, and so on. |
|  |  | wan | <#> |  |  | Monitor the bandwidth of class # in WAN. If the class is not specific, all the classes in WAN will be monitored. The first time you key the command will set it on; the second time you will set it off, and so on. |
|  |  | dmz | <#> |  |  | Monitor the bandwidth of class # in DMZ. If the class is not specific, all the classes in DMZ will be monitored. The first time you key the command will set it on; the second time you will set it off, and so on. |
|  |  | wlan | <#> |  |  | Monitor the bandwidth of class # in WLAN. If the class is not specific, all the classes in WLAN will be monitored. The first time you key the command will set it on; the second time you will set it off, and so on. |
|  | moveFilter | < channName> | <from> | <to> |  | User can move BWM filter order via this command. <channName>: lan, wan/wan1, dmz, wan2, wlan <from>: filter index <to>: filter index |
|  | config | save |  |  |  | Save the configuration. |
|  |  | load |  |  |  | Load the configuration. |
|  |  | clear |  |  |  | Clear the configuration. |
|  | vpnTraffic |  |  | <on¦off> |  | Change BWM classifier do classify based on inner protocol header or IPSec header. |
|  | packetBased |  |  | <on¦off> |  | Change BWM classifier based on stream based or packet based |

Load Sharing Command								Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| ls |  |  |  |  |
|  | band | <up¦down> | <WAN1 bandwidth+WAN2 bandwidth> | It is used to configure the bandwidth parameters. The CI format is ls band <method(up, down)¦WAN1 loading bandwidth ¦WAN2 bandwidth. Ex: “ls band up 100 200” will configure the Load Sharing function dispatch the loading between WAN1 and WAN2 with 100K and 200K upstream loading. |
|  | wrr |  | <Weight of WAN1> + <Weight of WAN2> | It is used to configure the weight parameters. The CI format is ls wrr <Weight of WAN1> + <Weight of WAN2>. The valid numver of weight is 0~10 Ex: “ls wrr 10 5” will configure the weight of the WAN1 to be 10, weight of the WAN2 to be 5. |
|  | spillover |  | < upper bandwidth of primary WAN > | It is used to configure the spillover upper bandwidth of primary WAN. Ex: “ls spillover 100”, the router will send the traffic to secondary WAN when the primary WAN bandwidth exceeds 100Kbps. |
|  | mode |  | <1:Least Load First¦2:WRR¦3:Spillover¦255:None> | Change the dispatch mode. 1 is for dispatch packets by Dynamic Load Balancing, 2 is for dispatch packets by WRR, 3 is dispatch packets by Spillover. And 255 is for disable the Load Sharing function. |
|  | timeframe |  | <10~600> | Change the Time Frame number. The valid number of it is 10~600 |
|  | disp |  |  | Display the Load Sharing configuration data |
|  | debug |  |  | Debug CI commands |
|  |  | online | <on¦off> | To toggle the debug message on or off. This command is useful for debugging. |

Bridge Related Command                  			                         Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| bridge |  |  |  |  |
|  | mode |  | <1/0> (enable/disable) | turn on/off (1/0) LAN promiscious mode |
|  | blt |  |  | related to bridge local table |
|  |  | disp | <channel> | display blt data |
|  |  | reset | <channel> | reset blt data |
|  |  | traffic |  | display local LAN traffic table |
|  |  | monitor | [on¦off] | turn on/off traffice monotor. Default is off. |
|  |  | time | <sec> | set blt re-init interval |
|  | brt |  |  | related to bridge route table |
|  |  | disp | [id] | display brt data |
|  |  | reset | [id] | reset brt data |
|  | cnt |  |  | related to bridge routing statistic table |
|  |  | disp |  | display bridge route counter |
|  |  | clear |  | clear bridge route counter |
|  | Iface |  |  | Related to “bridge mode” access interface |
|  |  | active | <yes/no> | Active bridge mode iface or not |
|  |  | address | [ip] | Remote access IP address |
|  |  | dns1 | [ip] | First DNS server |
|  |  | dns2 | [ip] | Second DNS server |
|  |  | dns3 | [ip] | Third DNS server |
|  |  | mask | [network mask] | Network mask |
|  |  | gateway | [gateway ip] | Network gateway |
|  |  | display |  | Display whole interface information |
|  | Stat |  |  | related to bridge packet statistic table |
|  |  | disp |  | display bridge route packet counter |
|  |  | clear |  | clear bridge route packet counter |
|  | Disp |  |  | display bridge source table |
|  | fcs |  | <BriFcsCtl> | set bridge fcs control flag |
|  | rstp |  |  |  |
|  |  | bridge |  |  |
|  |  |  | enable | enable this device RSTP function |
|  |  |  | disable | disable this device RSTP function |
|  |  |  | priority [pirority] | set RSTP pirority |
|  |  |  | maxAge [max age] | set RSTP max age |
|  |  |  | helloTime [hello time] | set hello time |
|  |  |  | forwardDelay [forwarding delay] | set forwarding delay |
|  |  |  | version <STP:0¦RSTP:2> | switch STP or RSTP |
|  |  | port |  |  |
|  |  |  | enable <Port_NO> | enable RSTP on this port |
|  |  |  | disable <Port_NO> | disable RSTP on this port |
|  |  |  | pathCost <Port_NO> [path cost] | set path cost on this port |
|  |  |  | pirority <Port_NO> [pirority] | set pirority on this port |
|  |  |  | edgePort <Port_NO> <True:1¦False:0> | set edge or non-edge on this port |
|  |  |  | p2pLink <Port_NO> <Auto:2¦True:1¦False:0> | set per to per link on this port |
|  |  |  | mcheck <Port_NO> | set migrate check on this port |
|  |  | disp |  | display RSTP information |
|  |  | trace |  | turn on debug/trace message |
|  |  | state |  | display RSTP information |

myZyXEL.com Command								Home

| Command |  |  |  | Description |
| --- | --- | --- | --- | --- |
| sys |  |  |  |  |
|  | myZyxelCom |  |  |  |
|  |  | checkUserName | <username> | Check the username exists or not |
|  |  | register | <username> <password> <email> <countryCode> | Inout the registration information, include username, password, email, and country code. |
|  |  | trialService | <service>, 1 : CF, 2 : 3in1, 3 : CF + 3in1 | Input the service that to be tried. |
|  |  | serviceUpgrade | <licence key> | Inout license key that you want to let service from trial to standard |
|  |  | serviceRefresh | NULL | Refresh the myZyXEL.com service status |
|  |  | display | NULL | Display all myZyXEL.com setting |
|  |  | serviceDisplay | NULL | Display all service status, include expired day. |

IDP Command										Home

| Command |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| idp |  |  |  |  | IDP CI commands |
|  | display |  |  |  | Display the enable setting and the protected interface setting |
|  | load |  |  |  | Load the enable setting and the protected interface setting |
|  | config |  |  |  | Config the enable setting and the protected interface setting |
|  |  | enable | <on¦off> |  | Config the enable setting. |
|  |  | lan-lan | <on¦off> |  | Config the protected interface setting. |
|  |  | lan-wan | <on¦off> |  | Config the protected interface setting. |
|  |  | lan-dmz | <on¦off> |  | Config the protected interface setting. |
|  |  | lan-wan2 | <on¦off> |  | Config the protected interface setting. |
|  |  | lan-wlan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan-lan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan-wan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan-dmz | <on¦off> |  | Config the protected interface setting. |
|  |  | wan-wan2 | <on¦off> |  | Config the protected interface setting. |
|  |  | wan-wlan | <on¦off> |  | Config the protected interface setting. |
|  |  | dmz-lan | <on¦off> |  | Config the protected interface setting. |
|  |  | dmz-wan | <on¦off> |  | Config the protected interface setting. |
|  |  | dmz-dmz | <on¦off> |  | Config the protected interface setting. |
|  |  | dmz-wan2 | <on¦off> |  | Config the protected interface setting. |
|  |  | dmz-wlan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan2-lan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan2-wan | <on¦off> |  | Config the protected interface setting. |
|  |  | wan2-dmz | <on¦off> |  | Config the protected interface setting. |
|  |  | wan2-wan2 | <on¦off> |  | Config the protected interface setting. |
|  |  | wan2-wlan | <on¦off> |  | Config the protected interface setting. |
|  |  | wlan-lan | <on¦off> |  | Config the protected interface setting. |
|  |  | wlan-wan | <on¦off> |  | Config the protected interface setting. |
|  |  | wlan-dmz | <on¦off> |  | Config the protected interface setting. |
|  |  | wlan-wan2 | <on¦off> |  | Config the protected interface setting. |
|  |  | wlan-wlan | <on¦off> |  | Config the protected interface setting. |
|  | save |  |  |  | Save the enable setting and the protected interface setting |
|  | tune |  |  |  | The tune command for IDP/Anti-Virus/Anti-Spam |
|  |  | Load |  |  | Load the tune configuration |
|  |  | Save |  |  | Save the tune configuration |
|  |  | display |  |  | Display the tune configuration |
|  |  | config |  |  | Config the tune configuration |
|  |  |  | l4Udpcksum | <on¦off> | Enable/Disable UDP checksum check |
|  |  |  | l4Icmpcjsum | <on¦off> | Enable/Disable ICMP checksum check |
|  |  |  | l4Tcpcksum | <on¦off> | Enable/Disable TCP checksum check |
|  |  |  | l4Tcpwindowck | <on¦off> | Enable/Disable TCP window check |
|  |  |  | l4Tcpmssck | <on¦off> | Enable/Disable TCP mss check |
|  |  |  | l7Smtpasm | <on¦off> | Enable/Disable TCP assembly for SMTP |
|  |  |  | l7Pop3asm | <on¦off> | Enable/Disable TCP assembly for POP3 |
|  |  |  | l7Httpasm | <on¦off> | Enable/Disable TCP assembly for HTTP |
|  |  |  | l7Ftpasm | <on¦off> | Enable/Disable TCP assembly for FTP |
|  |  |  | l7Ftpdataasm | <on¦off> | Enable/Disable TCP assembly for FTPDATA |
|  |  |  | l7Otherasm | <on¦off> | Enable/Disable TCP assembly for other protocols |
|  | update |  |  |  | The command about signature and signature update stuffs |
|  |  | display |  |  | Show the signature information and the update setting |
|  |  | load |  |  | Load the signature update setting |
|  |  | save |  |  | Save the signature update setting |
|  |  | start |  |  | Start the signature update |
|  |  | config |  |  | Config the signature update setting |
|  |  |  | autoupdate | <on¦off> | Enable/Disable the autoupdate |
|  |  |  | method | <1-3> | Config the update method |
|  |  |  | dailyTime | <00-23> | Config the daily hour update schedule |
|  |  |  | weeklyDay | <1-7> | Config the weekly day update schedule |
|  |  |  | weeklyTime | <00-23> | Config the weekly hour update schedule |
|  | signature |  |  |  | The command about signature post-process setting |
|  |  | display |  |  | Display the current signature setting |
|  |  | load | <Signature_ID> |  | Load the signature setting that its ID is SignatureIID |
|  |  | save |  |  | Save the signature setting |
|  |  | config |  |  | Config the current signature setting |
|  |  |  | active | <on¦off> | Enable/Disable the active option |
|  |  |  | log | <on¦off> | Enable/Disable the log option |
|  |  |  | alert | <on¦off> | Enable/Disable the alert option |
|  |  |  | action | <1-6> | Set the post action |
|  |  | reset |  |  | Reset the signature setting to the default setting |
|  | device |  |  |  |  |
|  |  | reg |  |  |  |
|  |  | rxring |  |  |  |
|  |  | rxbuf |  |  |  |
|  |  | txbuf |  |  |  |
|  |  | disp |  |  |  |
|  | hardware |  |  |  |  |
|  |  | enable | <on¦off> |  |  |

Anti-Virus Command									Home

| Command |  |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| av |  |  |  |  |  | Anti-Virus CI commands |
|  | display |  |  |  |  | Show the anti-virus setting |
|  | load |  |  |  |  | Load the anti-virus setting |
|  | config |  |  |  |  | Config the anti-virus setting |
|  |  | overZipSession | [0:Block¦ 1:Forward] |  |  | Forward session when the session number is over the maximum ZIP sessions. |
|  |  | enable |  |  |  | Enable/Disable the anti-virus function |
|  |  | httpScanAllMime | <on¦off> |  |  | Enable/Disable scanning all mime type files. If we don’t enable this option , ZyWall will just scan files with the application type |
|  |  | pop3ScanAllMime | <on¦off> |  |  | Enable/Disable scanning all mime type files. If we don’t enable this option , ZyWall will just scan files with the application type |
|  |  | smtpScanAllMime | Mon¦off> |  |  | Enable/Disable scanning all mime type files. If we don’t enable this option , ZyWall will just scan files with the application type |
|  |  | decompress | <on¦off> |  |  | Enable/Disable the decompress on the fly. You should also enable tcp assembly to support the decompress on the fly. |
|  |  | ftp |  |  |  | Config the anti-virus setting for FTP |
|  |  |  | display |  |  | Show the anti-virus setting for FTP |
|  |  |  | active |  | <on¦off> | Enable/Disable the anti-virus function for FTP |
|  |  |  | log |  | <on¦off> | Enable/Disable the log option |
|  |  |  | alert |  | <on¦off> | Enable/Disable the alert option |
|  |  |  | breakfile |  | <on¦off> | Enable/Disable the breakfile option |
|  |  |  | sendmsg |  | <on¦off> | Enable/Disable the sendmsg option |
|  |  | dir |  |  |  |  |
|  |  |  | lan-lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2 -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  | http |  |  |  | Config the anti-virus setting for HTTP |
|  |  |  | display |  |  | Show the anti-virus setting for HTTP |
|  |  |  | active |  | <on¦off> | Enable/Disable the anti-virus function for HTTP |
|  |  |  | log |  | <on¦off> | Enable/Disable the log option |
|  |  |  | alert |  | <on¦off> | Enable/Disable the alert option |
|  |  |  | breakfile |  | <on¦off> | Enable/Disable the breakfile option |
|  |  |  | sendmsg |  | <on¦off> | Enable/Disable the sendmsg option |
|  |  | dir |  |  |  |  |
|  |  |  | lan-lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2 -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  | smtp |  |  |  | Config the anti-virus setting for SMTP |
|  |  |  | display |  |  | Show the anti-virus setting for SMTP |
|  |  |  | active |  | <on¦off> | Enable/Disable the anti-virus function for SMTP |
|  |  |  | log |  | <on¦off> | Enable/Disable the log option |
|  |  |  | alert |  | <on¦off> | Enable/Disable the alert option |
|  |  |  | breakfile |  | <on¦off> | Enable/Disable the breakfile option |
|  |  |  | sendmsg |  | <on¦off> | Enable/Disable the sendmsg option |
|  |  | dir |  |  |  |  |
|  |  |  | lan-lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2 -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  | pop3 |  |  |  | Config the anti-virus setting for POP3 |
|  |  |  | display |  |  | Show the anti-virus setting for POP3 |
|  |  |  | active |  | <on¦off> | Enable/Disable the anti-virus function for POP3 |
|  |  |  | log |  | <on¦off> | Enable/Disable the log option |
|  |  |  | alert |  | <on¦off> | Enable/Disable the alert option |
|  |  |  | breakfile |  | <on¦off> | Enable/Disable the breakfile option |
|  |  |  | sendmsg |  | <on¦off> | Enable/Disable the sendmsg option |
|  |  | dir | lan-lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | lan-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | dmz -wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2 -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wan2-wlan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -lan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -dmz |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wan2 |  | <on¦off> | Config the protected interface setting |
|  |  |  | wlan -wlan |  | <on¦off> | Config the protected interface setting |
|  | save |  |  |  |  | Save the anti-virus setting |
|  | update |  |  |  |  | The command about signature and signature update stuffs |
|  |  | display |  |  |  | Show the signature information and the update setting |
|  |  | load |  |  |  | Load the signature update setting |
|  |  | save |  |  |  | Save the signature update setting |
|  |  | start |  |  |  | Start the signature update |
|  |  | config |  |  |  | Config the signature update setting |
|  |  |  | autoupdate | <on¦off> |  | Enable/Disable the autoupdate |
|  |  |  | method | <1-3> |  | Config the update method |
|  |  |  | dailyTime | <00-23> |  | Config the daily hour update schedule |
|  |  |  | weeklyDay | <1-7> |  | Config the weekly day update schedule |
|  |  |  | weeklyTime | <00-23> |  | Config the weekly hour update schedule |
|  | tune |  |  |  |  | The tune command for IDP/Anti-Virus/Anti-Spam |
|  |  | load |  |  |  | Load the tune configuration |
|  |  | save |  |  |  | Save the tune configuration |
|  |  | display |  |  |  | Display the tune configuration |
|  |  | config |  |  |  | Config the tune configuration |
|  |  |  | l4Udpcksum | <on¦off> |  | Enable/Disable UDP checksum check |
|  |  |  | l4Icmpcjsum | <on¦off> |  | Enable/Disable ICMP checksum check |
|  |  |  | l4Tcpcksum | <on¦off> |  | Enable/Disable TCP checksum check |
|  |  |  | l4Tcpwindowck | <on¦off> |  | Enable/Disable TCP window check |
|  |  |  | l4Tcpmssck | <on¦off> |  | Enable/Disable TCP mss check |
|  |  |  | l7Smtpasm | <on¦off> |  | Enable/Disable TCP assembly for SMTP |
|  |  |  | l7Pop3asm | <on¦off> |  | Enable/Disable TCP assembly for POP3 |
|  |  |  | l7Httpasm | <on¦off> |  | Enable/Disable TCP assembly for HTTP |
|  |  |  | l7Ftpasm | <on¦off> |  | Enable/Disable TCP assembly for FTP |
|  |  |  | l7Ftpdataasm | <on¦off> |  | Enable/Disable TCP assembly for FTPDATA |
|  |  |  | l7Otherasm | <on¦off> |  | Enable/Disable TCP assembly for other protocols |
|  | zipUnsupport |  | zipEncrypDrop flag [0/1] |  |  | Processing ZIP file will destroy encrypted file if flag is on, otherwise pass it. |

Anti-Spam Command								Home

| Command |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- |
| as |  |  |  |  | Anti-Spam CI commands |
|  | asAction | [0¦1] |  |  | Forward/Block exceeding mails sessions. |
|  | debug |  |  |  | Debug for AntiSpam |
|  |  | customListServ |  |  | Set custom server list server |
|  |  |  | ip | [IP address] | Set custom server list server IP address |
|  |  |  | enable | [0:disable¦1:enable] | Enable/Disable custom server list server |
|  |  | customRateServ |  |  | Set custom rating server server. |
|  |  |  | ip | [IP address] | Set custom rating server IP address |
|  |  |  | enable | [0:disable¦1:enable] | Enable/Disable custom rating server |
|  |  | envelope | [on¦off] |  | Enable/Disable envelope debug message. |
|  |  | http | [on¦off] |  | Enable/Disable http debug message. |
|  |  | mail | [on¦off] |  | Enable/Disable mail debug message. |
|  |  | pop3 | [on¦off] |  | Enable/Disable pop3 debug message. |
|  |  | smtp | [on¦off] |  | Enable/Disable smtp debug message. |
|  | delete |  |  |  | Delete AntiSpam static filter. |
|  |  | blackRule | <num start> [num end] |  | Delete black rule filter. User can delete one or a set of filter. |
|  |  | whiteRule | <num start> [num end] |  | Delete white rule filter. User can delete one or a set of filter. |
|  | display |  |  |  |  |
|  |  | antispam |  |  | Display AntiSpam configuration. |
|  |  | serverlist |  |  | Display rating server list. |
|  |  | runtimedata | <all¦black¦white> | [all¦ip¦mime¦email¦subject] | Display runtime data for anti-spam ACL structure. |
|  | enable | <0:disable ¦ 1:enable> |  |  | Enable/Disable AntiSpam. |
|  | failTolerance | [time] |  |  | Set rating server fail tolerance time. If the rating server timeout interval over this tolerance, this server will be removed from server list. |
|  | freeSession |  |  |  | Free all mail ssessions. |
|  | getServerList | <Y:Yes¦N:No> |  |  | Send server list request manually. |
|  | dir | <lan¦wan1¦dmz¦wan2¦wlan> | <lan¦wan1¦dmz¦wan2¦wlan> | <on¦off> | Enable or disable on direction of Anti Spam |
|  | scoreTimeout |  |  | value | Set the AS score query timeout value. |