CPU charge for streaming IMA/DVI ADPCM decoding

PWM interrupts charge (at 94 kHz):

800ns every 10.6us

-> 7.5% of CPU charge

For an 8000Hz sample rate sound:

Getting information through the USART + IMA/DVI decoding + circular buffers management:

3.6us every 117us

-> 3% of CPU charge

(3.2us are taken by the IMA/DVI ADPCM sample decoding).

For a 22050Hz sample rate sound:

Getting information through the USART + IMA/DVI decoding + circular buffers management:

3.6us every 43us

-> 8.25% of CPU charge

(3.2us are taken by the IMA/DVI ADPCM sample decoding).

IMA/DVI ADPCM Encoder:

Scheme:

| WAVE File |  | PC |  | AVR32UC3 |  | MIC |
| --- | --- | --- | --- | --- | --- | --- |

This is the CPU charge details for the AVR32UC3 while performing a streaming IMA/DVI ADPCM encoding process.

All these results have been obtained with a 48MHz clock frequency.

For an 8000Hz sample rate sound:

Getting information from the microphone + IMA/DVI encoding + circular buffers management + USART transmission:

780us every 63.6ms

-> 1.3% of CPU charge (This is different from the IMA/DVI ADPCM decoder because this application does not work in real time, it stores data in a buffer and then encode this buffer using the ADPCM encode block function and not the ADPCM encode stream function). The same process with the ADPCM encode stream function takes 1.35ms every 63.6ms -> 2.2% of CPU charge.

For a 22050Hz sample rate sound:

Getting information from the microphone + IMA/DVI encoding + circular buffers management + USART transmission:

780us every 22.9ms

-> 3.4% of CPU charge. The same process with the ADPCM encode stream function takes 1.35ms every 22.9ms -> 5.9% of CPU charge.